SA WG2 Meeting #143e	S2-2101027r02

# Feb 24th – March 9th, 2021 ; Elbonia

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Edge relocation with multiple AFs

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: This contribution proposes updates to AF request to influence traffic routing for scenarios with multiple AFs based on the conclusion in clause 9.2.6 in TR 23.748

# Proposal

It is proposed to add the following to TS 23.548.

START CHANGE

### 6.3.2	Edge relocation triggered by AF

Editor’s Note: this clause refers to TS 23.502 for procedures addressing the multiple AF scenario (e.g. central and distributed AFs for EC) like AF relocation at Edge relocation proposed in clause 9.2.6, and AF provide DNAI and target N6 info to the SMF as concluded in 9.2.3 in TR.

#### 6.3.2.X Edge relocation with multiple AFs

##### 6.3.2.X.1	Overview

This clause addresses scenarios where Edge Application Server(s) and Application Function are located at the Edge Data Network, thus implying multiple AF instances i.e. one (or more) local AF(s) and also a central AF. Application Function acts as control plane entity managing one or more EAS and responsible to interact with 5GC. In such deployment scenarios: as part of initial PDU session establishment, a central AF may be involved, however, due to relocation to Edge DNAI and corresponding EAS relocation, a new local AF serving the Edge Applications may be selected.

### 6.3.2.X.2	Procedure to edge relocation with multiple AFs

Detail procedure involving multiple AF scenarios are provided in the below figure 6.3.2.X.2-1.

Figure 6.3.2.X.1-1: Processing AF requests to influence traffic routing for scenarios with multiple AFs

0.	AF1 is interacting with SMF/5GC e.g. as part of initial PDU Session establishment procedure. AF1 has sent via Nnef-trafficinflence traffic routing guidance (as defined in 23.501 [2], Clause 5.6.7) that may include a priority value

NOTE: AF1 request can e.g. target multiple UEs such as a group of UEs or Any UE

1.	AF1 decides to relocate to a local AF i.e. to AF2 which serves local EAS located in the target DNAI. There could be multiple triggers for AF1 to relocate AF e.g. decision to relocate to local /another Edge Application Server , EAS load balancing, etc. The AF decision may also be based on a SMF notification due to change in DNAI as defined in 23.502 [3] clause 4.3.6.3.

2.	AF1 initiates AF relocation information transfer with AF2, the context transfer between AFs is out of scope of SA2.

3a.	AF2, based on the information received from AF1, sends via NEF a Nnef_TrafficInfluence_Create request targeting the UE ID provided by AF1. AF2 includes in this request: AF ID, notification address.

3b. Alternatively, AF1 sends to NEF, Nnef_TrafficInfluence_update request and providesa new notification address.

4.	This is same as steps 3a, 3b and 4 as described in figure 4.3.6.2-1 in TS 23502.

5. PCF determines that existing PDU Session is impacted by the AF2 requested information and so PCF updates the SMF with corresponding new PCC rule(s) by invoking Npcf_SMPolicyControl_UpdateNotify service operation. If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

6: Same as described in figure 4.3.6.2-1 in TS 23.502

7-8.  SMF sends Nsmf_EventExposure_Notify indicating the changes in the user plane management information to AF1/AF2, based on information received in step 4, either directly or via NEF. This is as defined in 23.502 clause 4.6.3.6

End of changes