3GPP TSG-SA/WG2 Meeting #144e	S2-2102426

# Elbonia, Apr. 12 – 16 , 2020			   (revision of S2-210xxxx)

Source:	Intel, Lenovo, Tencent, Motorola Mobility

Title:	Update Clause 6.3.3 on EAS IP Replacement in 5GC

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: This contribution updates clause 6.3.3 on EAS IP Replacement in 5GC.

# 1	Proposal

It is proposed to agree the changes into TS 23.548:

/******************Start of Change ********************/

### 6.3.3	Edge relocation using EAS IP replacement

EAS IP replacement enables the local PSA UPF to replace the source EAS IP address and port number with the target/new target EAS IP address and port number for the Destination IP address and Destination Port number field of the uplink traffic and replace the target/new target EAS IP address and port number with the source EAS IP address and port number for the Source IP address and Source Port number field of the downlink traffic based on the enhanced AF Influence information for EAS IP replacement (i.e. Indication of Enabling EAS IP Replacement, source EAS IP address and port number, target EAS IP address and port number). The source AS IP address and port number are the destination IP address and port number of the uplink traffic, generated by UE, for a service subject to Edge Computing. The source EAS IP address is the one discovered by UE for a service subject to Edge Computing.

EAS IP replacement requires support of TCP/TLS/QUIC context transfer between EAS.

NOTE 1:  The feasibility of this requirement i.e. TCP/TLS/QUIC context transfer between EAS is unclear and whether third party platforms would support this TCP/TLS/QUIC context transfer between EAS is unknown/not clear.

#### 6.3.3.1	EAS IP Replacement procedures

##### 6.3.3.1.1	Enabling EAS IP Replacement Procedure by AF

Figure 6.3.3.1.1-1: Enabling EAS IP Replacement Procedure by AF

NOTE 1: This procedure covers the scenarios that the UE moves from non-EC to EC or the AF decides to enable the EAS IP replacement in the middle of a session.

1.	UE requests to establish a PDU Session.

2.	UE is preconfigured with the Source EAS IP address or discovers the IP address of the application server for the service subject to Edge Computing, and the Source EAS IP address is returned to the UE via EAS Discovery procedure.

3.	UE communicates with the Source EAS.

4.	AF detects that the EAS is capable of runtime context minoring and an optimal EAS is found, then AF decides to influence the traffic routing in 5GC. The EAS IP replacement information (i.e. Indication of Enabling EAS IP Replacement, source EAS IP address and port number, target EAS IP address and port number) is sent to the SMF within the AF Influence information and the SMF reconfigures the UL CL UPF and Local PSA with EAS IP replacement information. Or when UE moves to an area where the Local PSA has been configured to enforce EAS IP address replacement.

UL CL is configured by SMF to forward the destination IP address in the UL packet equals to the Source EAS IP address to Local PSA.

Local PSA is configured by SMF to enforce the "Outer Header Creation" and " Outer Header Removal" as described in step 5. FARs "Outer Header Creation" and " Outer Header Removal" are reused for such an instruction from SMF to UPF.

Detailed enhancement to the AF Influence procedure is described in clause 6.3.3.2.

If a new local PSA is selected by SMF, the SMF may configure the new local PSA to buffer the uplink traffic per clause 6.3.5 and enforce the “Outer Header Creation” and “Outer Header Removal” as described in step 6.

5.	In step 4, if the target EAS IP address and port number are not included, Early Notification procedure with enhancement described in clause 6.3.3.2 is triggered, the SMF notifies AF about the target DNAI. Based on the target DNAI, the AF selects a proper target EAS, then the AF triggers to mirror the runtime context between Source EAS and Target EAS. Once the Target EAS is ready, AF responds to SMF about the Target EAS IP information (i.e. Target EAS IP address and port number). SMF reconfigures Local PSA for EAS IP address replacement between Source EAS and Target EAS.

6.	Local PSA starts to perform "Outer Header Creation" and " Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the Target EAS IP address and port number at Local PSA;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number at Local PSA.

NOTE 2: In this solution, the PSA UPF need not to understand the logic of EAS IP replacement.

7.	Late Notification procedure with enhancement described in clause 6.3.3.2 is triggered, SMF notifies AF about the start of the EAS IP replacement.

Then all subsequent uplink traffic of this EC service for this UE is forwarded to the target EAS.

NOTE 3: AF decides when and how to stop the Source EAS from serving the UE based on its local configuration.

##### 6.3.3.1.2	EAS IP Replacement Update upon DNAI and EAS IP Change

Figure 6.3.3.1.2-1: EAS IP Replacement Update upon DNAI and EAS IP change

1.	For UL traffic, the destination IP address is replaced with the old Target EAS IP address at Local PSA; for DL traffic, the source IP address is replaced back with the Source EAS IP address at Local PSA.

2.	When Early Notification happens, the SMF notifies AF about the target DNAI. Based on the target DNAI, the AF selects a proper new target EAS. The SMF decides whether to relocate UL CL and Local PSA and configures the EAS IP address replacement using "Outer Header Creation" and " Outer Header Removal" in Local PSA2 if PSA relocation happens.

NOTE 1: AF can initiate application context exchange between source and target EAS after receiving the early notification request.

3.	Local PSA2 starts to perform "Outer Header Creation" and " Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the new Target EAS IP address and port number at Local PSA;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number at Local PSA.

4.	Late Notification procedure with enhancement described in clause 6.3.3.2 is triggered, SMF notifies AF about the start of the EAS IP address replacement towards the new Target EAS.

NOTE 2: AF decides when and how to stop the old Target EAS from serving the UE based on its local configuration.

##### 6.3.3.1.3	Disabling EAS IP Replacement Procedure

Figure 6.3.3.1.3-1: Disabling EAS IP Replacement Procedure

1.	Local PSA performs "Outer Header Creation" and " Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the old Target EAS IP address and port number;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number.

2.	Due to UE Mobility to a Non-EC environment, Early Notification is triggered for the change from the UP path status where a DNAI applies to a status where no DNAI applies, AF knows the UE moves out of EC environment and mirrors the runtime session context from old Target EAS to Source EAS. Once ready, the AF indicates SMF to disable the local routing at UL CL and the EAS IP replacement at Local PSA for this PDU Session via AF Influence procedure.

3.	UL and DL traffic goes through Remote PSA, no EAS IP address replacement happens at Remote PSA.

NOTE 1: AF decides when and how to stop the old Target EAS from serving the UE based on its local configuration. In case of AF relocation, AF doesn’t have to disable the EAS IP Replacement in 5GC.

#### 6.3.3.2 Enhancement to AF Influence

The AF may additionally include the Indication of Enabling EAS IP Replacement, Source and Target EAS IP address(es) and Port number(s) in the Nnef_TrafficInfluence_Create/Update request. Based on the Source EAS IP address(es), the SMF knows which service flow(s) is(are) subject to EAS IP Replacement.

Using Late Notification procedure, the AF sends a late notification response to the SMF when EAS relocation is completed. The SMF sends the "Outer Header Creation" and " Outer Header Removal" FARs to (target) Local PSA UPF and (target) Local PSA UPF starts the EAS IP address replacement as described in clause 6.3.3.1.

For load balancing purpose, the AF may move some UE(s) from the old Target EAS) to the New Target EAS in the same local DN identified by the DNAI. For the abnormal condition of EAS, the AF may move all the UEs being served by the source EAS to a target EAS in the same local DN. For this purpose, the AF needs to include the Indication of Enabling EAS IP Replacement, List of UEs, the old Target EAS IP address and port number for the impacted DNAI, the new Target EAS IP address and port number for the impacted DNAI in the Nnef_TrafficInfluence_Create/Update request.

The additional parameters for enabling the EAS IP Replacement are defined in clause 5.6.7.1 of TS 23.501[xx], clause 4.3.6.3 and 4.3.6.4 of 23.502[yy].

/*******************End of Change****************/