3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102131

# Elbonia, April 12th - 16th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0257 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#7 - Extensions to User Data Congestion Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Spirent, Amdocs, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The AnalyticsID on User Data Congestion can provide one analytics subset for the list of applications contributing the most to the traffic in an area, however an operator may not want to provide this list to any consumer, in particular an AF outside the MNO domain may not be allowed to collect this list of applications. Therefore, It will be beneficial to check if this analytics subset can be sent to an AF eitehr directly or via NEF. This AnalyticsID was defined in Rel-16 when no analytics subsets existed, then in order to provide backward compatibility with a Rel-16 consumer not providing any analytics subset, the Network Status Report is always provided and the list of applications contributing the most to the traffic in an area are an analytics subset. The procedure for one-time or continuous reporting of analytics for user data congestion for a specific UE is not updated to over the conclusions from KI#7. Some corrections are needed to indicate the Target for Analytics can be a UE ID, and not only a SUPI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify which consumers can request each analytics subset. - In order to provide backward compatibility for a consumer that does not provide analytics subset, the Network status information is always provided therefore not an analytics subset. - Given that the only analytics subset is the list of applications contribution the most to the traffic, there is no need to define a different level of accurary. - Update procedure 6.8.4.2 to show AF and UPF as providers of input data. - Clarify that NWDAF collects data from UPF serving the area of interest. - Replace SUPI by UE ID(i.e.SUPI or GPSI). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the conclusions in KI#7. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1, 6.8.3, 6.8.4.1, 6.8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------------------First Change----------------------------------------------------------

### 6.8.1	General

The NWDAF can provide user data congestion related analytics, by one-time reporting or continuous reporting, in the form of statistics or predictions or both, to another NF. User Data Congestion related analytics can relate to congestion experienced while transferring user data over the control plane or user plane or both. A request for user data congestion analytics relates to a specific area or to a specific user. If the consumer of these analytics provides a UE ID, the NWDAF determines the area where the UE is located. The NWDAF then collects measurements per cell and uses the measurements to determine user data congestion analytics.

The request for user data congestion related analytics indicates the location area information where congestion related analytics is desired or indicates a UE Identity that can be used by the NWDAF to determine the location area information where congestion related analytics is desired. When requesting user data congestion, the consumer may request the identifiers of the applications that contribute the most to the traffic in the area. The consumer may indicate how many applications should be reported by providing the maximum number of applications in the request or subscription.

When the consumer of user data congestion related analytics subscribes to user data congestion related analytics, it may indicate a threshold and the NWDAF will provide analytics to the consumer when the congestion level crosses the threshold. The consumer can indicate an S-NSSAI in the request when congestion analytics are needed on a per slice level.

The service consumer may be an NF (e.g. NEF, AF, PCF).

The consumer of these analytics may indicate in the request or subscription the following parameters, its content is described in the clause 6.1.3:

-	Analytics ID set to "User Data Congestion".

-	Target of Analytics Reporting containing either a UE id (i.e. SUPI or GPSI), or "any UE".

NOTE: The Target of Analytics Reporting set to “any UE” applies when user data congestion analytics relates to a specific Area of Interest and is included to comply with the mandatory parameters in Nnwdaf_AnalyticsSubscribe and Nnwdaf_AnalyticsInfo services.

-	Analytics Filter Information containing:

-	Area of Interest (i.e. list of TAIs or CellIDs) which restricts the area in focus (mandatory if Target Of Analytics Reporting is set to "any UE", optional otherwise);

-	an optional list of analytics subsets that are requested, (see clause 6.8.3);

-	Optional S-NSSAI, in order to obtain congestion analytics only on a given slice.

-	Analytics target period.

-	Optional Reporting Threshold, which applies only for subscriptions and indicates conditions on the congestion level (Network Status Indication, see clause 6.8.3) to be reached in order to be notified by the NWDAF.

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the list of User Data Congestion statistics or predictions:

-	ordering by Applicable Time Window, chronological or reverse chronological order; or

-	ordering by Network Status Indication, ascending or descending;

-	Optional maximum number of objects;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested, either in the past or in the future.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.8.3.

### 6.8.2	Input data

The detailed information collected by the NWDAF is defined in Table 6.8.2-1.

NOTE 1:	Performance Measurements defined in TS 28.552 [8] represent resource utilisation but do not, by themselves, indicate the event of congestion or congestion levels. The NWDAF collects measurements from the OAM and how the NWDAF derives Network Status Indication (NSI) is not specified.

Table 6.8.2-1: Data Collected from the NF and OAM related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE Location | AMF | UE location information that NWDAF can use to derive the Area of Interest. |
| Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine congestion levels. Performance Measurements are related to information transfer over the user plane and/or the control plane (e.g. UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management, and Radio Resource Utilization as defined in TS 28.552 [8]). The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7]. |

Table 6.8.2-2: Data Collected from the UPF or from the AF related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| Application ID | UPF or AF | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (NOTE 1). |
| IP Packet Filter Set | UPF or AF | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (NOTE 1). |
| Measurement period | UPF or AF | Measurement period. |
| Throughput UL/DL | UPF or AF | Average Throughput UL/DL over the measurement period. |
| Throughput UL/DL (peak) | UPF or AF | Peak Throughput UL/DL over the measurement period. |
| Timestamp | UPF or AF | Time when measurements are taken. |
| NOTE 1: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 2: Multiple outputs are provided by the UPF when multiple Service Data Flows are running at the UPF for the same UE and measurement period. NOTE 3: How NWDAF collects information from UPF is not defined in this Release of the specification. |  |  |

Additionally, NWDAF may use statistics or predictions on service experience as specified in clause 6.4.3 as an input, e.g. for service experience in a given area or service experience for some specific applications such as high bandwidth applications.

### 6.8.3	Output analytics

The NWDAF outputs the user data congestion analytics for transfer over the user plane, for transfer over the control plane, or for both. The output may consist of statistics, predictions, or both. The detailed information provided by the NWDAF is defined in Table 6.8.3-1 for statistics and in Table 6.8.3-2 for predictions.

Table 6.8.3-1: User Data Congestion statistics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs |
| List of user data congestion Analytics (1..max) |  |
| >Type | User Plane or Control Plane |
| >Applicable Time Window | The time period that the analytics applies to |
| >Network Status Indication | Congestion Level |
| > List of top applications in UL (0..NU) (NOTE 1) (NOTE X) | The list of applications that contribute the most to the traffic in the UL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2). |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| > List of top applications in DL (0..ND) (NOTE 1) (NOTE X) | The list of applications that contribute the most to the traffic in the DL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2) |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 3: The listed applications are not necessarily ranked by any order of traffic contribution. NOTE X; This information element relates to congestion experienced while transferring user data over the user plane. |  |

Table 6.8.3-2: User Data Congestion predictions

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs. |
| List of user data congestion Analytics (1..max) |  |
| >Type | User Plane or Control Plane. |
| >Applicable Time Window | The time period that the analytics applies to. |
| >Network Status Indication | Congestion Level. |
| > Confidence | Confidence of this prediction. |
| > List of top applications in UL (0..NU) (NOTE 1) (NOTE X) | The list of applications predicted to contribute the most of the traffic in the UL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2). |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| >> Confidence | Confidence of this prediction. |
| > List of top applications in DL (0..ND) (NOTE 1) (NOTE X) | The list of applications predicted to contribute the most of the traffic in the DL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2). |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| >> Confidence | Confidence of this prediction. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 3: The listed applications are not necessarily ranked by any order. NOTE X: This information element relates to congestion experienced while transferring user data over the user plane. |  |

The number of user data congestion analytics entries is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The following list shows the applicability of the analytics subsets per consumer:

-     Analytics subset “List of top applications in UL (0..NU)” and “List of top applications in DL (0..ND)” are applicable to any consumer (e.g. PCF, AF, NEF), The NEF does not provide these Analytics subset to an AF outside the MNO domain. Whether the NWDAF provides these Analytics subset to an AF within the MNO domain depends on operator configuration.

### 6.8.4	Procedures

#### 6.8.4.1	Procedure for one-time or continuous reporting of analytics for user data congestion in a geographic area

The procedure as depicted in Figure 6.8.4.1-1 is used by an NF to retrieve congestion analytics for a specific geographic area. The procedure can be used to request a one-time or continuous reporting of congestion analytics.

Figure 6.8.4.1-1: Procedure for one-time or continuous reporting of analytics for congestion in a geographic area

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, indicating request for analytics for congestion in a specific location. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both, the Target for Analytics is set to “any UE” and Analytics Filter set to include a location (e.g. ECGI, TA) and an indication to provide the list of applications that contribute the most to the traffic.

2-3. If the request is authorized, and in order to provide the requested analytics, the NWDAF may request the measurement information for the requested location from OAM services following the data collection from OAM procedure as captured in 6.2.3.2. If the NWDAF already has information about the requested location, these steps are omitted. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

If the request is to provide the list of applications that contribute the most to the traffic, then the NWDAF collects input data from the AF for the applications being served by AF(s) by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF) or collected from the UPF or collected from both, AF and UPF.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics outcome for congestion to the NF using Nnwdaf_AnalyticsInfo_Request response (location, congestion information).

For continuous reporting:

6.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF, indicating request for analytics for congestion in a specific location (e.g. ECGI, TA), possibly with thresholds and including an indication to provide the list of applications that contribute the most to the traffic. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both.

7-8. The NWDAF subscribes to OAM services following the data collection from OAM procedure as captured in 6.2.3.2 to get measurement information for the requested location, possibly providing measurement thresholds for example, data congestion crossing values. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].  If a request is to provide the list of applications that contribute the most to the traffic, the NWDAF subscribes to the service data from AF by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF) or from the UPF or from both.

9.	The NWDAF derives requested analytics.

10.	The NWDAF provides the analytics for congestion to the NF.

11a.	A change of user data congestion status corresponding to crossing a threshold set by the NWDAF at step 7-8 is detected by OAM and notified to NWDAF.

11b. The AF notifies the NWDAF with the input data as defined in table 6.8.2-2.

11c. The UPF provides the NWDAF with the input data as defined in table 6.8.2-2.

12.	The NWDAF derives new analytics.

13.	The NWDAF provides a notification for analytics for the user data congestion to the NF.

#### 6.8.4.2	Procedure for one-time or continuous reporting of analytics for user data congestion for a specific UE

The procedure as depicted in Figure 6.8.4.2-1 is used by an NF to retrieve user data congestion analytics for a specific UE. The procedure can be used to request a one-time or continuous reporting of user data congestion analytics.

Figure 6.8.4.2-1: Procedure for one-time or continuous reporting of analytics for congestion for a specific UE

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, requesting for analytics for user data congestion for a specific UE id. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both, the Target of Analytics Reporting is set to UE id.

2-5. The NWDAF may already know the UE location. If not, the NWDAF checks the UE location by first retrieving the AMF serving the UE (steps 2-3) and then by interrogating the AMF about the UE location.

NOTE 1: The NF sends a request for a UE that is registered, so that NWDAF can retrieve the UE location.

6-7. The NWDAF requests measurement information for the UE location from OAM services (as captured in 6.2.3.2). These steps are omitted if the NWDAF already has the information. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

If the request is to provide the list of applications that contribute the most to the traffic, then the NWDAF collects input data defined in Table 6.8.2-2: from the UPF serving the UE location, or from the  AF(s) by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF) or or collects from both, AF and UPF.

8.	The NWDAF derives requested analytics.

9.	The NWDAF provides the analytics for congestion to the NF.

For continuous reporting:

10.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF. The NF can request for statistics or for predictions or for both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both.

11.	The NWDAF determines the UE location, either via internal information or by applying the same steps as steps 2 to 5. The NWDAF then determines an area of interest.

12-13. The NWDAF subscribes to OAM services (as captured in 6.2.3.2) to get the measurement information for the UE location, possibly providing measurement thresholds. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

If the request is to provide the list of applications that contribute the most to the traffic, then the NWDAF collects input data defiend in Table 6.8.2-2: from the UPF serving the UE location or from the AF(s) by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF) or from or collects from both, AF and UPF.

14.	The NWDAF derives requested analytics.

15.	The NWDAF provides the analytics for user data congestion status information to the NF.

16-17. The NWDAF subscribes to UE mobility event notification in order to be informed when the UE moves out of the area of interest (in order to define a new area of interest and request new information to OAM if the UE moves to a different area).

18a.	A change of user data congestion status corresponding to crossing a threshold set by the NWDAF is detected by OAM and notified to NWDAF.

18b.	The AF notifies the NWDAF with the input data as defined in table 6.8.2-2.

18c.	The UPF provides the NWDAF with the input data as defined in table 6.8.2-2.

19.	The NWDAF derives new analytics.

20.	The NWDAF provides a notification for analytics for the user data congestion status information to the NF.

------------------------------------------------------------END OF CHANGES--------------------------------------------------------