SA WG2 Meeting #144e	S2-2102162

# April 12 – 16, 2021; Elbonia             		 	(revision of S2-210)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	 Issue that we cannot guarantee that the UE uses the EASDF's IP address for its subsequent DNS Query

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: Issue that we cannot guarantee that the UE uses the EASDF's IP address for its subsequent DNS Query

# 1 Discussion

This PCR is part of multiple PCR(s) that address different aspects / items of 6.2.3.2.2 “EAS Discovery Procedure with EASDF”:

- General corrections

- EASDF registration to NRF

- The EN “How to guarantee that the UE uses the EASDF's IP address for the subsequent DNS Query in step 8 is FFS”

The intent is to allow discussion on orthogonal topics in different threads (avoiding one thread with 10 different topics) and to at the end merge the different PCR(s) if found useful.

This PCR addresses item 3.

# 2 Proposal

It is proposed to update TS 23.548 as follows

FIRST CHANGE only the part of the clause 6.2.3.2.2 that is modified by the PCR is shown, the rest of the clause is unchanged by the PCR

5.	The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

NOTE: 	It is not possible to guarantee that the UE uses the EASDF's IP address provided by the SMF for the subsequent DNS Query in step 8; some network-based mechanisms are nevertheless described in step 8;

6.	The SMF may invoke Neasdf_DNSContext_Update Request (PDU Session Context ID, rules to handle DNS queries from the UE) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

Editor's note:	It is FFS whether and how the AF-triggered DNS server information change trigger DNS context update procedure.

7.	The EASDF responds with Neasdf_DNSContext_Update Response.

8.	The UE sends DNS Query message to the EASDF.

The SMF may, based on local policies, have instructed an UPF to detect (UL) DNS traffic from the UE that is not targeting the EASDF's IP address provided to the UE by the SMF in step 5;

This detection may correspond to a PDR referring to the ports used by DNS protocols (53, 853) and to the EASDF's IP address provided by the SMF to the UE over NAS.

The SMF may, based on local policies, further request the UPF to forward such DNS traffic to the SMF in which case the SMF takes care itself of forwarding further the DNS traffic once it has applied corrective actions ; Or alternatively the UPF may be instructed by the SMF to buffer such DNS traffic and to report to the SMF; in this alternative case, once corrective actions have been carried out by the SMF, the SMF may ask the UPF to forward the buffered DNS traffic.

The corrective actions carried out by the SMF may be to insert offload capabilities (UL CL and local PSA) based on UE’s location for the PDU Session  regardless of the detection (via DNS requests) of the UE willingness to reach an EAS.

The local policies mentioned above may be based on the (DNN, S-NSSAI).

The rest of the procedure assumes the UE has indeed used the EASDF as its network DNS resolver.

NEXT CHANGE (2)

End of changes