SA WG2 Meeting #144e	S2-2102190

# April 12th – April 16th, 2021 ; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0547 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of AF guidance to PCF determination of proper URSP rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implementing in TS 23.503 the TR 23.748 conclusion clause 9.1.1 to allow AF to guide PCF determination and delivery to UE(s) of proper URSP rules: - TS 23.503 is referring to TS 23.548 and TS 23.548 to TS 23.503, but the description is missing. The description is also aligned with proposal in discssion paper S2-2102189 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.6.2.2 updated to describe how PCF may use information stored in the UDR using the Nnef_ServiceParameter service operation as input to create URSP rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 EC feature not completely defined which may lead to misalignments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 2608 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.6.2.2	Configuration and Provision of URSP

The UE may be provisioned with URSP rules by PCF of the HPLMN. When the UE is roaming, the PCF in the HPLMN may update the URSP rule in the UE. For URSP rules, the UE shall support the provisioning from the PCF in the HPLMN, as specified in TS 24.501 [22]. In addition, the UE may be also pre-configured with URSP rules (e.g. by the operator).

Only the URSP rules provisioned by the PCF is used by the UE, if both URSP rules provisioned by the PCF and pre-configured URSP rules are present. If no URSP rule is provisioned by the PCF, and the UE has pre-configured rules configured in both the USIM and ME, then only the pre-configured URSP rules configured in the USIM is used.

If the PCF receives application guidance for URSP determination that may apply to a given UE from UDR as specified in TS 23.502 [3] clause 4.15.6.7, the PCF may verify the requested parameters (which are described in clause 4.15.6.10 in TS 23.502 [3]) with regards to the existing URSP rules and (re-)compose the URSP rules for the UE as described below:

-	Application traffic descriptor and traffic matching priority are used to set the URSP Traffic Descriptor (e.g. Destination FQDNs or a regular expression in the Domain descriptor) and determine the URSP precedence in the URSP rule (defined in Table 6.6.2.1-2);

NOTE 1: when multiple Edge Computing specific parameters for the same application are received, the PCF decides the traffic matching priority Rule precedence value of the URSP rule (defined in Table 6.6.2.1-2). The PCF can make the decision based on Longest Match Algorithm when it has received rules with the same  traffic matching priority, for example, when detecting a FQDN: a rule targeting FQDN = toto.example.com has higher precedence than a rule targeting FQDN = *.example.com.

-	Each Route selection parameter is used to set a Route Selection Descriptor as follows:

-	 DNN and S-NSSAI from the Route selection parameter are used to set the DNN selection, Network Slice selection components in the Route Selection Descriptor of the URSP rule, respectively (defined in Table 6.6.2.1-3);

-	 Route selection precedence is used to set the Route Selection Descriptor Precedence in the Route Selection Descriptor (defined in Table 6.6.2.1-3);

-    The spatial validity condition for the Route selection precedence if any are used to set the Location Criteria in the Route Selection Descriptor of the URSP rule (defined in Table 6.6.2.1-3). As an altermative, PCF can   apply the spatial validity conditions itself and provide new URSPs as user moves instead.

END OF CHANGES