SA WG2 Meeting #144E (e-meeting)	S2-2102203

Elbonia, April. 12 – 16, 2021

Source:	Ericsson

Title:	Clause 1 and 4.1 update of TS 23.304 v0.1.0

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe/Rel.17

Abstract of the contribution: This contribution proposes an update of the Scope description in clause 1 and the General description in clause 4.1 to reflect the decision to add ProSe UE-to-Network Relay functionality and remove UE-to_UE functionality from release 17.

# Discussion

This contribution reflects the decision of removing ProSe UE-to-UE relay functionality and adding ProSe UE-to-Network Relay functionality into TS 23.304.

# Proposal

It is proposed to update the Scope clause and the General concept clause in TS 23.304 0.1.0.

* * * * First Change * * * *

# 1	Scope

The present document specifies the Stage 2 of the Proximity based Services (ProSe) features in 5GS. ProSe features consist of: ProSe discovery, ProSe Direct Communication and ProSe UE-to-Network Relay.

ProSe discovery identifies that specific ProSe-enabled UEs are in proximity.

ProSe Direct Communication enables establishment of communication paths between two or more ProSe-enabled UEs that are in direct communication range.

ProSe UE-to-Network Relay enables relay communication between NG-RAN and UEs (e.g. for UEs that are out of coverage of the network).

Editor's note: Security aspects of ProSe are to be defined by SA3. The reference to SA3 specification will be added when defined in SA3.

* * * * Second Change * * * *

## 4.1	General concept

Proximity based Services (ProSe) are services that can be provided by the 5GS based on UEs being in proximity to each other.

The 5GS enablers for ProSe include the following functions:

-	ProSe Direct Discovery;

-	ProSe Direct Communication;

-	ProSe UE-to-Network Relay;

* * * * End of Changes * * * *