3GPP TSG-SA WG2 Meeting #144 e-meeting	S2-2102211

# Elbonia, April 12-16, 2021									     			 (was S2-21XXXXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2704 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#6 : Support for Constraints on simultaneous use of a network slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-03-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR supports the case when the UE subscribed S-NSSAI has restictiions on the simultaneous use of some subscribed S-NSSAIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable the UE to reuse the same S-NSSAI information in Release 15 through the introduction of additional configured S-NSSAI Lists. These Lists are derived by the AMF or NSSF based on Subscribed NSSAI lists in the UE profile. Theses subscribed lists are provisioned by the operator according to the restriction on the S-NSSAIs that cant be used simultaneosuy by the UE. The list of Additional Configured NSSAI is returned to the UE in the Registration response. The UE can switch between list of Additional Configured NSSAI at any point in time. To that effect, the UE initiates a Mobility Registration Update and includes the identifier of the Additional Configured NSSAI including the S-NSSAIs the UE wants to use. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the case when the Subscribed S-NSSAI has restrictions on simultaneous use of some S-NSSAIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 ... CR 2613 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### 5.15.X	Constraints on Simultaneous Use of Network Slices

There may be constraints on simultaneous use of some of the subscribed S-NSSAIs. In this case, a UE that indicates that the UE supports constraints on simultaneous use of Network Slices, is allocated by the AMF a list of Additional Configured NSSAI, and each Additional Configured NSSAI includes an identifier of the Additional Configured NSSAI. The AMF derives the different Additional Configured NSSAI lists based on Subscribed NSSAI lists the AMF receives from the UDM. The number of Subscribed NSSAI lists in the user profile depends on the number of subscribed S-NSSAIs that have restrictions on simultaneous use. Only one Subscribed NSSAI list is allowed to be used at any time by the UE. Each Subscribed NSSAI list in the user profile includes the sets of subscribed S-NSSAIs that can be used simultaneously, as well as a Set Identifer.

The AMF acquire the Subscribed NSSAI lists from UDM during Registration procedure, the AMF or the NSSF derives the list of Additional Configured NSSAI and the AMF provides the list to the UE in the Registration response. The value of the identifier of each Additional Configured NSSAI may be set to the same value as the list identifier of the corresponding Subscribed NSSAI or AMF maintains both identifiers. The Additional Configured NSSAI includes both S-NSSAI values for the Serving PLMN as well as the HPLMN. The UE saves the list of Additional Configured NSSAI, for enabling to select which list to use, until the UE is updated with a new list of Additional Configured NSSAI and the AMF save the Subscribed NSSAI lists in the UE context for enforcement. The AMF or NSSF may determine more than one Additional Configured NSSAI list for one list of Subscribed NSSAI e.g. in case there is no AMF Sets in the Serving PLMN that can serve all the S-NSSAIs for the Serving PLMN required for the S-NSSAIs in the Subscribed NSSAI list.

The Registration response includes in addition to the list of Additional Configured NSSAI, the Allowed NSSAI, Mapping Of Allowed NSSAI, rejected S-NSSAIs, Pending NSSAI, Mapping Of Pending NSSAI. The Configured NSSAI for the Serving PLMN and the Mapping Of Configured NSSAI may be omitted as the list of Additional Configured NSSAI replaces them.

The Subscribed S-NSSAIs marked as default S-NSSAI shall all be part of the same Subscribed NSSAI.

The UE can switch between list of Additional Configured NSSAI at any point in time. To that effect, the UE initiates a Mobility Registration Update and includes the identifier of the Additional Configured NSSAI including the S-NSSAIs the UE wants to use.

When the UE selects a new Additional Configured NSSAI, based on operator policy, the network shall release PDU Sessions for S-NSSAIs not in the selected Additional Configured NSSAI list.

* * * * end of change * * * *