3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102219

Elbonia, April 12 – April 16, 2021

Title:	[DRAFT] LS Reply on Server Domain Name Usage for Application Traffic Detection

Response to:	LS (S2-2102102/S4- 210311) on LS on Server Domain Name Usage for Application Traffic Detection from SA4

Release:	Release 16

Work Item:	5GMS3

Source:	[Huawei to be] SA2

To:	SA4

Cc:	CT3

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA4 for the questions. SA2 would like to answer them as below:

1:	When invoking the Nnef_AFSessionWithQoS method, is it possible to use the application identifier (as a reference to a set of PFDs) instead of the flow description?

SA2 Answer: In this release (Rel-16), as described in clause 5.2.6.8 and 5.2.6.9 of TS 23.502, application identifier is not an alternative. The NEF could map the AF Identifier to Application Identifier during the Nnef_AFSessionWithQoS/ Nnef_ChargeableParty service operation, if this is necessary.

3:	When invoking a particular policy using Nnef_AFSessionWithQoS, is it possible to provide the DNN, the S-NSSAI, and/or a reference to the server domain name for the purposes of traffic identification?

NOTE:	The same questions apply to Nnef_ChargeableThird Party API.

SA2 Answer: In this release (Rel-16), as described in clause 5.2.6.8 and 5.2.6.9 of TS 23.502, the DNN, the S-NSSAI, and/or a reference to the server domain name cannot be used as traffic identification as a concrete description of the traffic (that is subject to the special QoS/charging treatment) is required. And this concrete description can only be achieved by using the Flow description(s) parameter (or the application identifier mapped from the AF identifier as described in the first answer).

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 group to

Take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #145E		May 17 – 28, 2021			Elbonia