SA WG2 Meeting #144E	S2-2102294

12 - 16 April, 2021, Electronic, Elbonia

Source:	Ericsson, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell

Title:	UUAA in PDU Session Establishment-Generic

Agenda item:	8.7

Document for:	Approval

WorkItem/Release:	ID_UAS / Rel-17

Abstract of the contribution:  This contribution proposes a procedure for the UUAA in the PDU Session Establishment as agree in the study phase. This is an updated revision of S2-210222r13 from SA2#143e meeting.

# Discussion

The procedures for UUAA at PDN Connection/PDU Session Establishment are introduced in TS23.256 as concluded in the TR23.754.

The below procedure for 5GS is based on the proposed CR S2-2102376 for the new clause "Secondary authorization/authentication using API during the PDU Session establishment" in TS 23.502. This procedure will be updated in accordance to the approved procedure in TS23.502. Same is valid for the EPS interworking procedure. (Related CR for TS 23.501 is in S2-2102375).

# Proposal

It is proposed to add the following to TS 23.256.

* * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] TS 23.501 [2],  and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or TS 23.501 [2].

3GPP UAV ID: Identifier assigned by the 3GPP system and used by external AF (e.g. USS) to identify the UAV. GPSI is used as the 3GPP UAV ID.

Broadcast Remote ID: The capability of providing Remote Identification and Tracking over broadcast radio links.

NOTE:	In the scope of this release, the radio link for Broadcast Remote ID is assumed to utilize radio technologies outside the scope of 3GPP.

CAA (Civil Aviation Administration)-Level UAV Identity: a UAV identity assigned by USS/UTM, and uniquely identifies a UAV at least within the scope of a USS.

Command and Control (C2) Communication: the user plane link to deliver messages with information of command and control for UAV operation from a UAV controller or a UTM to a UAV or to report telemetry data from a UAV to its UAV controller or a UTM.

Networked UAV Controller: a UAV Controller connected to the 3GPP network and connected to the UAV via a 3GPP network.

Non-Networked UAV Controller: a UAV Controller not connected to the 3GPP network and connected to UAV via a transport outside the scope of 3GPP, e.g. internet connectivity or direct wireless communication over a technology outside the scope of 3GPP.

Networked Remote ID: The capability of providing Remote Identification and Tracking to a USS over 3GPP network.

Remote Identification (Remote ID) of UAS: The ability of a UAS in flight to provide identification and tracking information that can be received by other parties, to facilitate advanced operations for the UAS (such as Beyond Visual Line of Sight operations as well as operations over people), assist regulatory agencies, air traffic management agencies, law enforcement, and security agencies when a UAS appears to be flying in an unsafe manner or where the UAS is not allowed to fly. The Remote ID information payload may include Serial Number or Session ID assigned to the UAV, location of the ground-station controller, emergency status indication etc.

Third Party Authorized Entity: is either a privileged Networked UAV Controller, or a privileged Non-Networked UAV Controller, or another entity which gets information on sets of UAV controllers and UAVs from the 3GPP network, and may be connected to the UAV via the Internet; it may be authorized by the UTM to interface with sets of UAV(s).

UAS NF: a 3GPP UAS Network Function for support of aerial functionality related to UAV identification, authentication/authorization and tracking, and to support Remote Identification.

UAS Service Supplier (USS): An entity that provides services to support the safe and efficient use of airspace by providing services to the operator / pilot of a UAS in meeting UTM operational requirements. A USS can provide any subset of functionality to meet the provider's business objectives (e.g., UTM, Remote Identification). In the scope of this specification, the term USS refers to both USS and USS/UTM.

UAS Traffic Management (UTM): a system that can safely and efficiently integrate the flying UAV along with other airspace users. It provides a set of functions and services for managing a range of autonomous vehicle operations (e.g. authenticating UAV, authorizing UAS services, managing UAS policies, and controlling UAV traffics in the airspace).

UAV controller: The UAV controller of a UAS enables a drone pilot to control an UAV.

UAV operator: the entity owning and operating a UAV.

UAS Services: refers to establishment of connectivity for a UAS for communication with USS, for C2, for remote identification, and for UAV location and tracking.

UUAA Aviation Payload: Contains application layer information exchanged between the UAS and the USS and that is transparent to the 3GPP System.

UUAA Autorization Payload: Contains application layer information exchanged between the UAS and the USS and that is transparent to the 3GPP System.

Uncrewed Aerial System (UAS): Composed of Uncrewed Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the control station, the UAV and the network, and for remote identification. An UAS may comprise of a UAV and a UAV controller.

UUAA: UAV USS authentication and authorization procedure of the UAV to ensure that the UAV has successfully registered with a USS and has therefore been authorized for operations by the USS. An UAV is authenticated and authorized by USS via a UUAA procedure with the support of the 3GPP system before connectivity for UAS services (e.g. UAS-USS connectivity) is enabled.

UUAA-MM: the UUAA procedure optionally performed during registration to a 5GS.

UUAA-SM: the UUAA procedure optionally performed (when UUAA-MM is not performed) during the establishment of a PDU session, and performed during the establishment of a PDN connection.

* * * Second Change * * * *

### 5.2.3	UUAA at PDN Connection/PDU Session Establishment

#### 5.2.3.1	General

An UAV uses PDU Sessions or PDN Connections for communication with the USS and for C2 communication with a networked UAV-C. A networked UAV-C is a  UE which  uses existing procedures for establishing PDU Session or PDN Connection for communication with the USS, and the procedures described in this clause do not apply to a networked UAV-C.

This clause describes procedure that applies both for 5GS and EPS, where PDU Session refers to 5GS and PDN Connection refers to EPS.

PDU Session/PDN Connection for UAV communication shall only be established after a UAV has been authenticated and authorized by the USS, as described in clause 5.2.2 and clause 5.2.3.

An UAV may use either:

-	a common PDU Session/PDN Connection for communication with the USS and C2 communication with the UAV-C, or

-	separate PDU Sessions/PDN Connections for communication with the USS and C2 communication with the UAV-C respectively.

If a common PDU Session/PDN Connection is used, the UAV may establish the PDU Session/PDN Connection and enable the C2 communication during the same procedure, or the UAV may establish the PDU Session/PDN Connection for communication with the USS first and then later enable the C2 communication using the PDU Session/bearer modification procedure. In both cases, UAV – UAV-C pairing authorization shall be successfully performed before the C2 communication is enabled as described in clause 5.2.3 and clause 5.2.5.

The UAV shall indicate that the PDU Session/PDN Connection is for the UAV communication and/or C2 communication in the PDU Session Establishment/PDN Connectivity request.

Editor's note:	How the UAVs indicate UAV communication and/or C2 communication, e.g. with an explicit indication or DNN/S-NSSAI combination, is FFS.

During the establishment or modification procedure of the PDU Session/PDN connection for C2 communication, the USS shall provide the 3GPP system with following information:

-	traffic filters for enabling C2 communication.

-	QoS requirements for C2 communication

Editor's note:	How the USS provides C2 QoS requirement through UAS-NF is FFS and may need coordination with SA6.

Clause 5.2.3 defines the USS UAV Authorization/Authentication (UUAA) procedures at PDU Session Establishment in 5GS and the PDN Connection Establishment in the Attach procedure for EPS using the interworking functionality.

The UUAA procedure is based on the "Secondary authorization/authentication using API during the PDU Session establishment" in TS 23.502 [3] and in the below figures and text, UUAA relevant information is added. Messages and procedure step descriptions are taken from TS 23.502 [3] for explanatory purposes only. These descriptions are in italic text and shall not be modified by the UUAA procedures. It cannot be assumed that the messages and procedure step descriptions that are taken from TS 23.502 [3] will be updated when modifications or corrections are performed for TS 23.502 [3]. If there are any discrepancies for these messages and procedure step descriptions TS 23.502 [3] takes precedence.

#### 5.2.3.2	USS UAV Authorization/Authentication (UUAA) during the PDU Session Establishment for 5GS

The USS UAV Authorization/Authentication (UUAA) is triggered by the SMF during the PDU Session Establishment, specified in TS23.502 [3], clause 4.3.2.2. The UUAA is triggered by the SMF based on the SM subscription data obtained from UDM, if the UE provides the CAA-level UAV ID in the PDU Session establishment request.

Figure 5.2.3.2 -1: UUAA during PDU Session Establishment

The procedure assumes that the UE/UAV has already registered on the AMF.

- Steps 1 - 5 as in 3GPP TS23.502[3] figure 4.3.2.2.1-1.

The UAV includes the CAA-Level UAV ID and may include the USS address and a UUAA Aviation Payload, etc. in a container provided by the UE in the PDU Session Establishment request.

NOTE 1: The definition of the contanier is for stage 3 to specify.

1.	This is step 1 to 3 in 3GPP TS 23.502 [3] clause 4.3.2.x	Secondary authorization/authentication using API during the PDU Session establishment.

The SMF invokes Nnef_Auth_Request, including the DN specific identity, external authenticating server address if it was provided by the UE, GPSI, PEI if available and UE IP Address.

In addition to above parameters the CAA-level UAV ID, USS address and UUAA Aviation Payload, etc. are included in the Nnef_Auth_Request.

The UAS NF/NEF selects a USS based on either CAA-Level UAV ID or the USS address as described in clause 4.4.2.

Editor's Note: How the 3GPP System shall handle an initial PDU Session Establishment Request from a UAV if UUAA has been performed at Registration is FFS.

2.	From UAS NF/NEF to USS: T8_Auth_Request forwarding the received information from the SMF.

3.	All steps 3a to 3h are optional.

3q.	From USS to UAS NF/NEF: T8_Auth_Response is sent depending on required authentication/authorization method used.

3a.	The authentication response from the AF shall include GPSI and may include an authentication message based on authentication method used, that is forwarded transparently to UE over NAS SM transport message. The AF may also include a callback URI to be used by the NEF for subsequent authentication requests.

3b.	The NEF sends Nnef_Auth_Resp message towards the SMF including the GPSI and the authentication message sent by the AF

3c.	SMF transfers the authentication message received in a DN Request Container from NEF towards the UE.

In non-roaming and LBO cases, the SMF invokes the Namf_Communication_N1N2MessageTransfer service operation on the AMF to transfer the DN Request Container information within N1 SM information sent towards the UE.

In the case of Home Routed roaming, the H-SMF initiates a Nsmf_PDUSession_Update service operation to request the V-SMF to transfer DN Request Container to the UE and the V-SMF invokes the Namf_Communication_N1N2MessageTransfer service operation on the AMF to transfer the DN Request Container information within N1 SM information sent towards the UE. In Nsmf_PDUSession_Update Request, the H-SMF additionally includes the H-SMF SM Context ID.

Editor’s Note: The interaction will be further updated once TS 23.502 is updated to include impacts from this work.

3d.	The AMF sends the N1 NAS message to the UE.

3e.	The UE responds with N1 NAS message containing DN Request Container information.

3f.	AMF informs the SMF by invoking the Nsmf_ PDUSession_UpdateSMContext service operation. The SMF issues an Nsmf_PDUSession_UpdateSMContext response.

In the case of Home Routed roaming, the V-SMF relays the N1 SM information to the H-SMF using the information of PDU Session received in step 3c via a Nsmf_PDUSession_Update service operation.

3g.	The SMF invokes Nnef_Auth_Req, including the authentication message information provided by the UE in step 3f. The request shall contain the DN specific identity and the GPSI of the UE. The SMF may include the PEI and UE IP address in the request.

3h.	The NEF invokes authentication request towards the external AF using the callback URI received in step 3a. The authentication request shall contain the authentication message provided by the UE, the GPSI and the DN specific identity of the UE. The authentication request may also include the PEI and the UE IP address if were provided by the SMF in Nnef_Auth_Req.

4.	(final) Authentication Response, shall include: GPSI, and a final authentication message (e.g., indicating success or failure) based on authentication method used, that is forwarded transparently to UE over NAS SM transport message.

The USS validates the request on application level e.g. including CAA-Level UAV ID and GPSI and optionally PEI. The USS notifies the UAS NF/NEF on the authentication/authorization result including Authentication/Authorization result, requested policy information,  a UUAA Authorization Payload.
From USS to UAS NF/NEF: T8_Auth_Response.

NOTE 2:  The USS stores a mapping between CAA-Level UAV ID and the External Identifier. The External Identifier and/or UAV IP Address can be used at a later point by the USS for accessing various services exposed by 3GPP network e.g. location information retrieval, monitoring event configuration etc.
The External Identifier and/or UAV IP Address can be used at a later point by the USS for requesting dedicated policies for e.g. C2, etc.

5.	The NEF confirms the successful authentication/authorization of the PDU Session. The NEF may provide:

-	an SM PDU DN Response Container to the SMF to indicate successful authentication/authorization;

-	DN Authorization Data provided by the AF in step 4;

-	an IP address (or IPV6 Prefix) for the PDU Session.

6.	The PDU Session establishment continues and completes. In the step 7b of the Figure 4.3.2.2.1-1, if the SMF receives the DN Authorization Profile Index in DN Authorization Data from the NEF, it sends the DN Authorization Profile Index to retrieve the PDU Session related policy information (described in TS 23.503 [20] clause 6.4) and the PCC rule(s) (described in TS 23.503 [20] clause 6.3) from the PCF. If the SMF receives the DN authorized Session AMBR in DN Authorization Data from the NEF, it sends the DN authorized Session AMBR within the Session AMBR to the PCF to retrieve the authorized Session AMBR (described in TS 23.503 [20] clause 6.4).

The UUAA result is transferred from SMF to the UAV/UE in a UUAA Authorization Payload within a container as in step 11, 12 and 13 in 3GPP TS 23.502 [3] figure 4.3.2.2.1-1.

NOTE 3: The definition of the contanier is for stage 3 to specify.

NOTE 4:	If C2 information is available from USS during the initial PDU Session Establishment procedure the SMF can interact with the PCF to set up PCC rule(s) for the C2 communication.

#### 5.2.3.3	USS UAV Authorization/Authentication (UUAA) during default PDN connection at Attach for EPS

In the figure 5.2.3.3-1 two options are specified for the execution of the UUAA. Option 1 (i.e. step 2 in figure 5.2.3.3-1) can be used if the timing of the UUAA is not seen as an issue to perform the Attach procedure. Option 2 (i.e. step 4 and 6 in figure 5.2.3.3-1) has to be used if the timing for the UUAA is seen as too long and may have negative effects on the Attach procedure e.g. the Attach procedure can time-out before response have been received from USS.

Figure 5.2.3.3-1: UUAA during Attach procedure in EPS

0.	Steps 1 - 13 in TS23.401 [5] figure 5.3.2.1-1 and steps 1 - 2 in TS23.502 [3] figure 4.11.1.5.2-1 or clause 4.11.2.4.1 in TS23.502 [3].
UAV/UE sends Attach Request including the CAA-Level UAV ID and may include the USS address and the UUAA Aviation Payload, etc. in the PCO to the SMF+PGW-C. 
The MME may determine the UAV has an aerial subscription and selects the Default APN for connectivity with the USS.

NOTE 1: The definition of the contanier is for stage 3 to specify.

1.	[OPTION 2] SMF+PGW-C configures an Access Control List (ACL) in UPF+PGW-U to stop any traffic over the default PDN Connection.

2.	[OPTION 1] UUAA is performed as described in steps 1, 2, 4 and 5 in figure 5.2.3.2-1.

Editor's note: Optional step 3 in figure 5.2.3.2-1 is not supported in current release of EPC.

3.	Steps 14 - 22 in TS23.401 [5] figure 5.3.2.1-1 and steps 3 - 6 in TS23.502 [3] figure 4.11.1.5.2-1 or clause 4.11.2.4.1 in TS23.502 [3].

4.	[OPTION 2] UUAA is performed as described in steps 1, 2, 4 and 5 in figure 5.2.3.2-1.

5.	Steps 23 - 24 in TS23.401 [5] figure 5.3.2.1-1.

6.	[OPTION 2] The PCO including the UUAA result is transferred from SMF+PGW-C to UAV/UE within a UUAA Authorization Payload in Update Bearer Request and Downlink NAS Transport (step 6a - 6c). The UAV/UE confirms the update in steps 6d - 6f.

* * * End of Change * * * *