3GPP TSG-SA2 Meeting # 144E (e-meeting)	S2-2102314

# Elbonia, February 24 – March 09, 2021										   (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3633 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Function Description for Paging Cause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introducing Paging Cause feature description for Multi-USIM device as agreed in TR 23.761. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description for Paging Cause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing function description for Paging Cause. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

4.3.x.3	Paging Cause indication for voice service

The UE and the network may support the Paging Cause feature. The Paging Cause feature is applied for the UE indicating MUSIM mode Indication only and it includes to indicate a Voice Service Indication as the paging cause to the UE only when the paging is triggered by voice service (MMTel voice or CS domain voice).

If receiving Multi-USIM Mode Indication from the UE, the MME supporting Paging Cause feature should determine the type of the downlink data or signalling which triggers the Paging and decides whether include Voice Service Indication in the S1AP Paging message. The MME should provide the Voice Service Indication in the S1AP Paging message only when it has received the Multi-USIM Mode Indication from the UE and detects that the downlink data or signalling which triggers the Paging message is related to voice service, e.g. based on the Paging Policy Indication dedicated for voice service in clause 4.9 or CS call indicator from the MSC.

Upon reception the Voice Service Indication in S1AP Paging Message, the eNodeB supporting the Paging Cause feature should include the voice service indication as the paging cause in the paging message to the UE.

The UE which supports the Paging Cause feature is capable of understanding the paging is triggered by voice service upon reception of the Voice Indication in the Uu Paging message and it is also capable of distinguishing the Paging message is triggered by the non voice or unknown (e.g. due to the network doesn't support Paging cause) .

Editor Note: how the UE can distinguish the Paging message is triggered by the non voice or unknown (e.g. due to the network doesn't support Paging cause) depends on RAN’s decision.

****************** END CHANGES ***************