3GPP TSG-SA2 Meeting #144-e	S2-2102335

# Online, Elbonia, 12th Apr 2021 - 16th Apr 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0274 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignments on analytics subsets, accuracy levels, and ordering of results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR implements some conclusions of TR 23.700-91 for KI#11 and KI#17 into clauses 6.10 to 6.13 (as done for other clauses by CR#0255 and other CRs): - The principle of selecting the relevant subsets in the output is useful in order to reduce the amount of output data (see TR 23.700-91 clause 8.11.3: "Additional Analytics filters shall be standardized on a per-analytics basis, in order to narrow down the scope of analytics, thus reducing signalling both for analytics provision and data collection"). - Accuracy depends on the subsets of analytics produced (continuous versus discrete values, vectors) per analytics ID. Accuracy constraints, in the scope of the NWDAF, are requirements on the quality of expected outputs. - Accuracy levels with finer grain can guide the choice of relevant data collection and calculation models, and the amount of resources (signalling, calculation) to be mobilized. The principle of additional accuracy levels together with finer grain per analytics subsets is part of conclusion of TR 23.700-91 (clause 8.17). - As concluded in TR 23.700-91 clause 8.11.3, criteria for ordering of results (together with maximum number or results) are useful to specify the desired order of results in a list of items according to certain criteria, in order to obtain the most important items in a list with less signalling. - Editorial errors were found in the specification while writing the CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The "list of analytics subsets that are requested" parameter of Analytics Filter Information is added to clauses 6.10.1, 6.11.1 and 6.12.1. Possible analytics subsets are defined in respective analytics output tables with a Note. - A parameter "Accuracy level per analytics subset" is added to 6.10.1 and 6.11.1. Possible analytics subsets are defined in respective analytics output tables with a Note. - The parameter "preferred level of accuracy of the analytics" is added to 6.10.1 and 6.13.1. - A parameter "preferred order of results" is added to clause 6.10.1, 6.11.1 and 6.13.1 in order to permit to select the ordering of results on the most useful criteria. - Indentation is fixed in tables of clause 6.10.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The impossibility to limit the output by selection of relevant subsets, the limited number of accuracy levels, as well as the lack of definition of semantics may lead to excess or deficiency in the quality of provided analytics as well as high signalling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.3.1, 6.10.3.2, 6.11.1, 6.11.3, 6.12.1, 6.12.3, 6.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.10.1	General

Dispersion analytics identifies the location (i.e. areas of interest, TAs, cells) or network slice(s) where a UE, or a group of UEs disperse most (if not all) of their data volume and sessions transactions (i.e. MM and SM messages).

The NWDAF shall be able to provide dispersion statistics or predictions and shall be able to collect UE dispersion related information from NFs.

Dispersion is the percentage of activity that a UE, or group of UEs, generates at a location or a slice during a period of interest. This enables the operator to rank hot locations or slices by the various activities and to identify the top contributors (i.e. UEs) for that activity. When heavy UEs at a particular location or slice are detected/predicted and reported to a policy entity, their QoS attributes, or access to an area of interest or a slice can then be limited.

-	Data dispersion - The percentage of data traffic volume that a UE, or a group of UEs, generated at a location or in a slice during the period of interest.

-	Transaction dispersion - The percentage of MM and SM messages that a UE, or a group of UEs, generated at a location or in a slice during the period of interest.

The operator may classify and assign one of the three mobility classes per dispersion characteristics for a UE or a group of UEs as fixed, camper or traveller. The classification is based on either Data dispersion or Transaction dispersion.

-	Data-Classification - fixed, camper, traveller data-classification per thresholds assigned by the operator.

EXAMPLE:	When a UE disperses, during the period of observation, most (threshold=95%) of its data at a location or a slice, the dispersion data-classification of the UE at that location or slice is "fixed".

-	Transaction-Classification - fixed, camper, traveller transactions-classifications per thresholds assigned by the operator. Top-Heavy UE is one who's dispersion percentile rating, at a location or a slice, is higher than a threshold value defined by the operator (e.g. 0.9 percentile), see percentile ranking in the output tables, clause 6.10.3.

EXAMPLE:	When a UE disperses, during the period of observation, (threshold=40%) of its session transactions at a location or a slice, the dispersion transaction-classification of the UE is camper at that location or slice.

Based on the dispersion analytics, the analytics consumer can determine that a data or transaction hot spot is formed when the amount of data or amount of transactions dispersed by most of the UEs at the area of interest or slice exceeds a certain data or transaction threshold established through trend analysis or operator policy.

The consumer of dispersion analytics may indicate in its request:

-	Analytics ID set to "UE Dispersion Analytics"

- Dispersion Analytics type set to Data Dispersion Analytics (DDA) or Transaction Dispersion Analytics (TDA).

-	Target of analytics reporting: single UE, any UE, or a group of UEs.

-	Analytics Filter Information:

-	optional list of TA(s), Area(s) of Interest, Cells, or S-NSSAI, Top-Heavy UEs, Fixed UEs, Camper UEs; and

-	optional list of analytics subsets that are requested (see clause 6.10.3);

-	Preferred level of accuracy of the analytics;

-	Accuracy level per analytics subset (see clause 6.10.3);

-	Preferred order of results for the list of UE Dispersion Analytics information:

-	ordering criterion: "start time", "duration", "dispersion" or "ranking"; and

-	order: ascending or descending;

-	Analytics target period indicating the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

NOTE:	Care must be taken with regards to load when requesting analytics for "Any UE". This could be achieved by utilizing event filters (e.g. Area of Interest for AMF) and possible analytics filters including Top-Heavy UEs, and/or fixed and/or camping UEs and/or Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

# * * * * Next change

#### 6.10.3.1	Data Dispersion Analysis

The data dispersion analytics results provided by the NWDAF can be UE data dispersion statistics as defined in table 6.10.3.1-1 and/or UE data dispersion predictions as defined in table 6.10.3.1-2.

Table 6.10.3.1-1: Data dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE dispersed its data. |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data dispersed (NOTE X) | Data volume dispersed at this location and application, if application ID is provided. |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >> Data ranking (NOTE X) | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group at the location and observation time. (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

Table 6.10.3.1-2: Data dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytics target period. |
| >> UE location | TA or cells where the UE is predicted to disperse its data. |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data dispersed (NOTE X) | Data volume dispersion prediction at this location and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location). |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Data ranking (NOTE X) | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

The data dispersion analytics results provided by the NWDAF can also be for a UE or group of UEs data dispersion statistics at a given slice as defined in Table 6.10.3.1-3 and/or dispersion predictions as defined in Table 6.10.3.1-4:

Table 6.10.3.1-3: Data dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data dispersed (NOTE X) | Data volume dispersed at this slice and application, if application ID is provided. |
| >> Data classification | Data mobility classification for slice: fixed, camper, traveller. |
| >> Data ranking (NOTE X) | Ranking of UE data usage at this slice. (See NOTE 2). |
| >> Data percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

Editor note:	Consider the addition of ">>Bandwidth" into tables 6.10.3.1-3 and 6.10.3.1-4, which is the utilized bandwidth at the slice calculated by the NWDAF as the volume over the period. See also SA WG5 Liaison letter S2-2008361 (S5-205436).

Table 6.10.3.1-4: Data dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data dispersion (NOTE X) | Data volume dispersion prediction at this slice and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this slice). |
| >> Data classification | Data mobility classification for this slice: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Data ranking (NOTE X) | Ranking of UE data usage at this slice. See NOTE 2. |
| >> Data percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

NOTE:	The Application ID in the Data Dispersion Analytics is optional. When the Application ID is missing, the Data Dispersion Analytics is applied across all the applications in an AoI or a slice.

#### 6.10.3.2	Transactions Dispersion Analysis

The transaction (MM and MS messages) dispersion analytics results provided by the NWDAF can be UE transaction dispersion statistics as defined in Table 6.10.3.2-1 and/or UE transaction dispersion predications as defined in Table 6.10.3.2-2:

Table 6.10.3.2-1: Transaction dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE dispersed its transactions. |
| >> Transaction dispersed (NOTE X) | Transactions amount dispersed at this location. |
| >> Transaction classification | Transactions mobility classification for this location: fixed, camper, traveller. |
| >> Transaction ranking (NOTE X) | Ranking of transactions amount at this location. See NOTE 2. |
| >> Transaction percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

Table 6.10.3.2-2: Transaction dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of predicted time slots. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytic target period. |
| >> UE location | TA or cells where the UE is predicted to disperse its transactions. |
| >> Transaction dispersion (NOTE X) | Transaction amount predicted to be dispersed at this location. |
| >>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location). |
| >> Transaction classification | Transaction mobility classification for this location: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Transaction ranking (NOTE X) | Ranking of transactions amount at this location. See NOTE 2. |
| >> Transaction percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

The transaction dispersion analytics results provided by the NWDAF can be UE or group of UEs transaction dispersion statistics at a given slice as defined in table 6.10.3.2-3 and/or dispersion predictions as defined in table 6.10.1.4.2-4:

Table 6.10.3.2-3: Transaction dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed its transactions. |
| >> Transaction dispersed (NOTE X) | Transactions amount dispersed at this location. |
| >> Transaction classification | Transaction mobility classification for slice: fixed, camper, traveller. |
| >> Transaction ranking (NOTE X) | Ranking of transactions amount at this slice. See NOTE 2. |
| >> Transaction percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

Table 6.10.3.2-4: Transaction dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE1). |
| Time slot entry (1..max) | List of predicted time slots. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its transactions. |
| >> Transaction dispersion (NOTE X) | Transaction volume to be dispersed at this slice. |
| >>> Confidence | Confidence of this prediction (i.e. transactions to be dispersed at this slice). |
| >> Transaction classification | Transaction mobility classification for this slice: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Transaction ranking (NOTE X) | Ranking of transactions amount at this slice. See NOTE2. |
| >> Transaction percentile ranking (NOTE X) | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

# * * * * Next change

### 6.11.1	General

The NWDAF provides WLAN performance analytics to a service consumer NF. The analytics results are generated based on the data from other 5GC NFs and OAM. The analytics results, provided in the form of statistics or predictions, contain quality and performance of WLAN connection of UE according to UE location and SSID. The consumer can request either one-time or continuous reporting.

The service consumer may be an NF (e.g. PCF).

If a service consumer is PCF, the WLAN performance analytics can be used to update WLANSP as defined in TS 23.503 [4].

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID set to "WLAN performance";

-	Target of Analytics Reporting: a UE, a group of UEs, or any UE;

-	Analytics Filter Information containing:

-	Area of Interest (list of TA or Cells);

-	SSID(s);

-	BSSID(s); and

-	optional list of analytics subsets that are requested (see clause 6.11.3);

-	An Analytics target period indicates the time period over which the statistics or prediction are requested, either in the past or in the future;

-	Optional maximum number of objects;

-	Preferred level of accuracy of the analytics;

-	Accuracy level per analytics subset (see clause 6.11.3);

-	Preferred order of results for the list of WLAN performance information:

-	ordering criterion: "start time", "duration", "number of UEs", "RSSI", "RTT" or "Traffic volume"; and

-	order: ascending or descending; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

If the Target of Analytics Reporting is any UE, then the Analytics Filter should at least include Area of Interest or SSID(s) or BSSID(s).

# * * * * Next change

### 6.11.3	Output Analytics

The NWDAF generates WLAN performance analytics. Depending on the Analytics Target Period, the output consists of statistics or predictions. The detailed information provided by the NWDAF is defined in Table 6.11.3-1 for statistics and Table 6.11.3-2 for predictions.

Table 6.11.3-1: WLAN performance statistics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell Ids |
| List of Analytics per SSID | SSIDs of WLAN access points deployed in the Area of Interest |
| > Time slot entry (1…max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI | Measured RSSI (NOTE X) |
| >> RTT | Measured RTT (NOTE X) |
| >> Traffic Information | UL/DL data rate, Traffic volume (NOTE X) |
| >> Number of UEs | Number of UEs observed for the SSID (NOTE X) |
| NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

Table 6.11.3-2: WLAN performance predictions

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell Ids |
| List of Analytics per SSID | SSIDs of WLAN access points deployed in the Area of Interest |
| > Time slot entry (1…max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI | Predicted RSSI (NOTE 1) |
| >> RTT | Predicted RTT (NOTE 1) |
| >> Traffic Information | Predicted UL/DL data rate, Traffic volume (NOTE 1) |
| >> Number of UEs | Number of UEs predicted for the SSID (NOTE 1) |
| >> Confidence | Confidence of the prediction |
| NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "accuracy level per analytics subset". |  |

# * * * * Next change

### 6.12.1	General

According to the Session Management Congestion Control (SMCC) mechanisms, i.e. DNN based congestion control defined in clause 5.19.7.3 of TS 23.501 [2] and S-NSSAI based congestion control defined in clause 5.19.7.4 of TS 23.501 [2], the SMF that is applying or has applied the SMCC mechanism does not store any historical information related to which UEs is/was subject to backoff timer settings. Therefore, fairness to apply the SMCC cannot be considered nor guaranteed. For example, among UEs that use a PDU Session associated with S-NSSAI#1, some of them may have experienced the S-NSSAI based high level of congestion control (e.g. receiving NAS SM reject messages with a long backoff timer) while some of the UEs may have experienced the S-NSSAI based low level congestion control (e.g. receiving NAS SM reject messages with a short backoff timer), within a specific period. The backoff timer provided to each UE can vary, e.g. up to 70 hours.

The SMF service consumer may request the NWDAF to provide Session Management Congestion Control Experience (SMCCE) statistical analytics for a specific DNN and/or S-NSSAI. The SMF uses potential congestion condition as a trigger to request the SMCCE analytics from the NWDAF.

The request by the SMF includes mainly the following parameters:

-	Analytics ID set to "Session Management Congestion Control Experience".

-	Target of Analytics Reporting: one or more SUPI(s).

NOTE 1:	The UE(s) contained in the Target of Analytics Reporting. These are UE(s) that have the PDU Session for the DNN and/or S-NSSAI indicated by Analytics Filter Information.

-	Analytics Filter Information containing:

-	DNN and/or S-NSSAI, and

-	optional list of analytics subsets that are requested (see clause 6.12.3);

-	Analytics target period: the time window for which the statistics are requested.

NOTE 2:	Predictions are not provided as output for the Session Management Congestion Control Experience analytics.

# * * * * Next change

### 6.12.3	Output Analytics

The NWDAF outputs the SMCCE statistical analytics. The detailed statistical information provided by the NWDAF is defined in Table 6.12.3-1.

Table 6.12.3-1: SMCCE statistics

| Information | Description |
| --- | --- |
| List of SMCCE Analytics (1..max) |  |
| > DNN | DNN that SMCC is applied. |
| > S-NSSAI | S-NSSAI that SMCC is applied. |
| > List of UEs classified based on experience level of SMCC | One, or more than one, of the following lists (SUPI is used to identify a UE). |
| >> List of high-experienced UEs (NOTE X) | A list of UEs whose experience level of SMCC for specific DNN and/or S-NSSAI is high. |
| >> List of medium-experienced UEs (NOTE X) | A list of UEs whose experience level of SMCC for specific DNN and/or S-NSSAI is medium . |
| >> List of low-experienced UEs (NOTE X) | A list of UEs whose experience level of SMCC for specific DNN and/or S-NSSAI is low. |
| NOTE: The high/medium/low-experience thresholds values are operator defined. NOTE X: This information element is an analytics subset that can be used in "list of analytics subsets that are requested". |  |

# * * * * Next change

### 6.13.1	General

This clause describes the Redundant Transmission Experience related analytics. This analytics may be used by the SMF to determine whether redundant transmission shall be performed, or (if it had been activated) shall be stopped.

The service consumer may be a NF (e.g. SMF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Redundant Transmission Experience".

-	The Target of Analytics Reporting can be a single UE, any UE, or a group of UEs.

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the list of Redundant Transmission Experience:

-	ordering criterion: "start time", "duration" or “Redundant Transmission Experience”; and

-	order: ascending or descending;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

# * * * * End of CR * * * *