3GPP TSG-SA2 # 144E (e-meeting)	S2-2102337

# April.12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0276 | rev | 0 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Experience Analytics outputs RAT Type and Frequency information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-3-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At SA2#143 emeeting, the expansion on Service Experience has FFS notes for the source of RAT Type and frequency for the NWDAF. And because of input issue, the expansion of output on RAT Type and freuqency is pending too. This CR is to complete the task. - According to DP S2-2102331 and CR S2-2102333, the SMF supports RAT Type Change event in Nsmf_EventExposure service. So, the NWDAF may subscribed for RAT Type information from SMF as input for Service experience. - According to LS-in from SA5 (S2-2100159), “when given a cell ID (cellLocalId), the frequencies (arfcnDL and arfcnUL) this cell is using are available” in OAM. In current operatores’ network, a cell is usually statically configured with 1 UL (or 2 UL if SUL is configured) and 1 DL frequencies. So even it is not supported for the NWDAF to retrireve frequency information from OAM in real time manner yet, as stated in SA5’s LS, it is practically to have locally configured reference of Cell-frequency mapping in the NWDAF for such information because it is not time variant. If the OAM supports providing frequency and other information to NWDAF in future release, the NWDAF will be able to retrieve that from the OAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR: [Change -1] clause 6.4.2 - Extend input data of Service Experience with “RAT type” and “Frequency”, and remove some Editor’s note. [Change-2] clause 6.4.3 - Extend output data of service experience with “RAT Type” and “Frequency”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Insufficient input and output data for service experience analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

6.4.2	Input Data

The service data and performance data collected from the AF, the network data from other 5GC NFs and the network data from OAM for observed service experience are defined in Table 6.4.2-1, 6.4.2-1a, Table 6.4.2-2, Table 6.4.2-3 and Table 6.4.2-4 respectively.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| IP filter information | AF | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE per service flow as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS for any kind of service including those not related to video or voice. |
| Timestamp | AF | A time stamp associated to the Service Experience provided by the AF, mandatory if the Service Experience is provided by the ASP. |
| Application Server Instance | AF | The IP address or FQDN of the Application Server that the UE had a communication session when the measurement was made. |

NWDAF subscribes to the service data from AF in the Table 6.4.2-1 either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID) as defined in TS 23.502 [3], or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID) where NEF translates the Area of Interest into geographic zone identifier(s).

NOTE:	When the Service Experience is expressed as a customized MOS, the customized MOS may be defined by the content provider or by the MNO and may be based on the nature of the targeted service type (e.g. web browsing, gaming, augmented reality, V2X, SMS).

Table 6.4.2-1a: Performance Data from AF

| Information | Source | Description |
| --- | --- | --- |
| UE identifier | AF | IP address of the UE at the time the measurements was made. |
| UE location | AF | The location of the UE when the performance measurement was made. |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| IP filter information | AF | Identify a service flow of the UE for the application. |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Application Server Instance address | AF/NEF | The IP address/FQDN of the Application Server that the UE had a communication session when the measurement was made. |
| Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: Average Packet Delay, Average Loss Rate and Throughput. |
| Timestamp | AF | A time stamp associated to the Performance Data provided by the AF. |

NWDAF subscribes to the performance data from AF in the Table 6.4.2-1a either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = Performance Data, Event Filter information = Area of Interest, Application ID) as defined in TS 23.502 [3], or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Performance Data, Event Filter information = Area of Interest, Application ID) where NEF translates the Area of Interest into geographic zone identifier(s).

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| (list of)SUPI(s) | AMF | If UE IDs are not provided as target of analytics reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| UPF info | SMF | UPF ID/address/FQDN information for the UPF serving the UE. |
| DNAI | SMF | Identifies the access to DN to which the PDN session connects. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF | The observed number of packet retransmission. |

NOTE 1:	How NWDAF collects QoS flow Bit Rate, QoS flow Packet Delay, Packet transmission and Packet retransmission information from UPF is not defined in this Release of the specification.

NOTE 2:	Care shall be taken with regards to load and major signalling caused when requesting Any UE. This could be achieved via utilization of some event filters (e.g. Area of Interest for AMF), Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation with the following Event IDs as input parameters:

-	AMF Source: Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest).

-	SMF Source: Nsmf_EventExposure_Subscribe (Event ID = QFI allocation).

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |
| The mapping information between cell ID and frequency | OAM | The mapping information between cell ID and frequency, as specified in clause 4.3.5 of TS 28.541 [22]. |
| Cell Energy Saving State | OAM | List of the cells which are within the area of interest and are in energy saving state, as specified in clauses 3.1 and 6.2 of TS 28.310 [24]. |
| Frequency | OAM | The frequency the UE camps on. |

NOTE: How NWDAF collects Frequency information from OAM in table 6.4.2-3 is not defined in this Release of the specification. The mapping information between cell ID and frequency can be statically configured in NWDAF by the operators.

NWDAF subscribes the network data from OAM in the Table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

Table 6.4.2-4: UE level Network Data from 5G NF related to the Service Experience

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). |
| RAT Type | SMF | The RAT type the UE camps on. |

The Event Filters for the service data collection from SMF, AMF and AF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

* * * 2nd Change * * *

6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Slice instance service experiences (0…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice of the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| >UPF Info | Indicating UPF serving the UE. |
| >DNAI | Indicating which DNAI the UE service uses/camps on. |
| >DNN | DNN for the PDU Session which contains the QoS flow. |
| >Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type | Indicatiing the RAT type for the application service experience analytics applies |
| > Frequency | Indicatiing the frequency for the application service experience analytics applies |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Network Slice instance service experiences (0…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice of the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| >UPF Info | Indicating UPF serving the UE. |
| >DNAI | Indicating which DNAI the UE service uses/camps on. |
| >DNN | DNN for the PDU Session which contains the QoS flow. |
| >Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type | Indicatiing the RAT type for the application service experience analytics applies |
| > Frequency | Indicatiing the frequency for the application service experience analytics applies |
| > Probability assertion | Confidence of this prediction. |

NOTE 1:	If multiple Network Slice instances are not deployed for the S-NSSAI or NSI IDs are not available, only one Network Slice instance service experience entry is provided. In that case, the NSI ID is not provided and the Network Slice instance service experience indicates the service experience for the S-NSSAI.

NOTE 2:	The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and with the same value(s) in the service experience information for the slice instance which the application belongs to.

NOTE 3:	If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF.

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

* * *End of Changes * * *