SA WG2 Meeting #144e	S2-2102365

# April 12th – April 16th, 2021 Elbonia                   		 	 				(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2736 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G URLLC Handling redundant PDU Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Convida Wireless, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_SE_RPS |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TEI17 WID objective: If the UE has the knowledge about PDU Session pair information for the redundant PDU Sessions, it is proposed that the UE provides PDU Session pair information to the SMF(s) so that the SMF(s) can provide this information to the NG-RAN that the NG-RAN can eventually use the information for SN selection, also gNB CU/DU Selection. This would allow the two PDU Sessions to be independently established without any constraints on the selected SMF(s). If the UE releases one of the redundant PDU sessions and establish the third PDU Session, the PDU Session pair information can be used for coordination with the new established PDU session. Backward compatibility with Rel-16 should be considered (e.g. Rel-17 UE(s) in Rel-16 network). In addition, how the UE obtains the knowledge of PDU Session pair information for the redundant PDU Sessions needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are proposed: - UE provides PDU Session pair identifier for each PDU Session. - SMF forwards the PDU Session pair identifier to NG-RAN - The two PDU sessions remain paired. - This allows the PDU session pair to be handled with two SMFs also more dynamically. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TEI17 WID remains incomplete and RAN3 LS that originated the LS remains unsolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.33.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.33.2	Redundant transmission for high reliability communication

#### 5.33.2.1	Dual Connectivity based end to end Redundant User Plane Paths

In order to support highly reliable URLLC services, a UE may set up two redundant PDU Sessions over the 5G network, such that the 5GS sets up the user plane paths of the two redundant PDU Sessions to be disjoint. The user's subscription indicates if user is allowed to have redundant PDU Sessions and this indication is provided to SMF from UDM.

NOTE 1:	It is out of scope of 3GPP how to make use of the duplicate paths for redundant traffic delivery end-to-end. It is possible to rely on upper layer protocols, such as the IEEE 802.1 TSN (Time Sensitive Networking) FRER (Frame Replication and Elimination for Reliability) [83], to manage the replication and elimination of redundant packets/frames over the duplicate paths which can span both the 3GPP segments and possibly fixed network segments as well.

NOTE 2:	The following redundant network deployment aspects are within the responsibility of the operator and are not subject to 3GPP standardization:

-	RAN supports dual connectivity, and there is sufficient RAN coverage for dual connectivity in the target area.

-	UEs support dual connectivity.

-	The core network UPF deployment is aligned with RAN deployment and supports redundant user plane paths.

-	The underlying transport topology is aligned with the RAN and UPF deployment and supports redundant user plane paths.

-	The physical network topology and geographical distribution of functions also supports the redundant user plane paths to the extent deemed necessary by the operator.

-	The operation of the redundant user plane paths is made sufficiently independent, to the extent deemed necessary by the operator, e.g. independent power supplies.

Figure 5.33.2.1-1 illustrates an example user plane resource configuration of dual PDU Sessions when redundancy is applied. One PDU Session spans from the UE via Master NG-RAN to UPF1 acting as the PDU Session Anchor, and the other PDU Session spans from the UE via Secondary NG-RAN to UPF2 acting as the PDU Session Anchor. As described in TS 37.340 [31], NG-RAN may realize redundant user plane resources for the two PDU Sessions with two NG-RAN nodes (i.e. Master NG-RAN and Secondary NG-RAN as shown in Figure 5.33.2.1-1) or a single NG-RAN node. In both cases, there is a single N1 interface towards AMF.

Based on these two PDU Sessions, two independent user plane paths are set up. UPF1 and UPF2 connect to the same Data Network (DN), even though the traffic via UPF1 and UPF2 may be routed via different user plane nodes within the DN.

In order to establish two redundant PDU Sessions and associate the duplicated traffic coming from the same application to these PDU Sessions, URSP or UE local configuration is used as specified in TS 23.503 [45].

NOTE 3:	Using URSP, duplicated traffic from the application, associated to the redundant PDU Sessions, is differentiated by two distinct traffic descriptors, each in a distinct URSP rule. These traffic descriptors need to have different DNNs, IP descriptors or non-IP descriptors (e.g. MAC address, VLAN ID), so that the two redundant PDU Sessions are matched to the Route Selection Descriptors of distinct URSP rules.

The redundant user plane set up applies to both IP and Ethernet PDU Sessions.

Figure 5.33.2.1-1: Example scenario for end to end redundant User Plane paths using Dual Connectivity

Support of redundant PDU Sessions include:

-	UE initiates two redundant PDU Sessions and provides different combination of DNN and S-NSSAI for each PDU Session.

-	The UE may determine the need to duplicate traffic from the application based on URSP Policies.

-	The UE may include a PDU Session Pair Identifier when it establishes each PDU Session so that the network can pair the redundant PDU Sessions.

-	The SMF determines whether the PDU Session is to be handled redundantly. The determination is based on the the indication that redundant PDU Session is required provided by PCF for the PDU Session, if dynamic PCC applies for the PDU Session or the combination of the S-NSSAI, DNN, user subscription and local policy configuration in the SMF if dynamic PCC is not used for the PDU Session. If the PDU session is to be handled redundantly and no PDU Session Pair Identifier is received from the UE, the SMF uses S-NSSAI, DNN to determine the RSN value which differentiates the PDU Sessions that are handled redundantly and indicates redundant user plane requirements for the PDU Sessions in NG-RAN.

-	When a PDU Session Pair Identifier is received from the UE, the SMF forwards the PDU Session Pair Identifier received from the UE to the NG-RAN for more flexible SN and/or gNB CU/DU selection.

-	Operator configuration of UPF selection ensures the appropriate UPF selection for disjoint paths.

-	At establishment of the PDU Sessions or at transitions to CM-CONNECTED state, the RSN parameter or the PDU Session Pair Identifier indicates to NG-RAN that redundant user plane resources shall be provided for the given PDU Sessions by means of dual connectivity. The value of the RSN parameter or the PDU Session Pair Identifier indicates redundant user plane requirements for the PDU Sessions. This request for redundant handling is made by indicating the RSN to the NG-RAN node on a per PDU Session granularity. PDU Sessions associated with different RSN values shall be realized by different, redundant UP resources. Based on the RSN, the PDU Session Pair Identifier and RAN configuration, the NG-RAN sets up dual connectivity as defined in TS 37.340 [31] so that the sessions have end to end redundant paths. When there are multiple PDU Sessions with the RSN parameter set, of different values of RSN, or the same PDU Session Pair Identifier, this indicates to NG-RAN that CN is requesting dual connectivity to be set up and the user plane shall be handled as indicated by the RSN parameter and the associated RAN configuration. If the RSN value or PDU Session Pair Identifier are provided to the NG-RAN, NG-RAN shall consider the RSN value or PDU Session Pair Identifier when it associates the PDU Sessions with NG-RAN UP.

NOTE 4:	The decision to set up dual connectivity remains in NG-RAN as defined today. NG-RAN takes into account the additional request for the dual connectivity setup provided by the CN.

-	Using NG-RAN local configuration, NG-RAN determines whether the request to establish RAN resources for a PDU Session is fulfilled or not considering user plane requirements indicated by the RSN parameter or the PDU Session Pair Identifier parameter by means of dual connectivity. If the request to establish RAN resources for PDU Session can be fulfilled by the RAN, the PDU Session is established even if the user plane requirements indicated by RSN cannot be satisfied. The decision for each PDU Session is taken independently (i.e. rejection of a PDU Session request shall not release the previously established PDU Session). The RAN shall determine whether to notify the SMF if the RAN resources indicated by the RSN parameter or the PDU Session Pair Identifier parameter can no longer be maintained and SMF can use that to determine if the PDU Session should be released.

-	In the case of Ethernet PDU Sessions, the SMF has the possibility to change the UPF (acting as the PSA) and select a new UPF based on the identity of the Secondary NG-RAN for the second PDU Session if the Secondary NG-RAN is modified (or added/released), using the Ethernet PDU Session Anchor Relocation procedure described in clause 4.3.5.8 of TS 23.502 [3].

-	The SMF's charging record may reflect the RSN information.

-	The RSN indication or the PDU Session Pair Identifier parameter is transferred from Source NG-RAN to Target NG-RAN in the case of handover.