3GPP TSG-WG SA2 Meeting #144E 	S2-2102373

# 12th April – 16th April 2021, Elbonia	(revision of S2-210xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	TS 23.256:		Updates to CAA-Level UAV ID Assignment

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract of the contribution:

# 1 Introduction

Current description of the CAA-Level UAV ID assignment in clause 4.4.3 of TS 23.256 assumes that the CAA-Level UAV ID used for Remote Identification contains atleast the following information:

-	An identity unique to the UAV and

-	A routing indentifier

It has been already agreed that "The format of the CAA-Level UAV ID is defined outside 3GPP". Has the above assumption that the CAA-Level UAV ID shall contain a routing information been verified with aviation community? Without verification/confirmation from the UTM community, it would be wrong to design the 3GPP system with isolated assumptions.

Proposal 1:	It is proposed to remove the texts with assumptions that mandates the CAA-Level UAV ID to contain certain information when it is already agreed that the CAA-Level UAV ID format is defined outside 3GPP.

For 3GPP-assisted CAA-Level UAV ID assignment, currently in ts 23.256 the procedure assumes that MNO details are provided during the UAV registration. This will bind the UAV with the MNO subscription. And everytime an UAV operator wants to use a different MNO subscription it has to update the information with the USS. We believe such restrictions should not be put in place without getting the same verified by the aviation community.

"When the UAV registers with the USS before registering to a 3GPP system for UAS services, the UAV operator provides information about the serving PLMN to the USS."

The procedure for 3GPP-assisted CAA-Level UAV ID assignment needs to be first discussed with the aviation community. We should not define a procedure at 3GPP with such assumptions on UAV registration process for assignment of CAA-Level UAV ID.

Proposal 2:	It is proposed to remove the text explaining how the 3GPP assisted CAA-Level UAV ID assignment is done and replace it with an Editor's Note to further study this use case and define a procedure with inputs from USS/UTM and aviation administrations responsible for UAS registration.

This paper also provides additional editorial updatses and some clarification on the scope of SMF and definition of 3GPP UAV ID

# 2 Proposal

It is proposed to approve the below content. All changes are marked with revision marks.

* * * * Begin of Changes * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2]  and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or TS 23.501 [2].

3GPP UAV ID: Identifier assigned by the 3GPP system and used by external AF (e.g. USS) to identify the UAV. GPSI is used as the 3GPP UAV ID.

NOTE 1:	As SMF+PGW-C is the source of information to USS we can use GPSI as 3GPP UAV ID for EPC as well.

Broadcast Remote ID: The capability of providing Remote Identification and Tracking over broadcast radio links.

NOTE 2:	In the scope of this release, the radio link for Broadcast Remote ID is assumed to utilize radio technologies outside the scope of 3GPP.

CAA (Civil Aviation Administration)-Level UAV Identity: a UAV identity assigned by USS/UTM, and uniquely identifies a UAV at least within the scope of a USS.

Command and Control (C2) Communication: the user plane link to deliver messages with information of command and control for UAV operation from a UAV controller or a UTM to a UAV or to report telemetry data from a UAV to its UAV controller or a UTM.

Networked UAV Controller: a UAV Controller connected to the 3GPP network and connected to the UAV via a 3GPP network.

Non-Networked UAV Controller: a UAV Controller not connected to the 3GPP network and connected to UAV via a transport outside the scope of 3GPP, e.g. internet connectivity or direct wireless communication over a technology outside the scope of 3GPP.

Networked Remote ID: The capability of providing Remote Identification and Tracking to a USS over 3GPP network.

Remote Identification (Remote ID) of UAS: The ability of a UAS in flight to provide identification and tracking information that can be received by other parties, to facilitate advanced operations for the UAS (such as Beyond Visual Line of Sight operations as well as operations over people), assist regulatory agencies, air traffic management agencies, law enforcement, and security agencies when a UAS appears to be flying in an unsafe manner or where the UAS is not allowed to fly. The Remote ID information payload may include Serial Number or Session ID assigned to the UAV, location of the ground-station controller, emergency status indication etc.

Third Party Authorized Entity: is either a privileged Networked UAV Controller, or a privileged Non-Networked UAV Controller, or another entity which gets information on sets of UAV controllers and UAVs from the 3GPP network, and may be connected to the UAV via the Internet; it may be authorized by the UTM to interface with sets of UAV(s).

UAS NF: a 3GPP UAS Network Function for support of aerial functionality related to UAV identification, authentication/authorization and tracking, and to support Remote Identification.

UAS Service Supplier (USS): An entity that provides services to support the safe and efficient use of airspace by providing services to the operator / pilot of a UAS in meeting UTM operational requirements. A USS can provide any subset of functionality to meet the provider's business objectives (e.g. UTM, Remote Identification). In the scope of this specification, the term USS refers to both USS and USS/UTM.

UAS Traffic Management (UTM): a system that can safely and efficiently integrate the flying UAV along with other airspace users. It provides a set of functions and services for managing a range of autonomous vehicle operations (e.g. authenticating UAV, authorizing UAS services, managing UAS policies, and controlling UAV traffics in the airspace).

UAV controller: The UAV controller of a UAS enables a drone pilot to control an UAV.

UAV operator: the entity owning and operating a UAV.

UAS Services: refers to establishment of connectivity for a UAS for communication with USS, for C2, for remote identification, and for UAV location and tracking.

UUAA Aviation Payload: Contains application layer information exchanged between the UAS and the USS and that is transparent to the 3GPP System

Uncrewed Aerial System (UAS): Composed of Uncrewed Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the control station, the UAV and the network, and for remote identification. An UAS may comprise of a UAV and a UAV controller.

UUAA: UAV USS authentication and authorization procedure of the UAV to ensure that the UAV has successfully registered with a USS and has therefore been authorized for operations by the USS. An UAV is authenticated and authorized by USS via a UUAA procedure with the support of the 3GPP system before connectivity for UAS services (e.g. UAS-USS connectivity) is enabled.

UUAA-MM: the UUAA procedure optionally performed during registration to a 5GS.

UUAA-SM: the UUAA procedure optionally performed (when UUAA-MM is not performed) during the establishment of a PDU session and performed during the establishment of a PDN connection.

* * * * Next Change * * * *

### 4.2.1 General

This specification covers UAV functionality provided by 5GC connected to NG-RAN and EPC connected to LTE.

The following functionality is defined for UAV support in the 3GPP system:

-	An UAV is authenticated and authorized by USS via a USS UAV Authentication & Authorization (UUAA) with the support of the 3GPP system before connectivity for UAS services (e.g. UAS-USS connectivity) is enabled.

-	Depending on 3GPP network operator and/or regulatory requirements, the UUAA is performed:

-	in 5GS: either as a separate procedure during the 5GS registration procedure (optional and based on specific PLMN policies, USS requirements, and geographic regulatory requirements), or when the UAV requests user plane resources for UAV operation (i.e. PDU session establishment). The UAV shall support UUAA during Registration and PDU session establishment procedure. The network shall support UUAA during PDU session establishment.

-	in EPS: during the attach procedure and the corresponding PDN connection establishment. The network shall support UUAA during PDN connection establishment. The UAV shall support UUAA during PDN connection establishment procedure.

-	A UAV that is in possession of a CAA-Level UAV ID provides the CAA-Level UAV ID in 5GS in both Registration and in PDU Session establishment. In EPC, a UAV that is in possession of a CAA-Level UAV ID provides the CAA-Level UAV ID in PDN Connection establishment in SM-PCO. The 5GC determines whether UUAA is executed at 5GS registration or at PDU session establishment, based on local policies.

-	If UUAA is not performed during the Registration procedure in 5GS, the UUAA is performed at PDU session establishment when the UAV requests user plane resources for UAV operation and the UAV provides its CAA Level ID during PDU session establishment.

-	The UAV flight authorization and UAV-UAVC pairing authorization is performed at PDU session/PDN connection establishment/modification procedures.

-	The 3GPP system supports USS authorization of pairing between a UAV and a networked UAVC or a UAVC that connects to the UAV via Internet connectivity during the establishment of the PDN connection/PDU session for UAS services (i.e. connectivity to USS and for C2 traffic). Modifications or establishment of the pairing or re-authorization take place via modification of the established PDN connection/PDU session. During such procedures, the USS provides to the 3GPP system information (e.g. QoS requirement, data flow descriptors, etc.) that enable traffic between the UAV and the UAVC.

NOTE:	How the USS is made aware of the UAVC is outside the scope of 3GPP in this Release.

-	For EPC, the PDN connections used by UAV are served by SMF+PGW-C regardless of whether the UAV support 5G NAS or whether their subscription allows access to 5GC. The APN(s) used by the UAV for contacting USS or for C2 communication always resolves to a SMF+PWG-C.

The following architectural assumptions apply:

-	It is assumed that the UAV trying to access UAS services using 3GPP connectivity is already registered with a USS and has been assigned a CAA-Level-UAV ID. The procedure for UAV registration and assignment of CAA-Level-UAV ID is out of scope of 3GPP. The USS assigns to the UAV a CAA-Level UAV ID, or is made aware of the assigned CAA-Level UAV ID.

-	A UAV is associated with an Aerial subscription in the UDM. The Aerial subscription contains aerial UE indication (to be used similarly to aerial UE indication defined in EPS) and SM data that indicate that authentication/authorization has to be done using API based mechanism, and optionally some information to determine the dedicated NF (e.g. UAS NF) to contact for authentication and authorization.

-	An UAV is identified by USS using a CAA-Level UAV ID, and identified by the 3GPP System using a 3GPP UAV ID assigned by the MNO.

-	it is assumed that an aerial subscription associated to a UAV includes at least one GPSI to be used as 3GPP UAV ID

-	A UAV is registered with the USS either before connecting with the 3GPP system or using plain internet connectivity via the 3GPP system. Before registering for UAS services with the 3GPP system, the UAV shall be provisioned with a CAA-Level UAV Identity.

-	In roaming scenarios, it is assumed that access to USS is in the VPLMN, thus packet data connectivity for UAV-USS communication is in local breakout, and the UAS NF function is located in the VPLMN.

-	In this release the UAV uses 3GPP access (i.e. LTE & NR) for 3GPP UAV related operations.

-	Activation of RAN aerial features for UAV accessing via E-UTRA reuses the existing mechanism defined in TS 36.300 [9].

-	For this release, it is assumed a UAV is served by the same USS for the duration of a flight.

-	One or more USS(s) may be present in a specific region and may manage UAVs over one or more 3GPP networks.

-	The 3GPP Network subscription for the UAV is not assumed to contain any information about the USS.

-	The USS address, if known to the UAV, is configured in the UAV via mechanisms outside the scope of 3GPP.

* * * * Next Change * * * *

### 4.3.5	SMF

In addition to the functionality defined in TS 23.501 [2], the SMF:

-	triggers the UUAA-SM procedure for a UE requiring UAV authentication and authorization by a USS when requesting user plane resources for UAV operation, or when the USS/UTM that authenticated the UAV triggers a re-authentication

-	may trigger the authorization of pairing between a UAV and a networked UAVC or a UAVC that connects to the UAV via Internet connectivity during the establishment or modification of the PDN connection/PDU session for UAS services.

* * * * Next Change * * * *

### 4.4.3	CAA-Level UAV ID Assignment

The format of the CAA-Level UAV ID is defined outside 3GPP, however how such identity is used to enable a TPAE to query about UAV information is defined with respect to the 3GPP functionality.

Editor's Note:	The following sentence needs clarification "however how such identity is used to enable a TPAE to query about UAV information is defined with respect to the 3GPP functionality ". Why should 3GPP define how CAA-Level UAV ID is used to query the 3GPP network?

In this release, the assignment of a CAA-Level UAV ID for Remote Identification functionality applies solely to the UAV. No CAA-Level UAV ID is assigned to and used by a UAVC.

Various formats of CAA-Level UAV ID must be supported by the UAV to support various geo-specific regulations. At least Serial Number Identification, a CAA-Issued Registration Identifier (aka Session ID), and USS Issued UUID shall be supported.

In the case of Session ID, though the actual format of the CAA-Level UAV ID is defined outside 3GPP and is not decided by 3GPP, it is assumed that the CAA-Level UAV ID may be used by the UAS NF to determine the USS/UTM to contact.

NOTE:	The TPAE could also use the CAA-Level UAV ID to determine the USS/UTM to contect but this is out of scope of 3GPP

Two types of CAA-Level UAV ID assignment are supported:

1.	USS-assigned CAA-Level UAV ID: the identity is assigned completely at USS level.

2.	3GPP-assisted CAA-Level UAV ID assignment.

Editor's Note:	It is FFS, how 3GPP-assisted CAA-Level UAV ID assignment is done. Aviation community, in particular the UAS registration authorities and USS/UTM providers shall be consulted for defining such procedure.

NOTE 2:	It is assumed that the UAV is not aware of which assignment mechanisms is used for the CAA-Level UAV ID.

* * * * End of Changes * * * *