3GPP TSG-WG SA2 Meeting #144E 	S2-2102374

# 12th April – 16th April 2021, Elbonia	(revision of S2-210xxxx)

Source:	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Title:	TS 23.256: Updates to UUAA-MM procedure

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract of the contribution:

# 1 Introduction

For 5GS the UUAA can be performed by AMF or SMF based on local regulatory policies and network configuration. However, the procedure followed for such authentication/authorization shall be same. Also, it has been agreed in the TR conclusions that

"The UAS-NF makes use of existing NEF/SCEF exposure services for UAV authentication/authorization, for UAV flight authorization, for UAV-UAVC pairing authorization, and related revocation "

Keeping this in mind, it would be desirable to align the two procedures in UUAA-MM and UUAA-SM.

Proposal 1:	It is proposed that AMF invokes Nnef service operation to initiate the authentication towards USS. The call flow in UUAA-SM case (generic procedure defined in 23.502) is aligned with the call flow for UUAA-MM procedure in clause 5.2.2.2 of TS 23.256 (see also CRxxxx Tdoc number S2-211abc2)

Proposal 2: If multiple round-trip is needed by the authentication mechanism, it is useful if the USS can provide a callback URI to UAS NF in authentication response (step 4a). This can be used by the UAS NF to generate the subsequent authentication request towards the USS (step 4f).

An Editor's Note is also added to discuss the below:

-	If the registration request comes to a new AMF, what is the impact on UUAA procedure

The pCR also provides a clarification on how the re-authentication is performed (both AMF initiated and USS initiated).

There are also some editorial corrections proposed in this paper e.g. using a uniform naming for "CAA-Level UAD ID"

# 2 Proposal

It is proposed to approve the below content. All changes are marked with revision marks.

* * * * Begin of Changes * * * *

#### 5.2.2.2 UUAA-MM Procedure

Figure 5.2.2.2-1: UUAA-MM procedure

1.	For a UE that requires UUAA or when triggered by re-authentication by USS, the AMF triggers a UUAA-MM procedure.

2.	AMF to UAS NF: The AMF invokes Nnef_Auth_Req service operation that shall include the GPSI and the CAA-Level UAV ID and may include USS address (e.g., FQDN). UAS NF resolves the USS address based on CAA-Level UAV ID or uses the provided USS address, as described in section 4.4.2.

NOTE 2:	Security details will be determined by SA WG3.

Editor’s Note: whether and how the USS address is protected by the UAV is FFS.

3.	UAS NF to USS: Authentication Request, shall include the GPSI and CAA-Level UAV ID.

4.	[Conditional] Multiple rount-trip messages as required by the authentication method used by USS. Authentication Response messages from USS shall include GPSI and may include a authentication message based on authentication method used that is forwarded transparently to UE over NAS MM transport messages. The Authentication Response message from USS in step 4a may also contain a callback URI to be used by the UAS NF for subsequent authentication request in step 4f.

5.	USS to UAS NF: (final) Authentication Response, shall include: GPSI, a UUAA result (success/failure), may include an authorized CAA-Level UAV ID and a payload delivering configuration information to the UAV (e.g. , security info to be used to secure communications with USS), and a final authentication message (e.g., indicating success or failure) based on authentication method used that is forwarded transparently to UE over NAS MM transport messages.

6.	UAS NF to AMF: (final) Authentication Response, forwards information received from USS in step 5.

7.	AMF to UE: (final) NAS MM transport message forwarding authentication message from USS including authentication/authorization result (success/failure).

8.	[Conditional] if UUAA-MM succeeded, AMF triggers a UE Configuration Update procedure to deliver to the UAV authorization information from USS, as described in clause 5.2.2.1. If UUAA-MM fails during a Re-authentication and Re-authorization and there are PDU session(s) established using UAS services, the PDU Sessions should be released with the appropriate cause value.

Editor’s Note: Whether AMF triggers the PDU sessions release or other mechanism is used is FFS.

9.	[Conditional] if UUAA-MM fails, based on network policy the AMF may trigger Network-initiated Deregistration procedure described (as specified in TS 23.502 [3], clause 4.2.2.3.3) and it shall include in the explicit De-Registration Request the appropriate rejection cause value.

At any time after the initial registration, the USS (via UAS NF) or the AMF may initiate Re-authentication procedure for the UAV. For AMF initiated case the Re-authentication procedure shall start from step 2. USS initiated re-authentication procedure is described in clause 5.2.4.

Editor's Note:	It is FFS how the UAS NF is informed of a new serving AMF when the UUAA context is transferred to a new AMF.

* * * * End of Changes * * * *