3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102390

Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	TS 23.247: Update Functional Entities

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution updates the main procedure for MBS Session Join and Establishment for MBS.

1. Introduction/Discussion

In SA2 #143E meeting, the functional entities part was introduced in section 5.3.2. Given that the association content in TR 23.757 was updated, therefore it was proposed that the counterpart of 23.247 needs to be updated accordingly.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change * * * *

### 5.3.2	Functional entities

Editor's Note: functional entities and functional requirement are not completed.

#### 5.3.2.1	PCF

The PCF is used to provide policy rules for MB services, and receive MB service information from AF, directly or indirectly. The PCF performs the following functions to support 5MBS if dynamic PCC for 5MBS is needed:

-	Supporting QoS handling for MBS Session.

-	Providing policy information regarding the MBS Session to MB-SMF for authorizing the related QoS profiles.

-	Interacting with UDR for QoS information retrieval.

#### 5.3.2.2	MB-SMF

The MB-SMF performs the following functions to support 5MBS:

-	General for multicast and broadcast sessions:

-	Supporting MBS session management (including QoS control).

-	Configuring the MB-UPF for multicast and broadcast flows transport based on the policy rules for multicast and broadcast services from PCF or local policy.

-	Specific for broadcast sessions:

-	Interacting with RAN (via AMF) to control data transport using 5GC Shared MBS traffic delivery method.

-	Specific for multicast sessions:

-	Interacting with SMF to modify PDU Session associated with 5MBS.

-	Interacting with RAN (via AMF and SMF) to establish data transmission resources between MB-UPF and RAN nodes for 5GC Shared MBS traffic delivery method.

-	Controlling multicast data transport using 5GC Individual MBS traffic delivery method.

#### 5.3.2.3	SMF

The SMF performs the following functions to support 5MBS:

-	Discovering MB-SMF for multicast session.

-	Authorizing multicast session join operation if needed.

-	Interacting with MB-SMF to obtain multicast context and manage multicast session context.

-	Interacting with RAN for shared data transmission resource establishment.

NOTE:	SMF and MB-SMF may be co-located or deployed separately.

#### 5.3.2.4	MB-UPF

The MB-UPF performs the following functions to support 5MBS:

-	General for multicast and broadcast sessions:

-	Packet filtering of incoming downlink packets for multicast and broadcast flows.

-	QoS enforcement (MFBR) and counting/reporting based on existing means.

-	Interaction with MB-SMF for receiving multicast and broadcast data.

-	Delivery of multicast and broadcast data to RAN nodes for 5GC Shared MBS traffic delivery method.

-	Specific for multicast sessions:

-	Delivery of multicast data to UPF for 5GC Individual MBS traffic delivery method.

#### 5.3.2.5	UPF

The UPF performs the following functions to support 5MBS:

-	Interacting with SMF for receiving multicast data from MB-UPF for individual delivery.

-	Delivering multicast data to UEs via PDU Session for 5GC Individual MBS traffic delivery method.

NOTE:	UPF and MB-UPF may be co-located or deployed separately.

#### 5.3.2.6	AMF

The AMF performs the following functions to support 5MBS:

-	Signalling with NG-RAN and MB-SMF for MBS Session management.

-	Selection of NG-RANs for broadcast.

#### 5.3.2.7	NG-RAN

The NG-RAN performs the following functions to support 5MBS:

-	Management of MBS QoS flows via N2.

-	Delivery of MBS data packets from 5GC shared for multiple UEs over radio using PTM or PTP.

-	Configuration of UE for MBS QoS flow reception at AS layer.

-	Control switching between PTM and PTP delivery per UE.

-	Support for multicast sessions continuity during Xn Handover and N2 Handover.

-	Support notification of session start over radio.

#### 5.3.2.8	UE

The UE may perform the following functions to support 5MBS:

-	Reception of multicast data using PTM/PTP.

-	Reception of multicast and broadcast data using PTM.

-	Handling of incoming MBS QoS flows.

-	Support of signalling for joining and leaving multicast session.

-	5MBS resource management support at AS layer.

-	Reception of notification from 5GC for MBS data transmission.

#### 5.3.2.9	AF

The AF performs the following functions to support 5MBS:

-	Requesting multicast or broadcast service from the 5GC by providing service information to 5GC.

-	Instructing MBS session operation towards 5GC if needed.

#### 5.3.2.10	NEF

The NEF performs the following functions to support 5MBS:

-	Providing an interface to AFs for 5MBS procedures including service provisioning, MBS session and QoS management.

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Selection of serving MB-SMF for an MBS Session.

#### 5.3.2.11	MBSF

The MBSF performs the following functions to support 5MBS:

-	Service level functionality to support 5MBS, and interworking with LTE MBMS

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Selection of serving MB-SMF for an MBS Session.

-	Controlling MBSTF if the MBSTF is used.

-	Determination of sender IP multicast address for the MBS session if IP multicast address is sourced by MBSTF.

NOTE:	MBSF functionality related to service and MBS data handling (e.g. encoding) is to be determined with SA WG4.

#### 5.3.2.12	MBSTF

The MBSTF performs the following functions to support 5MBS if deployed:

-	Media anchor for MBS data traffic if needed.

-	Sourcing of IP Multicast if needed.

-	Generic packet transport functionalities available to any IP multicast enabled application such as framing, multiple flows, packet FEC (encoding).

-	Multicast/broadcast delivery of input files as objects or object flows.

NOTE: MBSTF functionality related to MBS data handling (e.g. encoding) is to be determined with SA4.

#### 5.3.2.13	UDM

The UDM performs the following functions to support 5MBS:

-	Support management of subscription for authorization for multicast sessions.

# * * * * End of changes * * * *