3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102391

Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	TS 23.247: Update MB service provisioning

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution updates the Multicast/broadcast service provisioning part for MBS.

1. Introduction/Discussion

In SA2 #143E meeting, the functional entities part was introduced in section 4.2. This document further updates the description of that clause accordingly.

The proposed updates include:

-	Figure for multicast/broadcast: the original wording is “Transfer of data”, which is similar as the one mentioned in TS 23.246. It is considered to use the other wording “Data volume received by the UEs”, which seems to be more appropriate in 5G design given that “Transfer of data” may imply the RAN node’s totally transferred data.

-	Session Release: Session release should be the counter-procedure of session establishment. Session establishment is triggered by UE join, therefore the session release will be triggered by the UE leave.

Therefore it is proposed to update the related content in clause 4.2.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change * * * *

## 4.2	MB service provisioning

### 4.2.1	Multicast data provisioning

Multicast data provisioning is enabled by certain procedures that are illustrated in the Figure below.

Figure 4.2.1-1: Phases of Multicast data provisioning

The following phases are performed for a specific UE:

-	UE Session Join: UE Session Join is the process by which a UE joins an MBS Session, i.e. the UE indicates to 5GC that such UE wants to receive Multicast data identified by a specific MBS Session ID.

-	UE Session Leave: UE Session Leave is the process by which a UE leaves a MBS Session, i.e. the UE no longer wants to receive Multicast data identified by a specific MBS Session ID.

The following phases are performed for a specific service:

-	Service announcement: Service announcement is used to distribute to information about the service, parameters required for service activation (e.g., IP multicast address(es)) and possibly other service related parameters (e.g. service start time).

-	Session Establishment: Session Establishment is the point at which the transmission resources need to be established for transmitting the DL Multicast data between 5GC and NG-RAN. Session Establishment could be triggered by the UE Session Join request from UE if the Session Join request is accepted.

-	Data transfer: It is the phase when Multicast data are transferred to the UEs.

-	Session Release: It is the point at which there will be no more need to transmit Multicast data between 5GC and a certain NG-RAN node. At Session Release, the resources at NG-RAN node is released. Session Release could be triggered by no UE receiving multicast data for a certain NG-RAN node.

The phase of Multicast data provisioning is illustrated with the following example of timeline:

Time

UE 1 events

UE 2 events

Data volume received by the UEs

Multicast Session

Session Join

Session Leave

Session Join

Session Leave

Service Announcement

Session Establishment

Session Release

Data Transfer

Data Transfer

Data Transfer

Session existing period

Data sent to UE1

Data sent to UE1 and UE2

Data sent to UE1 and UE2

Data sent to UE1

Figure 4.2.1-2: Multicast service timeline example

Editor's note:	Details of the timeline is FFS.

### 4.2.2	Broadcast data provisioning

An example for the phases of broadcast data provisioning is described in the figure below:

Figure 4.2.2-1: Phases of Broadcast data provisioning

The following phases are performed for a specific service:

-	Service announcement: Service announcement is used to distribute to information about the service, parameters required for service activation (e.g., IP multicast address(es)) and possibly other service related parameters (e.g. service start time).

-	Session Establishment: Session Establishment is the point at which the transmission resources need to be established for transmitting the DL Broadcast data between 5GC and NG-RAN. Session Establishment is triggered by the request from AF.

-	Data transfer: It is the phase when Multicast data are transferred in the air interface.

-	Session Release: It is the point at which there will be no more need to transmit Broadcast data. At Session Release, the resources in 5GS are released.

The phase of Broadcast data provisioning is illustrated with the following example of timeline:

Time

UE 1 events

UE 2 events

Data volume received by the UEs

Multicast Session

UE1 local service activation

UE1 local service de-activation

UE2 local service activation

UE2 local service de-activation

Service Announcement

Session Establishment

Session Release

Data Transfer

Data Transfer

Data Transfer

Session existing period

Broadcast of Data, received by any UE which is present

Broadcast of Data, received by any UE which is present

Broadcast of Data, received by any UE which is present

Figure 4.2.2-2: Broadcast service timeline

Editor's note:	Details of the timeline is FFS.

# * * * * End of changes * * * *