3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102416

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2755 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | De-registration for onboarding registered UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the TR 23700-07, On-boarding network shall restrict the usage only for on-boarding service, and the on-boarding UE should be de-registrated after finishing the remote provisioning. When the remote provisioning for SNPN credentials is finished the UE or the ON-SNPN shall initiate de-registration from the ON-SNPN. AMF shall initiate the de-registration when the expiration of the on-boarding timer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the TR conclusion, add the description that the on-boarding network shall restrict the usage only for on-boarding service, and the on-boarding UE should be de-registrated after finishing the remote provisioning. AMF shall initiate the de-registration when the expiration of the on-boarding timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing new features as per conclusion of TR 23.700.07 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X.2.7 (NEW), 5.30.2.X.3.3 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

5.30.2.X.2.7	De-registration from the ON-SNPN for onboarding registered UE

When the remote provisioning for SO-SNPN credentials is finished the UE or the AMF shall initiate de-registration from the ON-SNPN so the UE later can select and register to SO-SNPN using the newly provisioned credentials.

In order to to prevent onboarding registered UEs (i.e., UEs registered within ON-SNPN using "SNPN Onboarding" Registration Type) from staying at the ON-SNPN endlessly, an on-boarding timer needs to be used. If an on-boarding timer is configured at AMF based on operator plicies, the AMF should start the on-boarding timer upon the UE has registered to the ON-SNPN successfully regardless of whether UP or CP remote provisioning is used and the AMF should initiate the de-registration procedure for on-boarding registered UE when the on-boarding timer expires regardless of the results of CP or UP remote provisioning. The value of the on-boarding timer can be derived from the AMF local configuration or from the configuration data in PCF if it is deployed.

5.30.2.X.3.3	De-registration from the PLMN for onboarding registered UE

When the remote provisioning for SO-SNPN credentials is finished the UE or the AMF shall initiate de-registration from the PLMN so the UE later can select and register to SO-SNPN using the newly provisioned credentials.

In order to to prevent onboarding registered UEs (i.e., UEs registered within PLMN using subscription dedicated for onboarding service) from staying at the PLMN endlessly an onboarding timer needs to be used. If an on-boarding timer is configured at AMF based on operator plicies the AMF should start the on-boarding timer upon the UE has registered to the PLMN successfully regardless of whether UP or CP remote provisioning is used and the AMF should initiate the de-registration procedure for on-boarding registered UE when the on-boarding timer expires regardless of the results of CP or UP remote provisioning. The value of the on-boarding timer can be derived from the AMF local configuration or from the configuration data in PCF if it is deployed or UE subscription data from UDM.

***** End of changes *****