SA WG2 Meeting #144e	S2-2102433

12 – 16 April 2021, e-Meeting	(revision of S2-200xxxx)

Source:	Qualcomm Incorporated

Title:	Discussion of Mobility Registration Update to support NR Satellite Access

Document for:	Discussion / Approval

Agenda Item:	8.11

Work Item / Release:	5GSAT_ARCH / Rel-17

Abstract of the contribution: Discusses impacts of TAC update in a radio cell for NR Satellite Access and evaluates alternative solutions to support a Mobility Registration Update.

# 1. Introduction

Broadcast of one TAC per PLMN in a radio cell (in SIB1) is referred to as “hard TAC update” or “hard switch” and aligns with support of TACs with TN. Broadcast of more than one TAC per PLMN in a radio cell is referred to as “soft TAC update” or “soft switch”. These were originally identified by RAN in TR 38.821 [1] (see clause 7.3.1.3.1). RAN2 has since agreed to support both of these as shown from an extract of the draft RAN2#113e meeting report [2] copied below.

Draft RAN2#113e Meeting Report [2]

Agreements: - In NTN, the UE determines the TA based on the broadcast information (the use of other information is not excluded). In any case RAN2 will not go in a different direction than other groups - In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation

Figure 1 – Illustration of Hard TAC Update

Figure 1 illustrates hard TAC update by reference to a satellite and radio cell moving from left to right (e.g. west to east) over two TAs, TA1 and TA2. At time T1, more of the radio cell coverage area is over TA1 than TA2. Accordingly, the radio cell indicates TA 1 and not TA 2 (e.g. in SIB1). At a later time T2, the satellite and the radio cell have moved such that more of the radio cell coverage area is now over TA 2 than TA 1. Accordingly, the radio cell can now indicate TA2 and not TA1. The sudden switch from TA1 to TA2  will look like mobility to UEs accessing the radio cell (even for UEs which are stationary) and may trigger Registration Updates from those UEs whose registration areas do not include TA2.

Observation 1	With hard TAC update, a radio cell indicates coverage of just one TA – e.g. a TA with greatest overlap with the radio cell coverage area. When the radio cell switches from one TA indication to another, served UEs may need to perform a Registration Update, even if they are stationary.

Figure 2 – Illustration of Soft TAC Update

Figure 2 illustrates the same scenario as in Figure 1 but with soft TAC update. At time T1 and at time T2, the radio cell indicates both TA1 and TA2, allowing access from UEs whose registration areas include at least one of TA1 and TA and avoiding a Registration update at time T2. At a later time T3 when the radio cell coverage area has moved completely out of TA1, the radio cell cam just indicate TA2 – and may also indicate other TAs (not shown in Figure 2) to the right of TA 2.

Observation 2	With soft TAC update, a radio cell can indicate coverage of more than one TA – e.g. all TAs with some overlap with the radio cell coverage area. A radio cell can cease indicating a particular TA once the radio cell coverage area no longer overlaps with the TA.

# 2. Consequences of TAC Update to Paging and NAS Procedures

For both hard TAC update and soft TAC update, an AMF may continue to page a UE using the TAs in the UE Registration Area (RA). The NG-RAN can determine which radio cells currently indicate a TA (for hard TAC update) or at least one TA (for soft TAC update) in the RA and can send paging messages only in those radio cells. (Note that this does not preclude more precise paging based on a last known CGI for a UE, but may be needed when the more precise paging fails to reach a UE.)  If a UE always performs a Registration update when accessing a radio cell which indicates a TA outside the RA (for hard TAC update) or which indicates TAs that are all outside the RA (for soft TAC update), the 5GC (AMF) can update the UE’s RA to include the new TA(s) and thereby allow access by the UE to the radio cell. Paging of the UE can then continue to function correctly. As an example, a UE, whose RA includes TA1 but not TA2, which accesses the radio cell in Figure 1 at time T2, would need to perform a Registration update to enable the AMF to change the UE’s RA to include TA2. Similarly, the same UE accessing the radio cell in Figure 2 would not need to perform a Registration at time T2 as the radio cell continues to indicate TA1, but would need to perform a Registration at or sometime before time T3 when the radio cell ceases to indicate TA1.

Observation 3	Both hard TAC update and soft TAC update enable reliable Paging if NG-RAN directs paging messages to radio cells indicating a TA (for hard TAC update) or at least one TA (for soft TAC update) in the UE’s RA.

One consequence of soft TAC update is that a UE may move into a new TA that is not part of the RA and not be aware of this. For example, in Figure 2, a UE may be inside TA1 at time T1 and move into TA2 at time T2. The UE would not be aware of this because the radio cell will continue to indicate both TA1 and TA2 at time T1 and time T2. The same may happen for hard TAC update in Figure 1 if a UE whose RA includes TA1 but not TA2 were to move from TA1 into TA2 at time T1 as the radio cell indicates TA1 at this time.

When a UE initiates a NAS procedure with a 5GC (e.g. a NAS Service Request), the  NG-RAN will need to indicate the current TA in which the UE is located to the 5GC (e.g. AMF) as part of NGAP User Location Information (ULI). If the NG-RAN indicates the correct TA according to configured geographic definitions of TAs and the current geographic location of the UE, there may be problems for both hard TAC update and soft TAC update if the correct TA for the UE indicated in the ULI is not part of the RA. This may be seen as an anomaly (a “TA anomaly”) by the 5GC (e.g. AMF) because the UE is meant to perform a Registration update with the 5GC whenever it moves outside of the RA. As an example, if UE charging is based on the RA and the RA is not updated based on the current TA, there may be a charging error.

Observation 4	For both hard TAC update and soft TAC update, a UE can enter a TA that is not part of the RA and remain unaware of this (e.g. not perform a Registration update). This may lead to a “TA anomaly” where the UE initiates a NAS procedure and the ULI delivered to the 5GC by NG-RAN indicates a TA that is not part of the RA.

The TA anomaly can be removed (at least superficially) for hard TAC update by “fudging” the TA in the ULI. Instead of including the correct TA in the ULI in which the UE is geographically located, the NG-RAN can instead indicate an incorrect TA which was indicated to the UE by the radio cell.  This incorrect TA will at least match the TA seen by the UE in the radio cell and will be part of the RA. Thus, the TA anomaly will not occur (or at least will not be observed), although the 5GC (AMF) will receive an indication of an incorrect TA. This incorrect TA indication will have other consequences, since the ULI will also need to include a CGI indicating a fixed cell in which the UE is located, in order to support services like routing of an emergency call to a PSAP, charging and possibly precise paging of a UE in a last known fixed cell. So the AMF will receive a CGI and TAC in the ULI where the CGI is not part of the TA indicated by the TAC, which can be considered as a “TAC-CGI mismatch anomaly”. In some implementations, this anomaly could trigger an error either in real time or later when the TAC and CGI are processed offline. Thus, even for hard TAC update, it may be preferable to deal with the “TA anomaly” rather than the “TAC-CGI mismatch anomaly”.

Observation 5	For hard TAC update but not soft TAC update, the TA anomaly can be removed if the NG-RAN indicates the same TA to the 5GC as is indicated to the UE in the radio cell. However, this may lead to a different “TAC-CGI mismatch anomaly” which may be worse than the TA anomaly. Therefore, both hard and soft TAC update may benefit from a more robust resolution for the TA anomaly.

# 3. Solutions to the TA Anomaly

Alternative solutions (referred to as options) are summarized below. In each case, it is assumed that a UE initiates a NAS procedure (e.g. a Service Request or PDU Establishment Request) and the AMF receives a ULI from the NG-RAN indicating a TA for the UE which is not part of the current RA for the UE.

Option A	Ignore the Anomaly.

The 5GC behaves as if the UE is still within the RA. At some point, the UE would be likely to access a radio cell that does not indicate any TA in the RA (as a simple consequence of the UE not being located in the RA) and the UE would then perform a Registration update allowing the 5GC to update the RA. Also, the UE will still perform periodic Registration update to allow similar update of the RA.

Option B			Immediate Registration

If the new TA indicated in the ULI is allowed, the AMF requests the UE to perform a Registration by (temporarily) rejecting the NAS procedure request with a new cause value indicating Registration. Once the Registration is complete, the UE can re-initiate the original NAS procedure if this is still needed.

Option C			Deferred Registration

The AMF performs the requested NAS procedure if the new TA indicated in the ULI is allowed for the UE. The AMF also sends a UE Configuration Update Command to the UE to request the UE to perform a Registration.  The UE Configuration Update Command can be sent as soon as the AMF discovers the UE is in a TA that is not part of the current RA or after the requested NAS procedure is complete.

Option D	Implicit Registration

The AMF treats the NAS procedure request as an implicit Registration update for the new TA indicated in the ULI. The AMF can also perform other functions associated with a Registration update (e.g. update charging information). The AMF then returns the results of the implicit Registration update (e.g. provides new TAIs for a new RA) to the UE using a UE Configuration Update Command.

Option E	Only Support Hard TAC Update

This is a solution in principle and removes the TA anomaly. But another anomaly remains as described above.

Option F	Configure Geographic Information for allowed TAs in a UE

The AMF includes a geographic definition of each TA that is part of an RA in a Registration Accept. The geographic definitions can be configured by O&M in AMFs and can be treated as non-interpreted blocks of data with low impact to an AMF. UEs then use GNSS (or other) location to determine their current TA and perform  a Registration update when no longer in any TA that is part of the current RA.

Options A-E (but not Option F) do not appear able to support forbidden (non-allowed) TAs. This is because a UE does not know in which TA it is actually located. The UE only knows which TA or TAs are supported (i.e.  broadcast in SIB1) by the radio cell currently being accessed. However, as shown in Figures 1 and 2, this is not enough to unambiguously indicate the actual TA of the UE. Thus, although an AMF could reject a UE Registration or other NAS procedure request from a  UE when the UE is located in a forbidden TA, the UE will not know for sure when it has exited the forbidden TA into a TA which is allowed for the UE. This could lead to a trial and error approach where the UE periodically reattempts a Registration or other NAS procedure to verify whether it is now in an allowed TA. Alternatively or in addition, a UE may deny service to a user for some period after receiving a NAS reject from an AMF even after the UE enters an allowed TA, as the UE will not when the allowed TA had been entered. It thus seems that support of forbidden TAs could not be supported for NTN in Release 17 for any of Options A-E. However, this may not be a severe disadvantage, since NTN with satellites does not rely on nearby terrestrial base stations which are often the main source of restrictions for a forbidden TA.

Observation 6	Options A-E (but not Option F) appear not able to support forbidden TAs

Looking now at each solution in turn, Option A has no obvious new impacts to a UE or AMF. However, ignoring that a UE is now in a TA which the UE is not aware of and which is outside the UE RA may impact and degrade subsequent paging of the UE. For example, the AMF knows that the UE is now in some fixed TA and fixed cell but cannot necessarily page the UE based on these because the UE will avoid accessing a radio cell which indicates support for this TA but not for any TA in the UE’s RA. Additionally, a 5GC may be configured to perform certain services and functions when a UE exits an RA, as part of a new registration, which must now be deferred for some indefinite time. It is unclear whether such deferral would necessarily be harmful and possibly this could depend on the network implementation, but it seems a risk.

Option B has a small NAS signaling impact to define a new cause value for the NAS reject message sent for the originally requested NAS procedure. There would also be small AMF and UE impacts to support the return of such a new cause value and for the UE to instigate the Registration and later re-initiate the originally requested NAS procedure. The temporary rejection of the originally requested NAS procedure and its re-initiation will add some small delay which may be significant in some cases - e.g., if an emergency services call is involved.

Option C avoids new NAS signaling impacts and impacts to the UE but has a small impact to the AMF to send the UE Configuration Update Command message. In addition, performing the original NAS procedure in the AMF before the Registration update has occurred seems to carry a small risk of some error since the UE is located in a TA which is outside the RA.

Option D has no new NAS signaling impact and no new UE impact. However, performing an implicit Registration update in an AMF in the absence of a Registration Request from the UE may be a medium to high impact as there are a large number of potential functions that can be performed by an AMF associated with a Registration update. In addition, it is unclear if an implicit Registration update can always be fully performed using only the information provided in the original NAS procedure request.

Option E does not seem a solution at all since, as indicated in section 2, removal of the TA anomaly for hard TAC update only introduces another anomaly which may be even more difficult to resolve.

Option F has significant extra UE impact and at least some extra AMF impact. It will also impact O&M (to configure standardized TAs in an AMF) and may restrict the flexibility of TA configuration since the geographic definition of a TA will have to conform to a standard.

The various characteristics above are summarized below in Table 1, where a characteristic is indicated as good (green), poor (red) or in between (yellow).

| Characteristic | Option A | Option B | Option C | Option D | Option E | Option F |
| --- | --- | --- | --- | --- | --- | --- |
| Support Forbidden TAs | No | No | No | No | No | Yes |
| NAS Signaling Impact | No | Low | No | No | No | Medium |
| AMF Impact | No | Low | Low | Medium to High | No | Low |
| UE Impact | No | Low | No | No | No | High |
| O&M Impact | No | No | No | No | No | High |
| Other problems | Maybe – e.g. paging and possibly others | Small extra NAS delay | Possible though none identified | Unknown | Yes – introduces another anomaly | None found |

Table 1 – Comparison of Options A-F to resolve the TA Anomaly

Table 1 indicates that Options A and C have fewest major disadvantages and thus seem preferred. The suitability of Option A depends on whether additional problems are found (e.g. associated with Paging as described above) which may depend on each AMF implementation.  Therefore, a more effective solution would be to combine Options A and C by making use of Option C optional (according to implementation). When Option C is not used, Option A would be used by default.

Observation 7	Six alternative solutions were compared for resolving the TA anomaly described in section 2. Of these, Options A and C appear more advantageous and may be combined such that an AMF can use either one or the other.

# 4. Proposal

The following observations have been made.

Observation 1	With hard TAC update, a radio cell indicates coverage of just one TA – e.g. a TA with greatest overlap with the radio cell coverage area. When the radio cell switches from one TA indication to another, served UEs may need to perform a Registration Update, even if they are stationary.

Observation 2	With soft TAC update, a radio cell can indicate coverage of more than one TA – e.g. all TAs with some overlap with the radio cell coverage area. A radio cell can cease indicating a particular TA once the radio cell coverage area no longer overlaps with the TA.

Observation 3	Both hard TAC update and soft TAC update enable reliable Paging if NG-RAN directs paging messages to radio cells indicating a TA (for hard TAC update) or at least one TA (for soft TAC update) in the UE’s RA.

Observation 4	For both hard TAC update and soft TAC update, a UE can enter a TA that is not part of the RA and remain unaware of this (e.g. not perform a Registration update). This may lead to a “TA anomaly” where the UE initiates a NAS procedure and the ULI delivered to the 5GC by NG-RAN indicates a TA that is not part of the RA.

Observation 5	For hard TAC update but not soft TAC update, the TA anomaly can be removed if the NG-RAN indicates the same TA to the 5GC as is indicated to the UE in the radio cell. However, this may lead to a different “TAC-CGI mismatch anomaly” which may be worse than the TA anomaly. Therefore, both hard and soft TAC update may benefit from a more robust resolution for the TA anomaly.

Observation 6	Options A-E (but not Option F) appear not able to support forbidden TAs

Observation 7	Six alternative solutions were compared for resolving the TA anomaly described in section 2. Of these, Options A and C appear more advantageous and may be combined such that an AMF can use either one or the other.

Based on these, the following proposals are made.

Proposal 1	Agree to Options A and C as alternatives for an AMF.

Proposal 2	Agree a CR to TS 23.501 [3] to describe the anomaly and the solution.

Proposal 3	Send an LS to CT1 to advise these agreements.

# 5. References

[1]	3GPP TR 38.821: “Solutions for NR to support non-terrestrial networks (NTN)”.

[2]	Draft RAN2#113e Meeting Report: “Report of 3GPP TSG RAN WG2 meeting #113-e”,  ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_113-e/Report/Draft_R2-113-e_Meeting_Report_v1.zip.

[3]	3GPP TS 23.501: “System architecture for the 5G System (5GS); Stage 2”.