3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102464

Elbonia, Apr 12 – Apr 16, 2021	(revision of S2-210XXXXX)

Source:	Huawei, HiSilicon

Title:	Discussion on SMF or NWDAF mapping PDU sessions to AoI

Document for:	Discussion

Agenda Item:	8.1

Work Item / Release:	eNA_Ph2 / Rel-17

Abstract: This contribution proposes discussion on alternatives for mapping PDU sessions in an AoI considering the signaling load reduction.

1. Introduction

Observation 1: The conclusions from KI#11 captured in TR 23.700-91 Clause 8.11.2 related to mechanisms in SMF to reduce the signalling for data collection of PDU sessions in an AoI have been accepted in TS 23.288.

Extracted from TS 23.288 :

6.2.2.1	General

[…]

To retrieve data from SMFs for analytics IDs subscription or requests for "any UE" including analytics filter information with specific Applications, DNNs, DNAIs and area of interest per TA granularity, NWDAF shall first discover the SMF serving the area of interest via NRF.

NWDAF may directly consume events from the discovered SMF using the event target set to "any PDU session", and event filters with the same parameters of the analytics filter information, i.e. list of Application IDs, and/or DNNs, and/or DNAI, and the area of interest related to the requested analytics ID.

1.	SMF may support the exchange of UE Location parameter when SMF interacts with AMF via Nsmf_PDUSession_Create / Update / CreateSMContext / UpdateSMContext due to session establishment, modification, or release, service request, or handover procedures

If there are any changes in PDU sessions in the area of interest, for the Application ID, and/or DNN, and/or DNAI subscribed by NWDAF, SMF notifies the detected changes to NWDAF via Nmsf_EventExposure_Notify service operation, enabling NWDAF to keep an updated map of SMF and PDU sessions associated with the analytics filter information in an area of interest.

Editor's note:	Whether the need for an SMF mapping the PDU Session association with TA or NWDAF performs such mapping is FFS.

2.	SMF may also subscribe to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "UE moving in or out of Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 from TS 23.502 [3] to retrieve the list of SUPIs (and GPSIs if available) in the area of interest. Based on the retrieved list of SUPIs in the area of interest, SMF identifies the PDU sessions in the area of interest.

Editor's note:	Whether the same discovery mechanism shall be supported by DCCF and whether further alignments are required is FFS.

Observation 2: It has been accepted that SMF may be able to expose information with target of event reporting set to “any PDU session”

Extracted from accepted TS 23.502:

5.2.8.3.1	General

[…]

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI), an Internal Group Identifier, an indication that any UE is targeted (e.g. on a specific DNN), or an indication that any PDU session is the target.

[…]

Observation 3: According with TS 23.501 Clause 5.6.11 (UE presence in Area of Interest reporting usage by SMF), SMF is already capable to identify via interaction with AMF the mapping of UEs or PDU sessions to an AoI.

Text extracted from TS 23.501 Clause 5.6.11:

5.6.11	UE presence in Area of Interest reporting usage by SMF

When a PDU Session is established or modified, or when the user plane path has been changed (e.g. UPF re-allocation/addition/removal), SMF may determine an Area of Interest, e.g. based on UPF Service Area, subscription by PCF for reporting UE presence in Presence Reporting Area, etc.

For 3GPP access, the Area of Interest corresponds:

-	either to Presence Information that may correspond to:

-	a list of Tracking Areas; or

-	a list of Presence Reporting Area ID(s) and optionally the elements comprising TAs and/or NG-RAN nodes and/or cells identifiers corresponding to the PRA ID(s); or

-	a LADN DNN.

For Non-3GPP access, the Area of Interest corresponds to:

-	N3GPP TAI (see clause 5.3.2.3).

For UE location change into or out of an "area of interest", the SMF subscribes to "UE mobility event notification" service provided by AMF for reporting of UE presence in Area of Interest as described in clause 5.3.4.4. The AMF may send the UE location to the SMF along with the notification, e.g. for UPF selection. Upon reception of a notification from AMF, the SMF determines how to deal with the PDU Session, e.g. reallocate UPF.

[…]

The subscription may be maintained during the life of PDU Session, regardless of the UP activation state of PDU Session (i.e. whether UP connection of the PDU Session is activated or not).

SMF may determine a new area of interest, and send a new subscription to the AMF with the new area of interest.

SMF un-subscribes to "UE mobility event notification" service when PDU Session is released.

Problem: There are two issues listed below that need to be solved to properly capture in TS 23.288 the benefits of signalling reduction for data collection of PDU session information mapped to an AoI and solve the FFS in TS 23.288 Clause 6.2.2.1 (highlighted in green):

- First Issue: If SMF does not support UE Location exchange with AMF, who should be responsible for creating this mapping: NWDAF or SMF?

- Second Issue: How can NWDAF know if SMF supports or not the mapping of PDU session per AoI, therefore NWDAF does not need to do the mapping of itself and use the mechanism for reducing data collection signalling from SMF?

2. Discussion

To solve the problem, the following three alternatives are proposed.

Alternative 1: Exposure in SMF NF profile the support to signalling reduction. If available NWDAF uses SMF mechanism. If not, NWDAF takes the responsibility of creating the mapping by itself.

- SMF NF profile is extended with a flag called “UE Location Support” that can be set to true or false.

  - When the value is set to true, this means that SMF is capable to do the mapping of UEs or PDU sessions to an AoI in TA granularity, as well as that NF consumers of SMF events can request event filters containing the AoI in TA granularity.

  - When the value is set to false, this means that SMF is not capable of mapping of UEs or PDU sessions to an AoI in TA granularity, as well as that NF consumers of SMF events should not request event filters containing the AoI in TA granularity.

- When NWDAF discovers the SMFs in a TA, it is also able to determine which SMFs support the mechanism to reduce signalling, by having the UE Location support to exchange such information with AMF.

- If “UE Location Support” is not support by SMF, NWDAF needs to create the mapping by itself performing a 5-step approach:

  - Manages the subscription/unsubscription to discovers and maintain via NRF the mapping of which AMFs are mapped to an AoI.

  - Based on the discovery AMFs, NWDAF manages the subscription/unsubscription (subscription modification) to the event to get the list of SUPIs of UEs moving in-and-out an AoI for the discovered AMFs.

  - Based on the SUPI list retrieved from the AMFs, manages the subscription/unsubscription (subscription modification) to UE Context Information from UDM to obtain the mapping of which SMFs serving such UEs.

  - Manage the subscription/subscription (subscription modification) to the events exposed by SMFs that are serving the identified UEs in the AoI. The subscription to SMF event has as target the list of UEs (that are in the AoI as per info from AMF).

  - Creates the mapping of the UE or PDU sessions in an AoI to enable analytics output generation.

Alternative 2: NWDAF requests the event from SMF assuming the mechanism is available. If not available, NWDAF takes the responsibility of creating the mapping by itself.

- NWDAF sends the subscription to SMF being agnostic whether the SMF has the “UE Location Support”.

- If SMF supports, SMF accepts the subscription, performs the mapping, and provides the events to NWDAF as per requested subscription fields

- If SMF does not have “UE Location Support”, the SMF rejects the subscription from NWDAF and indicates the reason of not supporting event filter per AoI.

- NWDAF be receiving the rejection determines that it needs to create the mapping by itself and executes the same 5-step approached description in Step 3 of Alternative 1.

Option 3. NWDAF requests the event from SMF assuming the mechanism is available. If not available, SMF takes the responsibility

- NWDAF sends the subscription to SMF being agnostic whether the SMF has the “UE Location Support”.

- If SMF supports, SMF accepts the subscription, performs the mapping, and provides the events to NWDAF as per requested subscription fields

- If SMF does not have “UE Location Support”, the SMF accepts the subscription from NWDAF, and executes a 3-step approach for mapping the PDU sessions or UEs to AoI:

  - Manages the subscription/unsubscription to discovers and maintain via NRF the mapping of which AMFs are mapped to an AoI.

  - Based on the discovery AMFs, NWDAF manages the subscription/unsubscription (subscription modification) to the event to get the list of SUPIs of UEs moving in-and-out an AoI for the discovered AMFs.

  - Based on the SUPI list retrieved from the AMFs, SMF creates the mapping of UEs or PDU sessions mapped to an AoI, and provide the required events to NWDAF.

Alternative 4. NWDAF requests the event from SMF assuming the mechanism is available. If not available, SMF takes the responsibility if possible (hybrid of Alternative 2 and 3)

- NWDAF sends the subscription to SMF being agnostic whether the SMF has the “UE Location Support”.

- If SMF supports “UE Location Support”, SMF accepts the subscription, performs the mapping, and provides the events to NWDAF as per requested subscription fields.

- If SMF does not have “UE Location Support” but has the support for creating such mapping, the SMF accepts the subscription from NWDAF, and executes the 3-step approach as described in Alternative 3.

- If SMF does not have “UE Location Support” and does not have the support for creating such mapping, the SMF rejects the subscription from NWDAF and indicates the reason of not supporting event filter per AoI.

- NWDAF be receiving the rejection determines that it needs to create the mapping by itself and executes the same 5-step approached description in Step 3 of Alternative 1.

Comparison of Alternatives 1, 2, and 3 when SMF has not the “UE Location Support”:

-	Alternatives 1 and 2 keep the responsibility of UEs or PDU sessions in AoI mapping at NWDAF.

However, these alternatives require the management of different subscriptions to information in 4 different NFs: NRF, AMF, UDM, SMF.

- 	Alternative 3 keeps the responsibility of UEs or PDU sessions in AoI mapping at SMF.

This approach requires the management of different subscriptions to information in 2 different NFs: NRF, AMF. Cutting in half the amount of signaling to obtain the mapping of UEs or PDU sessions in AoI.

However, this alternative delegates to SMF the task of discovery of entities for enabling NWDAF to collect data per AoI;

- 	Alternative 4 is a hybrid approach between Alternative 2 and 3.

As it has been highlighted in the Observation 3, SMF already in R16 has the capability of creating a mapping (via AMF) of UEs and PDU sessions in an AoI. Therefore, this is not a new capability of SMF.

This alternative enables the compatibility with R16 SMFs (even if SMFs are not using the UE reporting capability) pushing back the responsibility of mapping to NWDAF (and having a high signaling cost) but enables the use of R17 SMFs with support to reduce signaling for data collection.

Proposal: Alternative 4 is selected to enable the signalling efficient possibility for creating the mapping of Ues or PDU sessions in an AoI per TA granularity.

3. Proposal

It is proposed to approve the corresponding TS 23.288 CR S2-2102465 which reflects the implementation of Alternative 4.