3GPP TSG-SA WG2 Meeting #144E	S2-2102526

# Apr 12th – Apr 16th, 2021, Elbonia											(Revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2775 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE location procedure triggering for NR satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For NR satellite access, the AMF determine whether the Selected PLMN is allowed at the present UE location during Registration procedure. However, it is not clear how the AMF can verify the precise UE location during PDU Session Establishment procedure. The UE location can be changed after registration without being notified to the AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The AMF initiates the periodic, triggered, or event based UE location procedure to track the precise UE location after successful registration. - The AMF determines whether to reject the PDU Session Establishment request based on the UE location information provided by the LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how to handle PDU Session Establishment request for NR satellite access and cannot meet the regulatory requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change *****

5.4.10	Support for identification and restriction of using NR satellite access

Support for NR satellite access is specified in TS 38.300 [27].

Editor's note:	TS 38.300 [27] not yet updated by RAN groups.

The AMF determines the RAT Type for NR satellite access, i.e. NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT). When the UE is accessing NR using satellite access, an indication is provided in N2 interface indicating the type of NR satellite access, as defined in TS 38.413 [34].

Editor's note:	Further details on what type of satellite platforms OTHERSAT includes is FFS and to be aligned with RAN WG3.

The serving PLMN can enforce mobility restrictions for NR satellite access as specified in clause 5.3.4.1.

In order to enable efficient enforcement of Mobility Restrictions, cells of each NR satellite RAT Type (NR(LEO), NR(MEO), NR(GEO) or NR(OTHERSAT)) need to be deployed in TAs different from TAs for other NR satellite RAT Types as well as different from TAs supporting terrestrial access RAT Types.

When the UE performs a successful registration procedure, the AMF may initiate the Deferred 5GC-MT-LR procedure for Periodic, Triggered and UE Available Location Events as specified in TS 23.273 [51] clause 6.3.1 to track the UE location with sufficient accuracy (e.g. the country in which the UE is located) to determine whether the Serving PLMN is allowed to operate at the present UE location, as described in clause 4.2.2.2.2 in TS 23.502 [3].

Based on UE location information provided by the LMF, the AMF can determine whether to reject the PDU Session Establishment request and/or whether to execute the Network-initiated Deregistration procedure described in clause 4.3.2.2.1/4.2.2.3.3 in TS 23.502 [3].

*** End of changes ***