SA WG2 Meeting #144E (e-meeting)	S2-2102549

# April 12 – 16, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2780 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to satellite backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Arch |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the spec TS 23.503 and related discussions in the SA2 meetings, the UPF shall not directly report satellite backhaul to the SMF, instead, the UPF can just report the network instance with long latency to the SMF according to stage 3 spec TS 29.244. So here we propose to update the text to let the UPF report network instances associated with the GTP-U peer if it detects the GTP-U path has long transmission delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. clarifying the UPF reports network instances associated with the GTP-U peer. 2. clarifying the SMF determines satellite backhaul categories based on the the reported network instances and the configuration. 3. clarifying the cases of gNBs with hybrid satellite backhauls or satellite backhaul involving inter-satellite links are out of the scope of this release |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong UPF implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

5.8.2.15	Reporting of Network Instances using satellite backhaul to SMF

If the UPF is aware that a network instance with long delay (e.g. satellite backhaul) is used towards 5G AN, the UPF may report the network instance information to SMF as part of the N4 Node level procedures as described in TS 23.502 [3], clause 4.4.3.

NOTE 1:	Based on the report from the UPF, a SMF configured with the mapping between network instances and satellite backhaul categories can determine the satellite backhaul category in use based on the Network Instance associated to the PDU Session.

NOTE 2:	The cases of gNBs with hybrid backhauls (e.g., with hybrid satellite backhauls, or with a terrestrial backhaul and a satellite backhaul) or a satellite backhaul involving inter-satellite link are out of the scope of this release.

* * * End of changes * * *