3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102577

Elbonia, 12 - 16 April, 2021

Source:	Huawei, HiSilicon

Title:	Missed parameters within the Service Announcement

Document for:	Agreement

Agenda Item: 	8.9

Work Item / Release:	5MBS /Rel-17

Abstract of the contribution: This contribution proposes to add missed parameter which AF need inform UE in the service announcement.

- Discussion

For 5G MBS, the UE needs to know the DNN and S-NSSAI of the associated PDU Session. The UE join the 5MBS session in the related PDU Session. Also the related PLMN is expected to be included. It is proposed to send the PLMN ID, DNN and S-NSSAI of the associated PDU Session to UE in service announcement.

- Proposal

It is proposed to add the related information in TS 23.247.

**************************** 1st Change ***************************

7.1	Common procedure for Multicast and Broadcast

7.1.1	Configuration for MBS

Editor's note:	The heading and content needs to be revisited.

The configuration steps for MBS Session is used by the AF to start the MBS Session towards 5GC, and it applies to both multicast and broadcast communications unless otherwise stated.

Figure 7.1.1-1: Configuration for MBS

Editor´s note:	MBSTF could optionally be in the user plane and the call flow should be updated to also cover that possibility.

Editor's note:	The services and messages used in this procedure are FFS.

Steps 1 to 5 are optional and only applicable if TMGI is used as multicast session ID and required to be pre-allocated.

1.	AF sends Allocate TMGI Request () message to NEF/MBSF to request allocation of a TMGI to identify a new group.

NOTE 1:	Depending on the configuration, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2. AF checks authorization of AF.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration, and then sends an Allocate TMGI Request () message to the MB-SMF.

4.	MB-SMF allocates a TMGI and returns the TMGI to the NEF/MBSF.

5.	The NEF or MBSF responds to the AF by sending a Allocate TMGI Response (TMGI) message.

6.	The AF may perform a Service Announcement towards UE. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, source specific multicast address, PLMN ID of the PLMN in which the MBS Session is available, S-NSSAI and DNN of the PDU Session associated with the MBS Session in the PLMN. The Service Announcement may have  other information e.g., MBS service area, session description information, etc.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

7.	AF of content provider may provides contents for a multicast session (possibly providing information for a previously allocated TMGI) to NEF. If step 1-4 has not been executed before, The AF may provide a source specific multicast address or it may request that the network allocates an identifier for the multicast group (TMGI). MBS information may further include QoS requirements, UE authorization information (e.g. a GPSI or an External Group Id or a UE ID to identify UEs authorized to join the multicast service, service area identifying the service scope.

Editor's note:	What other information is to be sent by AF is FFS.

8.	NEF/MBSF checks authorization of content provider. NEF/MBSF-C selects MB-SMF as ingress control node, possibly based on location area.

9.	NEF/MBSF detect MB-SMF.

10.	NEF/MBSF requests MB-SMF to reserve ingress resources for a multicast distribution session and provides Multicast group ID or request allocation. It also indicates if the allocation of an ingress transport address is requested.

11.	[Optional] The MB-SMF sends SM MBS Policy Association Request to MB-PCF with the Multicast group ID, AF Identifier, and the QoS requirements.

12.	[Optional] The MB-PCF registers at the BSF that it handles the multicast session. It provides an identifier that the policy association is for multicast and the multicast group ID, it own PCF ID and optionally its PCF set ID.

13.	[Optional] The MB-PCF responds with SM MBS Policy Association Response with policies for the Multicast group ID.

In addition, determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration for this Application), and notifies the result to the MB-SMF. The PCF notifies the MB-SMF whether the transmission resources corresponding to the QoS request are established or not.

If the request is not authorized, the required QoS is not allowed, or transmission resources are not established, MB-SMF responds to the NEF in step 16 with a Result value indicating the failure cause, and NEF further notifies AF in step 17.

14.	MB-SMF selects the MB-UPF and requests it to reserve user plane ingress resources. If multicast transport of the multicast data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, source specific multicast address and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 7. It also indicates the success or failure of reserving transmission resources.

17.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF.

18.	Same as step 6. The AF may also perform a service announcement at this stage.

********************************* End Change ****************************