3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102582

Elbonia, April 12 – April 16, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Update of ProSe direct discovery procedure with Announcing PLMN ID

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution is to update the ProSe direct discovery with the addition of Announcing PLMN ID.

# 1. Introduction/Discussion

For ProSe enabled announcing UE uses the radio resource not from its serving PLMN, it shall report Announcing PLMN ID to the HPLMN and further to the PLMN providing the radio resource. This is for the purpose of inter-PLMN charging.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[10]	3GPP TS 29.345: "Inter-Proximity-services (Prose) Function signalling aspects; Stage 3".

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[17]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[18]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[19]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[20]	IETF RFC 4960: "Stream Control Transmission Protocol".

[21]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[X]	3GPP TS 32.277: " Proximity-based Services (ProSe) charging".

# * * * * Next change * * * *

## 5.9	Identifiers

### 5.9.1	Identifiers for ProSe Direct Discovery

#### 5.9.1.X	Announcing PLMN ID

Announcing PLMN ID is the PLMN identity of the serving PLMN which signalled the carrier frequency, when the serving PLMN is not the HPLMN nor the VPLMN, defined in TS 32.277 [X]. The announcing UE provides the Announcing PLMN ID to the 5G DDNMF when sending the Discovery Request message.

# * * * * Next change * * * *

#### 6.3.1.1	Overview

ProSe Direct Discovery is defined as the process that detects and identifies another UE in proximity using NR radio signals. There are two types of ProSe Direct Discovery supported over PC3a reference point: open and restricted, as defined in TS 23.303 [3]. ProSe Direct Discovery can be a standalone service or can be used for subsequent actions e.g. to initiate ProSe Direct Communication.

ProSe-enabled UEs which have obtained authorization to participate in ProSe Direct Discovery shall not continue in participating in ProSe Direct Discovery procedures over PC3a reference point defined in clause 6.3.1 when they detect loss of NG-RAN coverage in the serving PLMN.

With ProSe Direct Discovery, the UE can use inter-PLMN discovery transmission based on the indication from the serving NG-RAN or the provisioned radio resource on the UE. How the serving cell authorizes the UE to use inter-PLMN radio resource is specified in TS 38.331 [16]. When Inter-PLMN radio resources is used by the announcing UE, it provides the Announcing PLMN ID to the 5G DDNMF.

# * * * * End of changes * * * *