3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102585

Elbonia, April 12 – 16, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Update of Terms and Abbreviations

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: Addition of Terms and Abbreviations used in the TS taking inspiration from 5GS V2X.

# 1. Introduction/Discussion

This contribution proposes to update the Terms and Abbreviations which are used throughout TS 23.304. The definitions are inspired by the similar terms an abbreviations used by 5GS V2X in TS 23.287.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application Layer ID: An identity identifying a ProSe-enabled UE within the context of a specific application. The format of this identifier is outside the scope of 3GPP.

Direct Network Communication: One mode of network communication, where there is no ProSe UE-to-Network Relay between a UE and the 5G network.

Indirect Network Communication: One mode of network communication, where there is a ProSe UE-to-Network Relay between a UE and the 5G network.

Member ID: An identifier uniquely identifying a member in the Application Layer managed group and that is managed by the ProSe application layer.

Mode of communication: Mode of communication to be used by the UE over PC5 reference point, i.e. broadcast mode, groupcast mode or unicast mode.

Open ProSe Discovery: ProSe Direct Discovery without explicit permission from the ProSe-enabled UE being discovered, according to TS 22.278 [7].

ProSe Direct Discovery: A procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity based on direct radio transmissions between the two UEs with NR technology.

ProSe Direct Communication: A communication between two or more UEs in proximity that are ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node.

ProSe UE-to-Network Relay: A ProSe-enabled UE that provides functionality to support connectivity to the network for Remote UE(s).

ProSe UE-to-UE Relay: A ProSe-enabled UE that provides functionality to support connectivity between U2U UEs.

Remote UE: A ProSe-enabled UE that communicates with a DN via a ProSe UE-to-Network Relay.

Restricted ProSe Discovery: ProSe Direct Discovery that only takes place with explicit permission from the ProSe-enabled UE being discovered, according to TS 22.278 [7].

U2U UE: A ProSe-enabled UE that discovers or is discovered by a ProSe-enabled UE(s) via a ProSe UE-to-UE Relay; or a ProSe-enabled UE that communicates with other ProSe-enabled UE(s) via a ProSe UE-to-UE Relay.

For the purposes of the present document, the following term and definition given in TS 23.303 [3] apply:

ProSe-enabled UE

# * * * * Second change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ProSe	Proximity based Services

RPAUID	Restricted ProSe Application User ID

PDUID	ProSe Discovery UE ID

PFI	PC5 QoS Flow Identifier

PQI	PC5 5QI

# * * * * End of changes * * * *