3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102588

Elbonia, April 12 – 16, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Clarification on parameters for group member discovery

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to clarify the parameters for group member discovery.

# 1. Introduction/Discussion

The parameters defined in clause 5.9.1 covers both ProSe Direct Discovery and Group Member Discovery. It is not clear which parameter is included in Group Member Discovery message by only reference to clause 5.9.1. Thus, this contribution proposes to clarify the parameters for group member discovery.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

##### 6.3.2.2.2	Procedure for Group Member Discovery with Model A

Depicted in Figure 6.3.2.2.2-1 is the procedure for Group Member Discovery with Model A.

Figure 6.3.2.2.2-1: Group Member Discovery with Model A

1.	The announcing UE sends a Group Member Discovery Announcement (Source Layer 2 ID, Destination Layer 2 ID ,Discovery Group ID, User Info ID (i.e., Announcer Info)) message. The parameters contained in this message are described in clause 5.9.1.

NOTE:	A UE may send multiple Group Member Discovery Announcement messages (Model A) if the UE belongs to more than one discovery group.

##### 6.3.2.2.3	Procedure for Group Member Discovery with Model B

Depicted in Figure 6.3.2.2.3-1 is the procedure for Group Member Discovery with Model B.

Figure 6.3.2.2.3-1: Group Member Discovery with Model B

1.	The discoverer UE sends a Group Member Discovery Solicitation (Source Layer 2 ID, Destination Layer 2 ID, Discovery Group ID, User Info ID (i.e., Discoverer Info, Target Info)) message. The parameters contained in this message are described in clause 5.9.1.

2.	The discoveree UEs that match the values of the parameters contained in the solicitation message based on the Discovery Group ID, respond to the discoverer UE with a Group Member Discovery Response (Source Layer 2 ID, Destination Layer 2 ID, Discovery Group ID, User Info ID (i.e., Discoveree Info)) message. The parameters contained in this message are described in clause 5.9.1.

# * * * * End of changes * * * *