3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102590

Elbonia, April 12 – 16, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, MediaTek

Title:	Clause 6.1.1.7.2 CP protocol stack for Layer-2 UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: Addition of the control plane protocol stack for Layer-2 UE-to-Network Relay.

# 1. Introduction/Discussion

This contribution adds the control plane protocol stack for Layer-2 UE-to-Network Relay to TS 23.304.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

##### 6.1.1.7.2	Layer-2 UE-to-Network Relay

Figure 6.1.1.7.2-1 illustrates the protocol stack of the NAS connection for the Remote UE for NAS-MM and NAS-SM. The NAS messages are transparently transferred between the Remote UE and NG-RAN over the Layer 2 UE-to-Network Relay UE using:

-	PDCP end-to-end connection between the Remote UE and NG-RAN, where the role of the UE-to-Network Relay UE is to relay the PDUs over the signalling radio bear without any modifications and using the functionality of the adaptation layer as specified in TS 38.XXX.

-	Connection between NG-RAN and AMF over N2.

-	Connection between AMF and SMF over N11.

Figure 6.1.1.7.2-1: Control Plane for L2 UE-to-Network Relay

# * * * * End of changes * * * *