SA WG2 Meeting #144E e-meeting	S2-2102612

Elbonia, April 12 – 16, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0301 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add NSACF as input source for network slice load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  |  | Date: |  | 2021-02-18 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the description in clause 5.15.11, NSACF monitors the number of registeed UEs and established PDU Sessions per network slice. This CR is proposed to add NSACF as the source of input data for network slice load analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NSACF as the source of input data of network slice load analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The source of input data is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2A, 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

6.3.2A	Input data

The detailed information collected by the NWDAF is listed in Table 6.3.2A-1.

Table 6.3.2A-1: Input data for slice load analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamps | 5GC NF | A time stamp associated with the collected information. |
| UE registered in a Network Slice/Network Slice instance | AMF, OAM, NSACF | Current number of UEs registered in a NW slice or NW slice instance. |
| PDU session established on a Network Slice/Network Slice instance | SMF, OAM, NSACF | Current number of established PDU sessions in a NW slice or NW slice instance. |
| Load of NFs associated to Network Slice instance | OAM, NRF | Resource utilization information of a Network Slice instance obtained from its constituent NF instances. NF instance load input data collection is described in clause 6.5, Table 6.5.2-1. |

* * * * Next Change * * * *

6.3.4	Procedures

Figure 6.3.4-1: Network Slice load analytics provided by NWDAF

Figure 6.3.4-1 shows the procedure for NWDAF to derive slice load analytics. The steps are described as follows:

1.	A consumer NF subscribes to/requests a NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation (Analytics ID = Load level information and a set of Analytics Filters (e.g. S-NSSAI, NSI ID, Area of Interest)).

2.	[OPTIONAL] If the NWDAF does not have already the slice information, it gains the slice information from OAM (as described in clause 6.2.3) and discovers from NRF the AMF, SMF and NSSF instance(s) relevant to the Analytics Filters provided in the analytics subscription.

3.	[OPTIONAL] If the NSI ID(s) are not provided in the analytics subscription by the consumer NF, the NWDAF invokes Nnssf_NSSelection_Get service operation from NSSF to obtain the NSI ID(s) corresponding to the S-NSSAI in the subscription.

4a.	[OPTIONAL] The NWDAF may subscribe to input data in Table 6.3.2A-1 from the OAM according to the data collection principles from the OAM described in clause 6.2.3.

4b.	[OPTIONAL] The NWDAF may collect input data from the NRF (see clause 6.5) to derive slice instance resource usage statistics and predictions for a Network Slice instance.

5.	The NWDAF subscribes to the AMF's event exposure service to collect data on the number of UEs currently registered on certain Network Slice and, if available, its constituent Network Slice instance(s). An UE access and mobility information event using event ID "UE moving in or out of Area of Interest" is used for that purpose as defined in TS 23.502 [3] using as an example Event Filters S-NSSAI and, if available, NSI ID(s), and optionally Area of Interest. If required, the NWDAF may also collect the corresponding UE IDs.

6.	The NWDAF subscribes to the SMF's event exposure service to collect data on the number of PDU sessions currently registered on certain Network Slice and, if available, its constituent Network Slice instance(s). A PDU Session establishment/release related event is used for that purpose as defined in TS 23.502 [3].

7.	[OPTIONAL] The NWDAF may subscribe to the NSACF’s event exposure service to collect data on number of UEs and/ or PDU Sessions currently registered on certain Network Slice.

8.	The NWDAF derives slice load analytics.

9.	The NWDAF delivers analytics to the consumer NF by invoking Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response service operations.

* * * * End of Changes * * * *