3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-212689

# Elbonia, April 12th - 16th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0303 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNA KI#8 mapping between UE IP address and UE ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The mapping between UE IP adderss and UE ID needs to be resolved for eNA KI#8. Regarding the reason illustrated in DP S2-2102686, the CR is proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - AF/NEF provides S-NSSAI and DNN to NRF during NRF registration and the S-NSSAI and DNN is responded to NWDAF during NRF discovery. - Before sending AF the request of UE data reporting, NWDAF/NEF perform the retrival of UE IP address/prefix using S-NSSAI and DNN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE data collection procedure is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3, 6.2.8.1, 6.2.8.2.2, 6.2.8.2.4.1 (new clause), 6.2.8.2.4.2 (new clause), 6.2.8.2.4.3 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 23502 CR 2690 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------------------Strat of Change----------------------------------------------------------

6.2.2.3	Procedure for Data Collection from AF via NEF

The procedure in Figure 6.2.2.3-1 is used by NWDAF to collect information from AFs via the NEF.

NOTE 1:	In this release, AF registers its available data to NWDAF via OAM configuration at NEF.

The AF collectable data information includes: AF identification, AF service identification (e.g. endpoint information of Naf_EventExposure), available data to be collected per application (e.g. identified by Event ID(s)).

Figure 6.2.2.3-1: Data Collection from AF via NEF

1a.	After the registration of AF available data at the NEF, NEF generates an event exposure with new EventID to be associated with available data to be collected from AF. NEF invokes Nnrf_NFManagement_NFUpdate_request service operation to update its registration information (i.e. NEF Profile) including the generated Event IDs, and associated AF identification, Application ID(s), supported S-NSSAI and DNN.

1b.	NRF stores the received NEF registration information including available data to be collected from AF.

1c.	NRF sends Nnrf_NFManagement_NFUpdate_response message to NEF.

1d.	When NWDAF needs to discovery the available data from AFs and the appropriated NEF to collect this data, NWDAF invokes Nnrf_NFDiscovery_Request_request service operation using as parameter the NEF NF Type, a list of Event ID(s), and optionally AF identification, application ID.

1e.	NRF matches the requested query for available data in AFs with the registered NEF Profiles and sends this information via Nnrf_NFDiscovery_Request_response message to NWDAF. The NRF provides S-NSSAI and DNN to NWDAF if available.

NOTE 2:	After the registration and discovery procedure described in step 1, NWDAF identifies the available data per AF per application and the proper NEF to collect such data.

2.	The NWDAF subscribes to or cancels subscription to data in AF via NEF by invoking the Nnef_EventExposure_Subscribe or Nnef_EventExposure_Unsubscribe service operation. If the event subscription is authorized by the NEF, the NEF records the association of the event trigger and the NWDAF identity.

NOTE 3:	User consent for retrieving user data in AF via NEF is not specified in this Release.

3.	Based on the request from the NWDAF, the NEF subscribes to or cancels subscription to data in AF by invoking the Naf_EventExposure_Subscribe/ Naf_EventExposure_Unsubscribe service operation.

4.	If the NEF subscribes to data in AF, the AF notifies the NEF with the data by invoking Naf_EventExposure_Notify service operation according to Event Reporting Information in the subscription.

5.	If the NEF receives the notification from the AF, the NEF notifies the NWDAF with the data by invoking Nnef_EventExposure_Notify service operation.

When the Reporting type is provided at step 2, the NWDAF determines that the events are disappeared, if the same events are included in the notification compared to the previous notification. Otherwise, NWDAF determines the events are newly appeared or changed. Also, the NWDAF restores the events that are not included in the notification, but included in the previous notification.

If the Granularity of dynamics is applied to the subscription, the NWDAF shall infer the events in the AF from the events in the previous notification and the applied Granularity of dynamics.

-------------------------------------------------------------------Second Change----------------------------------------------------------

6.2.8.1	General

The NWDAF may interact with an AF to collect data from UE Application(s) as an input for analytics generation. The AF can be in the MNO domain or an AF external to MNO domain. The data collection request from NWDAF may trigger the AF to collect data from the UE Application.

The UE establishes a connection to the AF in the MNO domain or external to MNO domain over user plane via a PDU session. The AF communicates with the UE Application and collects data from UE Application.

For both an AF in trusted domain and an AF in untrusted domain (which supports to collect data from a UE application), the SLA between the operator and the application service provider (i.e. ASP) determines:

-	The AF for the UE Application to connect to (e.g. based on the FQDN).

-	The information that the UE Application shares with the AF, subject to user consent.

-	Possible Data Anonymization, Aggregation or Normalization, algorithms (if used).

-	The authentication information that enable the AF to verify the authenticity of the UE's Application that provides data.

NOTE 1:	The mutual authentication info that is used by the UE Application and the AF and how user consent is obtained is out of SA WG2's scope.

-	The supported S-NSSAI and DNN.

The AF (which supports the data collection) is configured based on the SLA above.

A UE Application (which supports to providing data to an AF) is configured/ provisioned with the below information from the ASP:

-	The address of the AF to contact.

-	The parameters that the UE Application is authorized to provide to the AF.

-	The authentication information to enable the UE Application to verify the authenticity of the AF that requests data.

NOTE 2:	The authentication and authorization info that is used by the UE Application and the AF for collection and how user consent is obtained is out of SA2's scope.

NOTE 3:	The configuration procedure for the above information from the ASP to the UE's Application is out of SA WG2 scope.

--------------------------------------------------------------------Next Change----------------------------------------------------------

6.2.8.2.2	AF registration and discovery with NRF

The AF registers its available NF profile to the NRF. The AF in trusted domain registers to the NRF by using the Nnrf_NFManagement service that is defined in clause 5.2.7.2 of TS 23.502 [3]. The AF in untrusted domain registers the available NF profile to the NRF via the NEF as described in clause 6.2.2.3.

Since the UE Application provides the external application ID as described in clause 6.2.8.2.1 while the NWDAF request contains the SUPI and the internal application ID, the AF needs to correlate both the external application ID and the internal application ID. The AF (in trusted domain) or NEF (for AF in untrusted domain) registers at the NRF both the external application ID and internal application ID (or any relevant Event ID) and the S-NSSAI and DNN as part of its available NF profile as also described in clause 6.2.2.3.

NOTE:	For the AF supporting data collection, it is only supported to register the external application ID and internal application ID (or any relevant Event ID) as AF profiles to NRF in this release.

The NWDAF discovers the AF serving the Internal Application ID (or any relevant Event ID) and the associated S-NSSAI and DNN from NRF by using the Nnrf_NFDiscovery service that defined in clause 5.2.7.3 of TS 23.502 [3].

--------------------------------------------------------------------Next Change----------------------------------------------------------

6.2.8.2.4	Correlation between UE data collection and the NWDAF data request

6.2.8.2.4.1	General

The UE IP address is used to identify the user plane connection established between the UE application and the AF for data collection, while the AF receives the Naf_Event_Exposure_Subscribe to request for the specific UE data collection by using SUPI (for AF in trusted domain) or external UE ID (i.e. GPSI) (for AF in untrusted domain).

6.2.8.2.4.2	Mapping between UE IP address and SUPI for AF in trusted domain

Figure 6.2.8.2.4.2-1 UE IP address mapping procedure for trusted AF

1.  NWDAF needs to get UE data for a SUPI and there is no mapping between SUPI and UE IP address, then NWDAF sends Nudm_EventExposure_Subscribe including SUPI and the S-NSSAI/DNN, requesting UDM to provide the serving SMF associated to the S-NSSAI/DNN.

2.  UDM responds the serving SMF ID associated to the S-NSSAI/DNN for the UE.

3. Based on the SMF ID received from UDM, NWDAF sends Nsmf_EventExposure_Subscribe including the UE ID to the SMF.

4. SMF responds the UE IP address/prefix to NWDAF.

5. NWDAF stores the mapping between UE IP address/prefix and SUPI. The mapping can be used for the data collection request in future.

6. NWDAF sends Naf_EventExposure_Subscribe including UE IP address/prefix and optionally SUPI to AF.

7. The AF responds to NWDAF with UE data.

Editor’s Note: It is FFS for the impact to the procedure when the HTTP connection was not extablished between AF and UE

If the target of UE data reporting sent from consumer is group of UEs or any UEs, the NWDAF may request UDM with a list of SUPIs corresponds to the group of UE and get one of more SMF IDs in step 1 and 2. And NWDAF interacts with the one of more SMFs to get the UE IP addresses/prefixes corresponding to each of the group of UEs in step 3 and 4. Then NWDAF sends requests to AF with multiple UE IP addresses/prefixes in step 6.

6.2.8.2.4.3	Mapping between UE IP address and UE External ID for AF in untrusted domain

Figure 6.2.8.2.4.3-1 is the call flow about UE IP address mapping for AF in untrusted domain. The main difference to Figure 6.2.8.2.4.2-1 is that the UE IP address retrieval is performed by NEF instead of NWDAF and NEF also translate SUPI to UE External ID if needed.

Figure 6.2.8.2.4.2-1 UE IP address mapping procedure for untrusted AF

1. NWDAF sends Nnef_EventExposure_Subscribe including SUPI, S-NSSAI, DNN to NEF.

2-5. Instead of NWDAF, the NEF performs the procedure for retrieval of UE IP address/prefix as described in step 1-4 in Figure 6.2.8.2.4.2-1.

6-7. NEF stores the mapping between UE IP address/prefix and SUPI. And sends Naf_EventExposure_Subscribe in cluding the UE IP address/prefix and optionally UE External ID to AF.

Editor’s Note: It is FFS for the impact to the procedure when the HTTP connection was not extablished between AF and UE

--------------------------------------------------------------------End of Changes --------------------------------------------------------