SA WG2 Meeting #144e	S2-2102694

# 12-16 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0307 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editor's note resolution for data and analytics collection via messaging framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Resolve ENs for Data and Analytics Collection via Messaging Framework |  |  |  |  |  |  |  |  |
| Summary of change: |  | - EN on how the DCCF leans/chooses the MFAF Notification Target Addess (+ MFAF Notification Correlation ID) is resolved. When data or analytics are not already being provided to the MFAF, the MFAF provides the MFAF notification endpoint information to the DCCF in the response to a “configure” message sent by the DCCF. - Redundant text in step 4 of clause 6.1.4.5 is removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs are not resolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4.4, 6.1.4.5, 6.2.6.3.4, 6.2.6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.1.4.4	Analytics Exposure via Messaging Framework

The procedure as depicted in Figure 6.1.4.4-1 is used by analytics consumer(s) (e.g. NFs/OAM) to subscribe/unsubscribe to NWDAF analytics and be notified of analytics information, using Ndccf_DataManagement_Subscribe service operation. The 3GPP DCCF Adaptor (3da) Data Management service and 3GPP Consumer Adaptor (3ca) Data Management service of the Messaging Framework Adaptor Function (MFAF) are used to interact with the 3GPP Network and the Messaging Framework. Whether a NWDAF service consumer directly contacts the NWDAF or goes via the DCCF is based on NWDAF service consumer configuration.

Figure 6.1.4.4-1: Network data analytics subscription via DCCF

1.	Analytics consumer subscribes to analytics information via DCCF by invoking the Ndccf_DataManagement_Subscribe (Nnwdaf service operation, Analytics Specification, Formatting Instructions, Processing Instructions, NWDAF (or NWDAF-Set) ID, ADRF Information, Analytics Consumer Notification Target Address (+ Notification Correlation ID)) service operation. The analytics consumer may specify one or more notification endpoints. Analytics consumer decides to go via DCCF based on internal configuration. The "Analytics Specification" provides Nnwdaf service operation specific parameters, e.g. Analytics IDs, target of analytics reporting, and optional parameters used to retrieve the analytics. The analytics consumer may provide the identity of the NWDAF to collect analytics from. The analytics consumer may provide additional information on possible notification endpoints or ADRF information to archive analytics.

2.	If the NWDAF instance or NWDAF Set is not identified by the analytics consumer, the DCCF determines the NWDAF instances that can provide analytics. If the consumer requested storage of analytics in an ADRF but an ADRF ID is not provided by the Analytics Consumer, or the collected analytics is to be stored in an ADRF according to configuration on the DCCF, the DCCF selects an ADRF to store the collected analytics.

3.	The DCCF determines whether the analytics requested in step 1 are already being collected. If the requested analytics are already being collected by an analytics consumer, the DCCF adds the new analytics consumer to the list of analytics consumers that are subscribed for these analytics.

4.	The DCCF sends an Nmfaf_3daDataManagement_Configure(Analytics Consumer Information, MFAF Notification Information, Formatting Conditions, Processing Instructions) to configure the MFAF to map notifications received from the NWDAF to outgoing notifications sent to endpoints, and to instruct the MFAF how to format and process the outgoing notifications.

"Analytics Consumer Information" contains for each notification endpoint, the analytics consumer Notification Target Address (+ Analytics Consumer Notification Correlation ID) to be used by the MFAF when sending notifications in step 7.

"MFAF Notification Information" is included if an NWDAF is already sending the analytics to the MFAF. MFAF Notification Information identifies Event Notifications received from the NWDAF and  comprises the MFAF Notification Target Address (+ MFAF Notification Correlation ID). If the MFAF does not receive MFAF Notification information from the DCCF, the MFAF selects an MFAF Notification Target Address (+ MFAF Notification Correlation ID) and sends the MFAF Notification Information, containing the MFAF Notification Target Address (+ MFAF Notification Correlation ID), to the DCCF in the Nmfaf_3daDataManagement_Configure Response.

5.	If the analytics subscribed in step 1 partially matches analytics that are already being collected by the DCCF from a NWDAF, and a modification of this subscription to the NWDAF would satisfy both the existing analytics subscriptions as well as the newly requested analytics, the DCCF invokes Nnwdaf_AnalyticsSubsription_Subscribe(Subscription Correlation ID) with parameters indicating how to modify the previous subscription (as specified in clause 6.1.1.1). The DCCF adds the analytics consumer to the list of analytics consumers that are subscribed for these analytics.

If the analytics requested at step 1 are not already available or not being collected yet, the DCCF subscribes to analytics from the NF using Nnwdaf_AnalyticsSubsription_Subscribe, setting the Notification Target Address (+ Notification Correlation ID)) to the MFAF Notification Target Address (+ MFAF Notification Correlation ID) received in step 4. The DCCF adds the analytics consumer to the list of analytics consumers that are subscribed for these analytics.

6.	When new output analytics are available, the NWDAF uses Nnwdaf_AnalyticsSubscription_Notify to send the analytics to the MFAF. The Notification includes the MFAF Notification Correlation ID.

7.	The MFAF uses Nmfaf_3caDataManagement_Notify to send the analytic to all notification endpoints indicated in step 4. Notifications are sent to the Notification Target Address(es) using the Analytics Consumer Notification Correlation ID(s) received in step 4. Analytics sent to notification endpoints may be processed and formatted by the MFAF, so they conform to delivery requirements specified by the analytics consumer. The MFAF may store the information in the ADRF if requested by consumer or if required by DCCF configuration

NOTE:	According to Formatting Instructions provided by the Analytics Consumer, multiple notifications from a NWDAF can be combined in a single Nmfaf_3caDataManagement_Notify, so many notifications from the NWDAF results in fewer notifications (or one notification) to the analytics consumer. Alternatively, a notification can instruct the analytics notification endpoint to fetch the analytics from the MFAF before an expiry time.

8.	If a Nmfaf_3caDataManagement_Notify contains a fetch instruction, the notification endpoint sends a Nmfaf_3caDataManagement_Fetch request to fetch the analytics from the MFAF.

9.	The MFAF delivers the analytics to the notification endpoint.

10.	When the analytics consumer no longer wants analytics to be collected, it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

11.	If there are no other analytics consumers subscribed to the analytics, the DCCF unsubscribes with the NWDAF.

12.	The DCCF de-configures the MFAF so it no longer maps notifications received from the NWDAF to the notification endpoints configured in step 4.

NEXT CHANGE

#### 6.1.4.5	Historical Analytics Exposure via Messaging Framework

The procedure as depicted in Figure 6.1.4.5-1 is used by an analytics consumer (e.g. NFs/OAM) to obtain historical analytics via the messaging framework. Historical analytics may be previously computed statistics or predictions stored in an NWDAF or ADRF. Statistics may be previously computed and stored in the ADRF or NWDAF and can be identified by a "target period" in the past (see clause 6.1.3). Requests for previously computed predictions have a "Time Window", which specifies an allowable span for when the predictions may have been computed. This allows the analytics consumer to request previously computed predictions for a target period.

The analytics consumer requests analytics via the DCCF, using Nddcf_DataManagement_Request service operation. Whether the NWDAF service consumer directly contacts the NWDAF / ADRF, or goes via the DCCF is based on configuration.

Figure 6.1.4.5-1: Historical Analytics Exposure via Messaging Framework

1.	The analytics consumer requests analytics via DCCF by invoking the Ndccf_DataManagement_Subscribe(Nnwdaf service operation, Analytics Specification, Time Window, Formatting Instructions, Processing Instructions, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID). The analytics consumer may specify one or more notification endpoints to receive the analytics.

2.	If an ADRF or NWDAF instance or ADRF or NWDAF Set ID is not provided by the Analytics Consumer, the DCCF determines if any ADRF or NWDAF instances might provide the analytics as described in clause 5A.

Editor's note:	Clause 5A is to provide an ADRF functional description. ADRF or NWDAF selection by the DCCF may be based on the ADRF or NWDAF profile in the NRF (e.g.: ADRF or NWDAF that stores analytics for a specific Analytics ID in a geographic area).

3.	The DCCF sends an Nmfaf_3daDataManagement_Configure(Analytics Consumer Information, Formatting Conditions, Processing Instructions) to configure the MFAF to map notifications received from the ADRF or NWDAF to outgoing notifications sent to endpoints, and to instruct the MFAF how to format and process the outgoing notifications.

"Analytics Consumer Information" contains for each notification endpoint, the analytics consumer Notification Target Address (+ Analytics Consumer Notification Correlation ID) to be used by the MFAF when sending notifications. The MFAF selects an MFAF Notification Target Address (+ MFAF Notification Correlation ID) and sends the MFAF Notification Information, containing the MFAF Notification Target Address (+ MFAF Notification Correlation ID), to the DCCF in the Nmfaf_3daDataManagement_Configure Response.

4.	(conditional) If the DCCF determines that an ADRF instance might provide the analytics, or an ADRF instance or Set was supplied by the Analytics Consumer, the DCCF sends a request to the ADRF, using Nadrf_DataRetrieval_Subscribe(Analytics Specification, MFAF Notification Information). The MFAF Notification information contains the MFAF Notification Target Address (+ MFAF Notification Correlation ID) received in step 3.)

Editor's note:	Appropriate reference to service operation specification clause will be added when available.

5.	The ADRF responds to the DCCF with an Nadrf_DataRetrieval_Subscribe response indicating if the ADRF can supply the analytics. If the analytics can be provided, the procedure continues with step 8.

6.	(conditional) If the DCCF determines that an NWDAF instance might provide the analytics or an NWDAF instance or Set was supplied by the Analytics Consumer, the DCCF sends a request to the NWDAF using Nnwdaf_DataManagement_Request(Analytics Specification, MFAF Notification Information). The MFAF Notification Information contains the MFAF Notification Target Address (+ MFAF Notification Correlation ID) received in step 3.

Editor's note:	Appropriate reference to service operation specification clause will be added when available.

7.	The NWDAF responds to the DCCF with an Nnwdaf_DataManagement_Subscribe response indicating if the NWDAF can supply the analytics.

8.	The ADRF or the NWDAF sends the requested analytics (e.g. one or more stored notifications archived from an NWDAF) to the MFAF. The analytics may be sent in one or more notification messages.

9.	The MFAF uses Nmfaf_3caDataManagement_Notify to send analytics to all notification endpoints indicated in step 3. Notifications are sent to the Notification Target Address(es) using the Analytics Consumer Notification Correlation ID(s) received in step 3. Analytics sent to notification endpoints may be processed and formatted by the DCCF, so they conform to delivery requirements specified by the analytics consumer.

NOTE:	According to Formatting Instructions provided by the Analytics Consumer, multiple notifications from an ADRF or NWDAF can be combined in a single Ndccf_DataManagement_Notify so many notifications from the ADRF or NWDAF results in fewer notifications (or one notification) to the Analytics Consumer. Alternatively, a Nmfaf_3caDataManagement_Notify can instruct the analytics notification endpoint to fetch the analytics from the DCCF before an expiry time.

10.	If a notification contains a fetch instruction, the notification endpoint sends a Nmfaf_DataManagement_Fetch request to fetch the analytics from the MFAF.

11.	The DCCF delivers the analytics to the notification endpoint.

12.	When the analytics consumer no longer wants analytics to be collected or has received all the analytics it needs, it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

13.	If the analytics are being provided by an ADRF and there are no other analytics consumers subscribed to the analytics, the DCCF unsubscribes with the ADRF.

14.	If the analytics are being provided by an NWDAF and there are no other analytics consumers subscribed to the analytics, the DCCF unsubscribes with the NWDAF

15.	The DCCF de-configures the MFAF so it no longer maps notifications received from the NWDAF to the notification endpoints configured in step 3.

NEXT CHANGE

##### 6.2.6.3.4	Data Collection via Messaging Framework

This procedure depicted in figure 6.2.6.3.4-1 is used by a data consumer (e.g. NWDAF) to obtain data and be notified of events using the DCCF and a Messaging Framework. The 3GPP DCCF Adaptor (3da) Data Management service and 3GPP Consumer Adaptor (3ca) Data Management service of the Messaging Framework Adaptor Function (MFAF) are used to interact with the 3GPP Network and the Messaging Framework. Whether the data consumer directly contacts the Data Source or goes via the DCCF is based on configuration.

Figure 6.2.6.3.4-1: Data Collection via Messaging Framework

- The data consumer subscribes to data via the DCCF by invoking the Ndccf_DataManagement_Subscribe(Service_Operation, Data Specification, Formatting Instructions, Processing Instructions, NF (or NF-Set) ID, ADRF Information, Data Consumer Notification Target Address (+ Notification Correlation ID)) service operation. The data consumer may specify one or more notification endpoints and the NF or NF set to collect data from.

Service_Operation is the service operation to be used by the DCCF to request data (e.g.: Namf_EventExposure_Subscribe or OAM Subscribe). Data Specification provides Service_Operation-specific required parameters (e.g. event IDs, UE-ID(s), target of event reporting) and optional input parameters used to retrieve the data. Formatting and Processing Instructions are as defined in clause 5A.4. The data consumer may optionally include the Data Source NF Instance (or NF Set) ID and ADRF information indicating whether the data are to be stored in an ADRF and, optionally, an ADRF ID.

Editor’s Note: Clause xx will provide the Ndccf_DataManagement service description.

2.	If the NF instance or NF Set ID is not provided by the data consumer, the DCCF determines the NF instances that can provide data as described in clause 5A.2 and clause 6.2.2.2. If the consumer requested storage of data in an ADRF, but the ADRF ID is not provided by the data consumer, or the collected data is to be stored in an ADRF according to configuration on the DCCF, the DCCF selects an ADRF to store the collected data.

3.	The DCCF determines whether the data requested in step 1 are already being collected, as described in clause 5A.2.

If the data requested are already being collected by a data consumer, the DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

4.	The DCCF sends an Nmfaf_3daDataManagement_Configure(Data Consumer Information, MFAF Notification Information, Formatting Conditions, Processing Instructions) to configure the MFAF to map notifications received from the Data Source to outgoing notifications sent to endpoints, and to instruct the MFAF how to format and process the outgoing notifications.

Data Consumer Information contains for each notification endpoint, the data consumer  Notification Target Address (+ Data Consumer Notification Correlation ID to be used by the MFAF when sending notifications in step 7.

MFAF Notification Information is included if a Data Source is already sending the data to the MFAF. MFAF Notification Information identifies Event Notifications received from the Data Sources and comprises the MFAF Notification Target Address (+ MFAF Notification Correlation ID). If the MFAF does not receive MFAF Notification information from the DCCF, the MFAF selects a MFAF Notification Target Address (+ MFAF Notification Correlation ID) and sends the MFAF Notification Information, containing MFAF Notification Target Address (+ MFAF Notification Correlation ID), to the DCCF in the Nmfaf_3daDataManagement_Configure Response.

5.	If the data subscribed in step 1 partially matches data that are already being collected by the DCCF from a Data Source, and a modification of this subscription to the Data Source would satisfy both the existing data subscriptions as well as the newly requested data, the DCCF invokes Nnf_EventExposure_Subscribe(Subscription Correlation ID) with parameters indicating how to modify the previous subscription (as specified in clause 5A.2). The DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

If the data requested at step 1 are not already available or not being collected yet, the DCCF subscribes to data from the NF using the Nnf_EventExposureSubscribe(Data Specification, MFAF Notification Target Address (+ MFAF Notification Correlation ID)) service operation as specified in clause 5A.2 and clause 6.2.2.2, using the MFAF Notification Target Address (+ MFAF Notification Correlation ID) received in step 4. The DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

6.	When new output data are available, the Data Source uses Nnf_EventExposure_Notify to send the data to the MFAF. The Notification includes the MFAF Notification Correlation ID.

7.	The MFAF uses Nmfaf_3caDataManagement_Notify to send the data to all notification endpoints indicated in step 4. Notifications are sent to the Notification Target Address(es) using the Data Consumer Notification Correlation ID(s) received in step 4. Data sent to notification endpoints may be processed and formatted by the MFAF, so they conform to delivery requirements specified by the data consumer. The MFAF may store the information in ADRF if requested by consumer or if required by DCCF configuration

NOTE:	According to Formatting Instructions provided by the data consumer, multiple notifications from a Data Source can be combined in a single Nmfaf_3caDataManagement_Notify, so many notifications from the Data Source results in fewer notifications (or one notification) to the data consumer. Alternatively, a notification can instruct the data notification endpoint to fetch the data from the MFAF before an expiry time.

8.	If a Nmfaf_3caDataManagement_Notify contains a fetch instruction, the notification endpoint sends a Nmfaf_3caDataManagement_Fetch request to fetch the data from the MFAF.

9.	The MFAF delivers the data to the notification endpoint.

10.	When the data consumer no longer wants data to be collected, it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

11.	If there are no other data consumers subscribed to the data, the DCCF unsubscribes with the Data Source.

12.	The DCCF de-configures the MFAF so it no longer maps notifications received from the Data Source to the notification endpoints configured in step 4.

NEXT CHANGE

##### 6.2.6.3.5	Historical Data Collection via Messaging Framework

The procedure depicted in figure 6.2.6.3.5-1 is used by data consumers (e.g. NWDAF) to obtain historical data, i.e. data related to past time period. The data consumer obtains data using Ndccf_DataManagement_Subscribe service operation, where the subscription results in one or more notifications depending on how the data is retrieved from the ADRF or NWDAF and how the data is formatted. Whether the data consumer uses this procedure or directly contacts the ADRF or NWDAF is based on configuration.

Figure 6.2.6.3.5-1: Historical Data Collection via Messaging Framework

1.	The data consumer requests data via DCCF by invoking the Ndccf_DataManagement_Subscribe(Service Operation, Data Specification, Time Window, Formatting Instructions, Processing Instructions, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID). The data consumer may specify one or more notification endpoints to receive the data.

Service_Operation is the service operation used to acquire the data from a data source, Data Specification provides Service_Operation-specific required parameters (e.g. event IDs, UE-ID(s) and optional input parameters used to retrieve the data. Time Window specifies a past time period and comprises a start and stop time, and Formatting and Processing Instructions are as defined in clause 5A4. The data consumer may optionally include the ADRF or NWDAF instance (or ADRF Set or NWDAF Set) ID where the stored data resides.

Editor's note:	Clause xx will provide the Ndccf_DataManagement service description.

2.	If an ADRF or NWDAF instance or ADRF Set ID or NWDAF Set ID is not provided by the data consumer, the DCCF determines if any ADRF or NWDAF instances might provide the data as described in clause 5B and 5A.2.

NOTE:	An ADRF or NWDAF may have previously registered data it is collecting with the DCCF.

Editor's note:	Clause 5B is to provide an ADRF functional description. ADRF or NWDAF selection by the DCCF may be based on the ADRF or NWDAF profile in the NRF (e.g.: ADRF or NWDAF that stores data for a specific NF Type in a geographic area) in addition to registrations by the ADRF or NWDAF to the DCCF of data the ADRF or NWDAF is collecting.

3.	The DCCF sends an Nmfaf_3daDataManagement_Configure(Data Consumer Information,  Formatting Conditions, Processing Instructions) to configure the MFAF to map notifications received from the ADRF or NWDAF to outgoing notifications sent to endpoints, and to instruct the MFAF how to format and process the outgoing notifications.

"Data Consumer Information" contains for each notification endpoint, the data consumer Notification Target Address (+ Data Consumer Notification Correlation ID) to be used by the MFAF when sending notifications. The MFAF selects an MFAF Notification Target Address (+ MFAF Notification Correlation ID) and sends the MFAF Notification Information, containing MFAF Notification Target Address (+ MFAF Notification Correlation ID), to the DCCF in the Nmfaf_3daDataManagement_Configure Response.

4.	(conditional) If the DCCF determines that an ADRF instance might provide the data, or an ADRF instance or Set was supplied by the data consumer, the DCCF sends a request to the ADRF, using Nadrf_DataRetrieval_Subscribe(Data Specification, MFAF Notification Information)  containing the MFAF Notification Target Address (+ MFAF Notification Correlation ID) received in step 3

Editor's note:	Appropriate reference to service operation specification clause will be added when available.

5.	The ADRF responds to the DCCF with an Nadrf_DataRetrieval_Subscribe response indicating if the ADRF can supply the data. If the data can be provided, the procedure continues with step 8.

6.	(conditional) If the DCCF determines that an NWDAF instance might provide the data, or an NWDAF instance or NWDAF Set was supplied by the data consumer, the DCCF sends a request to the NWDAF, using Nnwdaf_DataManagement_Subscribe(Data Specification, MFAF Notification Information). MFAF Notification Information contains the MFAF Notification Target Address (+ MFAF Notification Correlation ID) received in step 3. .

Editor's note:	Appropriate reference to service operation specification clause will be added when available.

7.	The NWDAF responds to the DCCF with an Nnwdaf_DataManagement_Subscribe response indicating if the NWDAF can supply the data.

8.	The ADRF or the NWDAF sends the requested data (e.g. one or more stored notifications archived from a data source) to the MFAF. The data may be sent in one or more notification messages.

9.	The MFAF uses Nmfaf_3caDataManagement_Notify to send data to all notification endpoints indicated in step 3. Notifications are sent to the Notification Target Address(es) using the Data Consumer Notification Correlation ID(s) received in step 3. Data sent to notification endpoints may be processed and formatted by the MFAF, so they conform to delivery requirements specified by the data consumer.

NOTE:	According to Formatting Instructions provided by the data consumer, multiple notifications from an ADRF or NWDAF can be combined in a single Nmfaf_3caDataManagement_Notify so many notifications from the ADRF or NWDAF results in fewer notifications (or one notification) to the data consumer. Alternatively, a Nmfaf_3caDataManagement_Notify can instruct the data notification endpoint to fetch the data from the MFAF before an expiry time.

10.	If a notification contains a fetch instruction, the notification endpoint sends a Nmfaf_3caDataManagement_Fetch request to fetch the data from the MFAF.

11.	The MFAF delivers the data to the notification endpoint.

12.	When the data consumer no longer wants data to be collected or has received all the data it needs, it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

13.	If the data are being provided by an ADRF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the ADRF.

14.	If the data are being provided by an NWDAF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the NWDAF.

15.	The DCCF de-configures the MFAF so it no longer maps notifications received from the ADRF or NWDAF to the notification endpoints configured in step 3.

END OF CHANGES