3GPP TSG-SA2 # 144E (e-meeting)	S2-2102711

# April 12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0312 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data Collection coordinated by DCCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2020-12-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the existing process of Data collection using DCCF, data delivered via the DCCF or a messaging framework, but data delivered can not directly send to the data consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The signaling of data collection can be coordinated by DCCF, and data delivered directly to data consumer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data consumer can not collect data directly from the data source during the coordination of DCCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.1, 6.2.6.3.x(new), 6.2.6.3.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2.6.3	Data Collection using DCCF

6.2.6.3.1	General

This clause specifies procedures for data collection using the DCCF described in clause 5A for cases other than obtaining analytics from an NWDAF (which is specified in clause 6.1.2). Three options are supported: data delivered via the DCCF, according to clauses 6.2.6.3.2 and 6.2.6.3.3, and data delivered via a messaging framework according to clauses 6.2.6.3.4 and 6.2.6.3.5, and data delivered directly to data consumer according to clauses 6.2.6.3.x and 6.2.6.3.y. Which option to be used is determined by DCCF configuration.

* * * Next Changes * * * (all new text)

6.2.6.3.x	Data Collection coordinated by DCCF

The procedure depicted in Figure 6.2.6.3.x-1 is used by a data consumer (e.g. NWDAF) to obtain data and be notified of events directly from Data Source.

Figure 6.2.6.3.x-1: Data Collection coordinated by DCCF

1.	The data consumer subscribes to data via the DCCF by invoking the Ndccf_DataManagement_Subscribe(Service_Operation, Data Specification, NF (or NF-Set) ID) service operation.

Service_Operation is the service operation to be used by the DCCF to request data (e.g.: Namf_EventExposure_Subscribe or OAM Subscribe). Data Specification provides Service Operation-specific parameters (e.g. event IDs, UE-ID(s), target of event reporting) used to retrieve the data. The data consumer may include the Data Source, e.g. NF Instance (or NF Set) ID from which the data needs to be collected.

Editor's note:	Clause 7A will provide the Ndccf_DataManagement service description.

2.	The DCCF determine the NF type(s) and/or OAM to retrieve the data based on the Service Operation requested in step 1. If the NF instance or NF Set ID is not provided by the data consumer. The DCCF determines the NF instances that can provide data as described in clause 5A.2 and clause 6.2.2.2.

3.	The DCCF determines whether the data requested in step 1 is available at DCCF, as described in clause 5A.2.

4.	(conditional) If the data requested are already being collected by a data consumer, the DCCF uses Ndccf_DataManagement_Notify to send data to the data consume. Notifications are sent to the Notification Target Address(es) using the data consumer Notification Correlation ID(s) received in step 1.

5.	The DCCF sends a request to the data consumer using Nnf_EventExposure_Subscribe service operation as specified in clause 5A.2 and clause 6.2.2.2, with DCCF indicated as Notification Target Address. The DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

6.	When new output data are available, the Data Source uses Nnf_EventExposure_Notify to send the data to the  data consumer.

7.	If the data consumer needs to retrieve data from OAM, procedure for data collection from OAM as per steps 1-4 from clause 6.2.3.2 is used.

8.	If a NdccfDataManagement_Notify contains a fetch instruction, the notification endpoint sends a Ndccf_DataManagement_Fetch request to fetch the data from the DCCF.

9.	The DCCF delivers the data to the notification endpoint

10.	When the data consumer no longer wants data to be collected it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

11.	If there are no other data consumers subscribed to the data, the DCCF unsubscribes with the Data Source.

6.2.6.3.y	Historical Data Collection coordinated by DCCF

The procedure depicted in figure 6.2.6.3.y-1 is used by data consumers (e.g. NWDAF) to obtain historical data, i.e. data related to past time period. The data consumer requests data using Ndccf_DataManagement_Subscribe service operation and be notified of events directly from ADRF or NWDAF.

Figure 6.2.6.3.y-1: Historical Data Collection coordinated by DCCF

1.	The data consumer requests data via DCCF by invoking the Ndccf_DataManagement_Subscribe(Service_Operation, Data Specification, Time Window, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID).

"Service_Operation" is the service operation used to acquire the data from a data source. "Data Specification" provides Service_Operation-specific parameters (e.g. event IDs, UE-ID(s)) used to retrieve the data. "Time Window" specifies a past time period and comprises a start and stop time. The data consumer may optionally include the ADRF or NWDAF instance (or ADRF Set or NWDAF Set) ID where the stored data resides.

Editor's note:	Clause 7A will provide the Ndccf_DataManagement service description.

2.	If an ADRF or NWDAF instance or ADRF Set ID or NWDAF Set ID is not provided by the data consumer, the DCCF determines if any ADRF or NWDAF instances might provide the data as described in clause 5B and 5A.2.

NOTE:	An ADRF or NWDAF may have previously registered data it is collecting with the DCCF.

Editor's note:	Clause 5B is to provide an ADRF functional description. ADRF or NWDAF selection by the DCCF may be based on the ADRF or NWDAF profile in the NRF (e.g.: ADRF or NWDAF that stores data for a specific NF Type in a geographic area) in addition to registrations by the ADRF or NWDAF to the DCCF of data the ADRF or NWDAF is collecting.

3.	The DCCF responds to the data consumer with an Ndccf_DataManagement_Subscribe service response with a confirmation of successful request and the subscription correlation ID identifying the requested data.

4.	(conditional) If the DCCF determines that an ADRF instance might provide the data, or an ADRF instance or Set was supplied by the data consumer, the DCCF sends a request to the ADRF, using Nadrf_DataRetrieval_Subscribe(Data Specification, Notification Target Address=the data consumer address) service operation. The ADRF responds to the DCCF with an Nadrf_DataRetrieval_Subscribe response indicating if the ADRF can supply the data. If the data can be provided, the procedure continues with step 6.

Editor's note:	Appropriate reference to service operation specification clause will be added when available.

5.	(conditional) If the DCCF determines that an NWDAF instance might provide the data or an NWDAF instance or Set was supplied by the data consumer, the DCCF sends a request to the NWDAF using Nnwdaf_DataManagement_Request(Data Specification, Notification Target Address=the data consumer address) service operation. The NWDAF responds to the DCCF with an Nnwdaf_DataManagement_Subscribe response indicating if the NWDAF can supply the data. If the data can be provided, the procedure continues with step 6.

Editor's note:	Appropriate reference to service operation specification clause will be added when available.

6	The ADRF or the NWDAF sends the requested data (e.g. one or more stored notifications archived from a data source) to the Data Consumer. The data may be sent in one or more notification messages.

7.	If a notification contains a fetch instruction, the notification endpoint sends a Ndccf_DataManagement_Fetch request to fetch the data from the DCCF.

8.	The DCCF delivers the data to the notification endpoint.

9.	When the data consumer no longer wants data to be collected or has received all the data it needs, it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

10.	If the data are being provided by an ADRF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the ADRF.

11.	If the data are being provided by an NWDAF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the NWDAF.

* * *End of Changes * * *