3GPP TSG-SA2 # 144E (e-meeting)	S2-2102732

# April 12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0314 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Historical Data Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-4-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no procedures for storing historical data of NWDAF, DCCF, MFAF into ADRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedures to store historical data of NWDAF, DCCF, MFAF into ADRF. If there is no matching data in ADRF, it will store the historical data, if there is matching data in ADRF, it will refuse to store the historical data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for historical data handling not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2.6.3.x	Historical Data Handling

The procedure depicted in figure 6.2.6.3.x-1 is used by data consumers (e.g. NWDAF, DCCF or MFAF) to store historical data, i.e. data related to past time period. After the data consumer obtains data, it is necessary for data consumer to store historical data in an ADRF. Whether the data consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

Figure 6.2.6.3.x-1: Historical Data Handling

1.	The data consumer subscribes to data via the DCCF by invoking the Nadrf_DataManagement_request(collected data information, analytics information) service operation. The data consumer may specify one or more ADRF.

Editor's note:	Clause 7.A will provide the Nadrf_DataManagement service description.

2.	The ADRF receives data store request send by data consumer, and then the ADRF determines whether the same data and analytics is being collected or has been collected based on the collected data information and/or analytics information.

3. The ADRF sends Nadrf_DataManagement_Response message to data consumer. When ADRF is determined that there is same data and analytics is being collected or has been collected, ADRF sends a rejection message to data consumer. Conversely, ADRF accepts the request from data consumer.

4. The data consumer notifies the ADRF with the collected data and/or analytics by invoking notify service operation.

* * *End of Changes * * *