3GPP TSG-SA2 # 144E (e-meeting)	S2-2102733

# April 12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0315 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the procedure for Data Collection from NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-4-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The description for the case that NWDAF may trigger further data collection is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional description for the case that NWDAF may trigger further data collection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R17 TS 23.288 does not involve the additional description for the case that NWDAF may trigger further data collection.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2.6.2	Procedure for Data Collection from NWDAF

The procedure in Figure 6.2.6.2-1 is used by NWDAF service consumer to invoke the data management services at NWDAFs in order to retrieve runtime and historical data.

Figure 6.2.6.2-1: Data Collection from NWDAF via Data Management Service

1.	NWDAF service consumer (e.g. NWDAF, DCCF) identifies that further data from an NWDAF is required in order to perform some operation related to analytics ID. The triggers for further data collection are related to:

a)	the local policies of NWDAF, DCCF (e.g. preparation for future requests for analytics ID as specified in clause 6.2.2.1);

b)	a request for analytics generation requiring data not available or not directly reachable via the NWDAF service consumer (e.g. out of the serving area);

c)	a request for model training;

d)	a request for data collection that NWDAF service consumer cannot provide by itself.

Editor's note:	The discovery of the appropriated NWDAF providing the data collection service and the type of data exposed by the NWDAF shall be aligned with the extensions of the NWDAF discovery description.

2a.	NWDAF service consumer invokes Nnwdaf_DataManagement_Subscribe service from NWDAF to request a required data.

When the required data is bulked data event type, the input parameters of the service request include as defined in clause 6.2.6.1: the Data Specification (Event ID, Target of Event Reporting with Analytics ID, type of requested bulked data, analytics stage; Event Filter Information with Target of Analytics Information, Analytics filter information), Formatting and Processing possibly including Consumer Triggered Notification, interval of data sample selection; periodicity of bulked data generation, periodicity of event collection, data collection mode, minimum or maximum number of samples to be included in the bulked data, fetch flag, bulked data deadline, processing rules; and a Notification Target Address (+ Notification Correlation ID) as defined in clause 6.2.6.1.

When the required data is a list of Events, the input parameter of the service request include: Service_Operation is the service operation to be used by NWDAF to request data (e.g.: Namf_EventExposure_Subscribe or OAM Subscribe). Data Specification provides Service Operation-specific parameters (e.g. event IDs, UE-ID(s), target of event reporting) used to retrieve the data. Formatting and Processing Instructions are as defined in clause 5A.4.

The NWDAF service consumer may include the Data Source, e.g. NF Instance (or NF Set) ID from which the data needs to be collected. The NWDAF service consumer may include ADRF information indicating whether the data are to be stored in an ADRF, and optionally an ADRF ID.

Editor's note:	The description of the input parameters of Nnwdaf_DataManagement_Subscribe services when the required data is a list of events has to be aligned with description in clause 5A.4.

Editor's note:	Merge of the two sets of parameters above is FFS.

2b.	Based on the received request, NWDAF creates a new bulked data for the requesting consumer.

If the NWDAF receives a request without the fetch flag activated, NWDAF immediately sends Nnwdaf_DataManagement_Subscription service response with a confirmation of successful request and the subscription correlation ID identifying the requested data.

If the NWDAF receives a request with the fetch flag activated, NWDAF executes step 3 (a, b, c) and 4 before sending the Nnwdaf_DataManagement_Subscription service response.

NOTE 1:	Subscription Correlation ID allows the NWDAF service consumer to request to NWDAF any changes in the generation of a requested data.

3.	NWDAF determines whether the request data is available at such NWDAF.

4a.	(Optional) If NWDAF receives a request for data that is not available or not reachable by such NWDAF (e.g. out of serving area), NWDAF determines the sources for the data that is not available.

4b.	(Optional) NWDAF may trigger further data collection using any of the available mechanisms in clause 6.2.2 and clause 6.2.6 (e.g. recursively using data collection services from other needed NWDAFs, DCCFs, ADRFs or if the data subscribed in step 2a partially matches data that are already being collected by the NWDAF from a Data Source, NWDAF sends a message to the data source, and after receiving a successful response to modify the subscription data, a modification of this subscription to the Data Source would satisfy both the existing data subscriptions as well as the newly requested data ).

5.	Based on the properties of the received request and considering local rules, NWDAF generates the requested data including the available or collected data (e.g. from other NWDAFs, DCCFs or ADRFs).

6a.	If the fetch flag is set to true in step 2a, NWDAF waits until the requested data is ready and sends a Nnwdaf_DataManagement_Notify service message with a confirmation of successful request, the Notification Correlation ID and the target address where the data may be retrieved.

When the requested data is bulked data, the requested data is ready when NWDAF generated the bulked data based on the available or collected data samples from the event notifications and the processing rules applicable.

6b.	If the fetch flag is set to false in step 2a, NWDAF uses the Nnwdaf_DataManagement_Notify service to send the requested data to the NWDAF service consumer.

When the requested data is bulked data, NWDAF waits until the bulked data is generated or the bulked data deadline expires and uses Nnwdaf_DataManagement_Notify service with the indication of the outcome. In case of successful bulked data generation, NWDAF to provide the bulked data via Nnwdaf_DataManagement_Notify service as defined in clause 6.2.6.1. In case of bulked data deadline expiration and unsuccessful bulked data generation, NWDAF to provide the bulked data via Nnwdaf_DataManagement_Notify an indication of an unsuccessful bulked data generation with expired deadline. The Notification Target Address is used by the Nnwdaf_DataManagement_Notify service operation to deliver the message to the NWDAF service consumer.

7(a.b).	Alternatively, when the fetch flag is set to true in Step 2a, the NWDAF service consumer shall fetch the required data from NWDAF via Nnwdaf_DataManagement_Fetch service operation. The NWDAF service consumer invokes the Nnwdaf_DataManagement_Fetch service operation with the input parameters including the Subscription Correlation ID, to identify the data that needs to be fetched and receives a response with the requested data, as defined in clause 6.2.6.1.

8.	The NWDAF service consumer uses the requested data for performing further processing. If the NWDAF service consumer is an NWDAF the requested data can be used for analytics generation or model training or for further exposing such data to other NWDAFs. If the NWDAF service consumer is a DCCF, the requested data can be used to be provided to a DCCF data consumer.

9.	When the NWDAF service consumer determines that no more data is required, it unsubscribe to the requested data from NWDAF. If NWDAF had triggered further data collection in Step 3a and 3b, NWDAF also unsubscribe to all data sources.

NOTE 2:	It is also possible that instead of providing the dataset of the generated bulked data in steps 6a, 6b, 7b, the NWDAF provides a reference to where the dataset can be retrieved by the NWDAF service consumer.

* * *End of Changes * * *