SA WG2 Meeting #144E e-meeting	S2-2102740

Elbonia, April 12 – 16, 2021

Source:	vivo

Title:	Introducing UE behaviour for Edge Computing in SSC mode 2 and 3

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEdge_5GC /Rel-17

Abstract of the contribution: This contribution introduces a new UE behaviour for Edge Computing in SSC mode 2 and SSC mode 3

1	Introduction

As per TR 23.748 clause 9.2.2 " Conclusions for Key Issue #2: UE based EAS rediscovery ", it introduces new UE behaviour to support edge computing characteristic. The UE can remove the old EAS information associated with the released PDU Session and can reselect a new EAS after the UE receives a new IP address for SSC mode 2 and SSC mode 3.

This new UE behaviour in edge relocation should be reflected according to the summarised conclusions in TR 23.748 clause 9.2.2.

2	Proposal

It is proposed to adopt the following changes into TS 23.548.

********** Start Changes(All new text) **********

6.2.2.3	EAS Re-discovery Procedure at Edge Relocation

In order to change the PDU Session Anchor serving a PDU Session of SSC mode 2/3 for a UE, SMF triggers session continuity, service continuity and UP path management procedures as indicated in clause 4.3.5.1 and 4.3.5.2 of TS 23.502[3].

During this procedure, for SSC mode 2/3, the UE may apply following behaviour:

When the UE detects the PDU Session release or IP address changes, the UE removes the old DNS cache related to released PDU session, for example, the old Edge Application Server address information. And UE reselects a new EAS after the UE receives a new IP address.

*************** End Changes ***************