3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102753

Elbonia, April 12 – April 16, 2021	(revision of S2-210xxxx)

Source:	China Mobile, Huawei

Title:	TS 23.256 UUAA Revocation procedure by USS

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract: This contribution introduces a new procedure on UUAA Revocation by USS.

1. Introduction/Discussion

Procedure on UUAA Revocation by USS is not documented in the TS 23.256.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.256. All texts are new.

# * * * * First change * * * *

5.2.7	UUAA Revocation by USS

1. Nnef_Authentication_revoke

4. Nnef_Authentication_revoke

5. Nnef_Authentication_revoke

USS

NEF

SMF

AMF

UE (UAV)

1. Determine the UUAA is revoked

6. De-registration procedure of UAV

7. PDU session release procedure

3. Determine SUPI of UAV

Figure 5.2.7-1: Procedure for UUAA Revocation by USS in 5GS

1.	USS determines that UUAA is to be revoked, due to the reason e.g. UAV registration at USS is withdrawn.

2.	USS sends an Nnef_Authentication_revoke message to the NEF, including the External ID of the UAV, IP address(es) of the UAV (optional), and a Revocation Cause.

3.	NEF translates External ID of the UAV to SUPI of UAV, and determines the AMF serving the UAV by invoking the Nudm_UECM_Get service operation. Subsequently, NEF executes step 4a.

If IP address(es) of the UAV is provided in step 2, NEF determines the SMF serving the PDU sessions of the UAV, by invoking a Nbsf_service operation. Subsequently, NEF executes step 4b.

4.	NEF sends an Nnef_Authentication_revoke message to the AMF, including the SUPI of the UAV, and a Revocation Cause.

5. NEF sends an Nnef_Authentication_revoke message to the SMF, including the SUPI of the UAV, IP address(es) of the UAV and a Revocation Cause. NEF may send the message to one or more SMF, if the UAV PDU sessions are served by different SMF.

6.	Once receiving the message in step 4a, AMF may determine to deregister the UE, based on the operator’s policy. AMF triggers the Deregistration procedure as described in clause 4.2.2.3.3 in TS 23.502[x].

7.	Once receiving the message in step 4b, SMF shall release the PDU sessions of the UAV, identified by the UAV IP address. The procedures are reuses PDU Session Release procedure described in clause 4.3.4.2, TS 23.502[X].

Editor’s note: Procedure for EPS is FFS.

# * * * * End of changes * * * *