SA WG2 Meeting #S2-144E	S2-210xxxx

12 April - 16 April 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2702 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on EPS beaer can be handed over from EUTRAN to GERAN/UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the RAN3 LS (S2-2102093), it states: “when perform SRVCC from 4G to 3G, if the UE was earlier handed over from 5G and having the PS bearer (no voice) from 5G, eNB would perform SRVCC with two Iu connections (Iu-CS and Iu-PS) and informs the target RNC. But the Forward Relocation Request message may never be sent to the target node due to the QoS flow established in 5G does not contain Transaction Identifier (TI) and this TI is mandatory in the Forward Relocation Request message. This causes the SRVCC delay and in the worst case could even cause failure.” To avoid the aboved issue, it proposes the MME may send the indicaiton to eNB in the E-RAB eatablishement related procedure, to indicate whether this bearer can be handed over to GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause, During E-RAB setup related procedure, the MME may send the indication in S1AP whether the bearer can be handed over to GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause the SRVCC failure, if the RNC can not receives the Forward relocation for the PS HO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.110a.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.11.0a	Impacts to EPS Procedures

#### 4.11.0a.X	E-RAB setup

During E-RAB setup related procedure, the MME may send the indication in S1AP whether the bearer can be handed over to GERAN/UTRAN according to whether the PDN connection supporting 5GS interworking and operator’s policy.

END OF CHANGES