SA WG2 Meeting SA2#144-e	S2-2102858

12 - 16 April, 2021, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2837 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Emergency and Priority Services in Network Slice Admission Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, CISA ECD, Verizon, AT&T, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | April 06 2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | TS 23.700-40 includes the following architectural requirement: “Subject to operator policy and national/regional regulations, solutions shall be able to exempt Emergency services, Multimedia Priority Service, and Mission Critical Service from quota limits. NOTE: Handling of UE/PDU Session quota exemption and any pre-emption to other non-mission critical services is expected to be controlled by operator policy.” The newly specified Network Slice Admission Control Function in Rel-17 TS 23.501 does not indicate this capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NSACF to support exemption of emergency and priority services in clause 5.15.11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | During congestion, priority users might not have access to the system. Similarly, an emergency call can’t be established when the slice has reached its max number of UEs or PDU sessions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST and ONLY CHANGE

### 5.15.11	Network Slice Admission Control Function

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice for the network slices that are subject to Network Slice Admission Control (NSAC).

The NSACF is configured with the maximum number of UEs per network slice which are allowed to be served by each network slice that is subject to NSAC.

Subject to operator policy and national/regional regulations, Emergency and Priority services (e.g., MPS) shall be exempted from NSAC.

END OF CHANGES