SA WG2 Meeting #144E e-meeting	S2-2102882

April 12 – April 16, 2021, Elbonia

Source:	Qualcomm Incorporated

Title:	TS 23.304: Clause 6.3.2.3 UE-to-Network Relay Discovery

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution adds the details on UE-to-Network Relay Discovery into TS 23.304.

# 1.	Introduction

Based on the KI#3 conclusion of TR 23.752, this contribution added the details on UE-to-Network Relay Discovery into TS 23.304 as clause 6.3.2.3.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

### 6.3.2.3	UE-to-Network Relay Discovery

6.3.2.3.1	General

UE-to-Network Relay Discovery is applicable to both Layer 3 and Layer 2 UE-to-Network relay discovery for public safety use and commercial services. To perform UE-to-Network Relay Discovery, the Remote UE and the UE-to-Network Relay are pre-configured or provisioned with the related information as described in clause 5.1.

The UE-to-Network Relay discovery procedure is of restricted type, in that only Remote UEs with valid credentials and pre-configurations are able to successfully complete the discovery procedure. In UE-to-Network Relay Discovery, the ProSe Restricted Code is not used, and UEs use pre-configured or provisioned information for the discovery procedures as defined in clause 5.1.2.1 and clause 5.1.4.1.

The Relay Service Code (RSC) is used in the UE-to-Network Relay discovery, to indicate the connectivity service the UE-to-Network Relay provides to the Remote UE. The RSCs are configured on the UE-to-Network Relay and the Remote UE as defined in clause 5.1.4. The RSC can also indicate if the UE-to-Network Relay is a Layer 3 or Layer 2 UE-to-Network Relay.

Additional information used for the UE-to-Network Relay (re)selection and connection maintenance can be advertised using a separate discovery messages of type "Relay Discovery Additional Information". This may include for example the related system information of the UE-to-Network Relay's serving cell, as defined in TS 38.300 [x].

Both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

For Relay Discovery Additional Information, only Model A discovery is used.

6.3.2.3.2	Procedure for UE-to-Network Relay Discovery with Model A

Depicted in Figure 6.3.2.3.2-1 is the procedure for UE-to-Network Discovery with Model A.

Figure 6.3.2.3.2-1: UE-to-Network Relay Discovery with Model A

1. The UE-to-Network Relay sends a UE-to-Network Relay Discovery Announcement message. The parameters contained in this message are described in clause 5.9.3.

Remote UE (1 to 3) monitors announcement messages with the UE-to-Network RSC corresponding to the desired services.

The ProSe UE-to-Network Relay may also send Relay Discovery Additional Information messages. The parameters contained in this message are described in clause 5.9.3.

6.3.2.3.3	Procedure for UE-to-Network Relay Discovery with Model B

Depicted in Figure 6.3.2.3.3-1 is the procedure for UE-to-Network Relay Discovery with Model B.

Figure 6.3.2.3.3-1: UE-to-Network Relay Discovery with Model B

1. The Remote UE sends a UE-to-Network Relay Discovery Solicitation message. The parameters contained in this message are described in clause 5.9.3. The Remote UE discovering a UE-to-Network Relay sends a solicitation message with the UE-to-Network RSC which is associated to the desired connectivity service.

2. The UE-to-Network Relays (1 to 3) that match the values of the RSC contained in the solicitation message respond to the Remote UE with a UE-to-Network Relay Discovery Response message. The parameters contained in this message are described in clause 5.9.3.

The Remote UE selects the UE-to-Network Relay based on the information received in step 2.

# * * * * End of changes * * * *