3GPP TSG-WG SA2#144E Meeting emeeting 	S2-2102899

# emeeting, Apr 12-Apr 16, 2021	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2719 | rev |  | Current version: | h.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE initiated PDN connection modification after 5GS to EPS handover with MA PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When MA PDU session is handed over from 5GC to EPC, the SMF should decide whether to release the user plane resource in non-3GPP access. If both the UE and SMF supports MA PDU Session with 3GPP access connected to EPC, the user plane resource in non 3GPP access should be kept, otherwise it should be released. The issue is when the UE is handed over to EPC, the SMF cannot know if the UE support this feature or not, therefore the SMF cannot make decision whether to keep or release the user plane resource in non 3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that when the UE is handover from 5GC to EPC, the UE may trigger the UE initiated PDN connection modification procedure, to provide an indication in PCF that the PDN Connection is requested to be associated with a MA PDU Session. Based on this indication, the SMF then can determine how to handle the user plane resource in non 3GPP access. This can be applied to both IDLE mobility and CONNECTED mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the SMF handle the user plane resource in non 3GPP access is ambigouse. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.6.2.1,4.22.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.22.6.2.1	5GS to EPS handover using N26 interface

Based on the signalling flow in Figure 4.11.1.2.1-1, the procedure is performed with the following differences and modifications:

-	Step 2 is also performed with all the SMF+PGW-Cs corresponding to MA PDU Sessions with allocated EBI(s).

-	In step 12e, the AMF requests the release of the 3GPP access of the MA PDU Session which has resources established for 3GPP access, but not expected to be transferred to EPC, i.e. no EBI(s) allocated to the MA PDU Session by triggering Nsmf_PDUSession_UpdateSMContext service operation.

NOTE:	When the SMF received the release request from the AMF, the SMF decides whether the MA PDU Session is completely released or released over a single access based on its local policy.

-	In step 16, if the MA PDU Session is established in both 3GPP and non-3GPP accesses and the MA PDU Session is moved to EPS and if the network does not supports MA PDU Session with 3GPP access connected to EPC, the SMF triggers the MA PDU Session Release procedure over non-3GPP access. Otherwise the SMF start a timer. After the handover the UE may trigger UE initiated PDN connection modification procedure to provide indication in PCO that the PDN Connection is requested to be associated with a MA PDU Session. After receiving this indication the SMF should stop the timer and keep the user-plane resources over non-3GPP access in 5GC and use the PDN Connection as the 3GPP access leg of the MA PDU Session. After the timer expires the SMF triggers the MA PDU Session Release procedure over non-3GPP access.

4.22.6.2.2	5GS to EPS idle mode mobility using N26 interface

Based on the signalling flow in Figure 4.11.1.3.2-1, the procedure is performed with the following differences and modifications:

-	Step 5a is also performed with all the SMF+PGW-Cs corresponding to the MA PDU Sessions with allocated EBI(s).

-	In step 12, if the MA PDU Session is established in both 3GPP and non-3GPP accesses and the MA PDU Session is moved to EPS and if the network does not support MA PDU Session with 3GPP access connected to EPC, the SMF triggers the MA PDU Session Release procedure over non-3GPP access. Otherwise the SMF start a timer. After the handover the UE may trigger UE initiated PDN connection modification procedure to provide indication in PCO that the PDN Connection is requested to be associated with a MA PDU Session. After receiving this indication the SMF should stop the timer and keep the user-plane resources over non-3GPP access in 5GC and use the PDN Connection as the 3GPP access leg of the MA PDU Session. After the timer expires the SMF triggers the MA PDU Session Release procedure over non-3GPP access.

-	In step 15a, the AMF also requests the release of the MA PDU Session which has resources established for 3GPP access, but not expected to be transferred to EPS, i.e. no EBI(s) allocated to the MA PDU Session by triggering Nsmf_PDUSession_UpdateSMContext service operation.

NOTE:	When the SMF received the release request from the AMF, the SMF decides whether the MA PDU Session is completely released or released over a single access based on its local policy.

# * * * * end of change * * * *