3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102903

Elbonia, Apr. 12-16, 2021

Source:	ZTE

Title:	KI#1 EAS Rediscovery Procedure

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract of the contribution: This contribution proposes update on the EAS rediscovery Procedure in TS 23.548.

- Discussion

- Proposal

It is proposed to update the TS 23.548 as follows.

# * * * * First change * * * *

6.2.3.3	EAS Re-discovery Procedure at Edge Relocation

Editor's note:	This clause also describes rediscovery (UE based), and aspects and assumptions based on applicable clause 9.2.2 in the TR.

For PDU Session with Session Breakout connectivity, the UE may need to re-discover the EAS after the insertion/change/removal of an L-PSA.

This procedure is used by the SMF to trigger the EAS rediscovery procedure when a new connection to EAS need to be established. It applies to both Session Breakout using ULCL and Session Breakout using BP.

Editor's note:	It is FFS whether the same procedure is needed for change of SSC mode 3 PSA with IPv6 multi-homing scenario. For this scenario, the UE's IPv6 prefix can be released with the existing procedure, therefore, the UE is aware when to release DNS cache if needed.

Figure 6.2.3.3-1: EAS re-discovery procedure at Edge relocation

During a previous EAS Discovery procedure on this PDU Session the UE may have cached EAS information (i.e. EAS IP address corresponding to an EAS FQDN) locally, e.g. during the previous connection with the EAS (for more information see Annex C UE considerations for EAS (re)discovery).

1.	Due to the UE mobility the SMF triggers L-PSA insertion, change or removal for the PDU Session, the UP management event notifying to the AF may trigger the EAS relocation.

1a. The AF triggers EAS relocation e.g. due to EAS load balance or maintenance, etc, and informs the SMF the related information.

2.	The SMF executes the network requested PDU Session Modification procedure from the step 3b-11b as defined in clause 4.3.3.2 TS 23.502 [3]. The SMF makes the decision to indicate the EAS rediscovery to the UE based on information provided by AF or based on SMF's local configuration.

Editor's note:	It is FFS for the detail information provided by the AF.

The SMF sends PDU Session Modification Command (EAS rediscovery indication, [impact field]) to UE. The impact field is used to identify which EAS(s) need to be rediscovered. If the impact field is not included, it means all EAS(s) associated with this PDU Session need to be rediscovered. The SMF may chooses a new local DNS server or EASDF, and configures it to the UE as new DNS server (Option C in clause 6.2.3.2.3) . Otherwise the UE uses old DNS server for EAS rediscovery.

Editor's note:	Which information that is used by SMF to compose the optional impact field and how that is provisioned to SMF is FFS.

For the following connection with the EAS(s) for which the EAS rediscovery need be executed per the received EAS rediscovery indication and impact field, the UE does not use the old EAS information stored locally. Instead it triggers EAS discovery procedure to get new EAS information as defined in clause 6.2.3.2.

NOTE:	The active connection(s) between the UE and the EAS(s) are not impacted.

Editor's note:	Whether interaction between the 5GS and application layer DNS cache for EAS rediscovery is required or not is FFS.

# * * * * end of change * * * *