3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102905

Elbonia, Apr. 12-16, 2021

Source:	ZTE

Title:	KI#3 Network Exposure to Edge Application Server via Local NEF

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract of the contribution: This contribution proposes update on the Network Exposure to Edge Application Server via Local NEF in TS 23.548.

- Discussion

It should be clarified that the real time notification is per service data flow.

It should be clarified that the local UPF notifies the local NEF after the threshold is reached, i.e. no need to notify the local NEF when the threshold is not reached so as to reduce the number of notification messages.

In order to address the duplicated notification from both PCF and UPF, it is proposed that the L-UPF sends initiate notification to the local NEF when it receives N4 rules to activate the QoS monitoring for the service data flow. The local NEF then disable the notification from PCF.

- Proposal

It is proposed to update the TS 23.548 as follows.

# * * * * First change * * * *

6.4.2	Network Exposure to Edge Application Server via Local NEF

Editor's note:	This clause describes network exposure via local NEF as described in bullets in TR 23.748 clause 9.4.

Local NEF deployed at the edge may be used to support network exposure timely to local AF. The local NEF may support one or more of the functionalities described in TS 23.501 [2] clause 6.2.5.0. and may support a subset of the APIs specified for capability exposure based on local policy. In order to support the network exposure locally, the local NEF shall support event exposure service operation to the local AF. The local NEF selection by AF is described in TS 23.501 [2] clause 6.2.5.0 and 6.3.14.

The local AF subscribes the low latency exposure of QoS Monitoring results to PCF via a local NEF or NEF. The local AF may also subscribe the events via PCF directly. Based on the indication of local event notification and operator's policy, the PCF may include an indication of local event notification (including target local NEF address) within the PCC rule that it provides to the SMF.

The SMF sends the QoS monitoring request to the RAN and N4 rules to the L-PSA UPF. N4 rules may indicate the service data flow needs local notification of QoS Monitoring. When GTP-U Path monitoring is used for QoS monitoring, that is also activated if needed. This is as defined in TS 23.501 [2] clauses 5.33.3. When N4 rules indicate the service data flow needs local notification of QoS Monitoring, upon the detection of the QoS monitoring event(e.g. when latency threshold of the QoS flow is reached), the L-PSA UPF sends the notification to the AF via Local NEF.

During UE mobility, the SMF may trigger the L-PSA UPF relocation/reselection and then send the N4 rules to the new L-PSA UPF to indicate the service data flow needs local notification of QoS Monitoring. The UE mobility may also trigger AF relocation or local NEF reselection, then the local AF should update the subscription for local exposure with QoS monitoring results via local NEF, towards the PCF. This updated /new subscription is then propagated via SMF (via PCC rule updates) and then to the L-PSA UPF via N4 rules.

Figure 6.46.2-1: The association establishment between local UPF and local NEF

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3] A Local PSA is used by this PDU Session.

1.	The AF initiates an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

In the request, the AF may subscribe local notification of QoS monitoring for the service data flow to PCF via Local NEF or NEF. For the QoS monitoring, the AF shall include the corresponding QoS monitoring parameters for the service data flow as defined in clause 5.33.3 of TS 23.501 [2].

The local AF may find a local NEF based on existing method as in TS 23.501 [2] clause 6.2.5.0 and parameters in clause 6.3.14.

Editor's note:	The local NEF discovery is still FFS.

The AF may also first initiate an AF Session with PCF and later subscribe to local notification of QoS monitoring to PCF by invoking Npcf_PolicyAuthorization_Subscribe service operation.

2.	The PCF makes the policy decision and initiates the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3], step 1b, 3b, 4-8b.	If the local notification of QoS monitoring is subscribed, the PCF includes the indication of local event notification (including target local NEF address) of the service data flow within the PCC rule. The SMF sends local QoS monitoring parameters of the service data flow to the L-PSA UPF via N4 rules.

3.	The L-PSA UPF obtains QoS monitoring information as defined in TS 23.501 [2] clause 5.33.3.

4.	If the L-UPF supports this feature, it sends the initial notification related with QoS monitoring information of the service data flow over Nx.

Editor's note:	It is FFS whether the QoS monitoring result report from L-PSA UPF to local NEF is based on SBI or not.

Later on when latency threshold of the QoS flow is reached, the L-UPF sends the notification related with QoS monitoring information of the service data flow over Nx.

NOTE x:	The notification towards SMF and PCF may still be needed for other service data flows.

5.	When the local NEF receives initial notification from the L-UPF, it disables the QoS monitoring notification from PCF by invoking Npcf_PolicyAuthorization_Subscribe service operation.

6.	Local NEF reports the real-time network information to local AF through Nnef_EventExposure_Notify.

7.	Due to e.g. UE mobility, the PSA relocation and/or EAS relocation may happen as described in clause 6.3.

8.	The new AF may initiate a new AF session to the new Local NEF or NEF/PCF to (re-)subscribe the local notification of QoS monitoring as described in steps 2-4.

9.	The old AF revokes the AF session with the old local NEF or NEF/PCF.

# * * * * end of change * * * *