# 3GPP TSG-SA WG2 Meeting #144E	S2-2102911

12 - 16 April, 2021, Electronic meeting

Title:	LS Response on N3IWF FQDN for emergency service

Response to:	S2-2102109 / C4-211521

Release:	Release-17

Work Item:	5GS_Ph1,TEI17

Source:	SA2

To:	CT4

Cc:	CT1

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu (dot) jinguo (at) zte (dot) com (dot) cn

Attachment: 23.501CR2844

1. Overall Description:

SA2 thanks CT4 for the LS on N3IWF FQDN for emergency service.

CT4 raised following questions:

"Question 1): Which one does the highlighted "Operator Identifier Emergency FQDN" and "Visited Country Emergency FQDN" refer to?

a) emergency ePDG FQDN

b) either emergency ePDG FQDN or emergency N3IWF FQDN

Question 2): If the answer to Q1) is "b", why does emergency N3IWF FQDN need to be defined?"

SA2 would like to provide following answers:

The UE performs PLMN selection for emergency service first and then performs the N3IWF and ePDG selection based on the Non-3GPP Access Node Selection Information for the selected PLMN.

When the UE is in home country, it shall always select the HPLMN for emergency service, therefore the Operator Identifier Emergency FQDN is not used for PLMN selection. When the UE is in visited country, the UE shall use a specific FQDN to determine which PLMNs in the visited country support emergency services in non-3GPP access. This FQDN doesn’t refer to ePDG FQDN or N3IWF FQDN. SA2 has agreed to use new name "Visited Country non-3GPP Emergency FQDN" for this purpose.

SA2 agreed that the emergency N3IWF FQDN is not needed. SA2 has agreed the attached CR to capture the answer above.

2. Actions:

To CT WG4 group.

ACTION: 	SA2 kindly asks CT4 to take the answer above into account.

3. Date of Next SA2 Meetings:

3GPP SA2#145e	17th – 28th May 2021	e-meeting