# SA WG2 Meeting #144E	S2-2102918

# 12 – 16 April 2021, Electronic meeting

Source:	vivo Mobile Communications Ltd

Title:	TS 23.304: Functional Entities: Section 4.3.1 – UE – Add relay aspects

Agenda item:	8.8

Document for:	Approval

1. Introduction

SA/RAN#91E has decided what aspects of ProSe Relay should be in Rel-17.  UE-to-Network L2 and L3 would be specified in the normative phase.  SA2 was allowed to still study if UE-to-UE could be done for L3 without any RAN involvement.  This PCR captures the first conclusion and removes the editors note associated with this aspect.

**** Start of Changes****

### 4.3.1	UE

Any ProSe-enabled UE may support the following functions:

-	Exchange of ProSe control information between ProSe-enabled UE and the 5G DDNMF over PC3 reference point.

-	Procedures for ProSe Direct Discovery of other ProSe-enabled UEs over PC5 reference point.

-	Procedures for ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode ProSe Direct Communication.

-	Indicate 5G ProSe Policy Provisioning Request in UE Policy Container for UE triggered 5G ProSe Policy provisioning.

-	Receive the 5G ProSe Policy from 5GC over N1 reference point.

-	Configuration of parameters for ProSe Direct Discovery and ProSe Direct Communication, UE-to-Network relay (e.g., including IP addresses, ProSe Layer-2 Group IDs, see clause 5.1). These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

Editor's Note:	Which subset of parameters can be provisioned over PC1 is FFS.

-	Report the following capabilities to 5GC over the N1 reference point:

-	PC5 Capability;

-	5G ProSe capability as Remote UE;

-	Procedures to act as a ProSe UE-to-Network Relay. The Remote UE communicates with the ProSe UE-to-Network Relay over PC5 reference point. The ProSe UE-to-Network Relay will relay the control information over NR-Uu interface towards 5G-DDNMF when received from a Remote UE via PC5 and vice versa.