3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102976

# Elbonia, April 12 – 16, 2021	(revision of S2-2102416)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2755 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | De-registration for onboarding registered UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the TR 23700-07, On-boarding network shall restrict the usage only for on-boarding service, and the on-boarding UE should be de-registrated after finishing the remote provisioning. When the remote provisioning for SNPN credentials is finished the UE or the ON-SNPN shall initiate de-registration from the ON-SNPN. AMF shall initiate the de-registration when the expiration of the on-boarding timer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the TR conclusion, add the description that the on-boarding network shall restrict the usage only for on-boarding service, and the on-boarding UE should be de-registrated after finishing the remote provisioning. AMF shall initiate the de-registration when the expiration of the on-boarding timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing new features as per conclusion of TR 23.700.07 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X.2.7 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

5.30.2.X.2.7	Deregistration from the ON-SNPN for onboarding registered UE

Once remote provisioning of SO-SNPN credentials is completed, the UE should initiate de-registration from the ON-SNPN.

NOTE: 	Based on ON-SNPN policies, the AMF can start an implementation specific timer when the UE has registered to the ON-SNPN for the purpose of onboarding. Expiry of this implementation specific timer triggers the AMF to de-register the onboarding registered UE from the ON-SNPN.

***** End of changes *****