3GPP TSG-SA2 Meeting #144-e	S2-2103276

# Online, Elbonia, 12th Apr 2021 - 16th Apr 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0273 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Preferred granularity of location in analytics outputs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR implements some conclusions of TR 23.700-91 for KI#11 related to the granularity of analytics. The UE location is provided as "TA or cells which the UE stays" but without control by the NWDAF consumer. The required level of granularity depends on the use case (handover, paging, application-related use case); therefore the NWDAF consumer should be able to specify whether the UE location is required per TA or per cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an optional parameter in the request or subscription of analytics “UE mobility”, “Service Experience”, “Dispersion analytics” which allows to define the preferred granularity of location information in results (TA level or cell level). Fix a hanging paragraph. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Higher signalling and computation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.3, 6.7.2.1, 6.7.2.3, 6.10.1, 6.10.3.0 (new), 6.10.3.1, 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming as well as services that are not audio-visual streaming such as V2X and Web Browsing services) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for a Network Slice: Service Experience for a UE or a group of UEs or any UE in a Network Slice;

-	Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications.

-	Service Experience for an Edge Application over a UP path: Service experience for a UE or a group UEs or any UE in an Application or a set of Applications over a specific UP path (UPF, DNAI and EC server).

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF, NSSF, AMF), AF, or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1;

-	optionally, maximum number of objects and maximum number of SUPIs;

-	optionally, preferred level of accuracy of the analytics;

-	optionally, accuracy level per analytics subset (Network Slice instance service experience, Application service Experience);

-	optionally, preferred order of results for the list of Application Service Experiences and/or Slice instance service experiences: "ascending" or "descending"; and

-	optionally, preferred granularity of location information: TA level or cell level.

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description | Mandatory |  |  |
| --- | --- | --- | --- | --- |
|  |  | Application | Network Slice | Edge Applications over a UP path |
| Application ID (0...max) | The identification of the application(s) for which the analytics information is subscribed or requested. | Y | N | Y |
| S-NSSAI | Identifies the Network Slice for which analytics information is subscribed or requested. | N | Y | N |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. | N | N | N |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. | N | N | N |
| Media/application bandwidth | Identifies the Media/application bandwidth requirement of the application. | N | N | N |
| DNN | DNN to access the application. | N | N | N |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. | N | N | Y |
| RAT Type | Identifies the RAT type. | N | N | N |
| Frequency | Identifies the frequency. | N | N | N |
| Edge Application Server (NOTE 1) | IP address/FQDN of the Application Server that a UE has a communication session | N | N | Y |
| UPF anchor ID (NOTE 1) | Identifies the UPF where a UE has an associated PDU session | N | N | N |
| NOTE 1: These parameters may be provided when a consumer requires analytics for an edge application over a UP path |  |  |  |  |

NOTE:	A service consumer may use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

Based on the Analytics Filter information in Table 6.4.1-1 and the Target of Analytics reporting provided by the service consumer in the analytics subscription or request, NWDAF determines whether service experience analytics should be delivered for:

i)	Application(s);

ii)	Network Slice;

iii)	both Application(s) and Network Slice;

iv)	Edge Applications over a UP path.

If NWDAF is unable to differentiate based on the analytics subscription or request, it provides service experience analytics for both Application(s) and Network Slice.

If service experience for both Application(s) and Network Slice is desired but the Target of Analytics reporting or Analytics Filter information values (e.g. Area of Interest) need to be different, separate subscriptions/requests may be provided by the service consumer.

# * * * * Next change

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Slice instance service experiences (0…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice of the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE X). |
| >UPF Info | Indicating UPF serving the UE. |
| >DNAI | Indicating which DNAI the UE service uses/camps on. |
| >DNN | DNN for the PDU Session which contains the QoS flow. |
| >Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Network Slice instance service experiences (0…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice of the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE X). |
| >UPF Info | Indicating UPF serving the UE. |
| >DNAI | Indicating which DNAI the UE service uses/camps on. |
| >DNN | DNN for the PDU Session which contains the QoS flow. |
| >Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |

NOTE 1:	If multiple Network Slice instances are not deployed for the S-NSSAI or NSI IDs are not available, only one Network Slice instance service experience entry is provided. In that case, the NSI ID is not provided and the Network Slice instance service experience indicates the service experience for the S-NSSAI.

NOTE 2:	The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and with the same value(s) in the service experience information for the slice instance which the application belongs to.

NOTE 3:	If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF.

NOTE X:	When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information.

Editor's note:	How to conclude on extend the output with RAT type and frequency is FFS.

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

# * * * * Next change

#### 6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM, and to perform data analytics to provide UE mobility statistics or predictions.

The service consumer may be a NF (e.g. AMF, SMF).

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs;

-	Analytics Filter Information optionally containing:

-	Area of Interest;

NOTE:	For LADN service, the consumer (e.g. SMF) provides the LADN DNN to refer the LADN service area as the Area Of Interest.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, maximum number of objects;

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the time slot entries: ascending or descending time slot start;

-	Optionally, preferred granularity of location information: TA level or cell level; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

# * * * * Next change

#### 6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics |
| >> UE location | TA or cells which the UE stays (see NOTE X). |
| >> Ratio | Percentage of UEs in the group (in the case of an UE group) |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or SUPI (see NOTE). |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location prediction during the Analytics target period |
| >> UE location | TA or cells where the UE or UE group may move into (see NOTE X). |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in the case of an UE group) |

NOTE:	When target of analytics reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

NOTE X:	When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information.

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of objects provided as part of Analytics Reporting Information

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

# * * * * Next change

### 6.10.1	General

Dispersion analytics identifies the location (i.e. areas of interest, TAs, cells) or network slice(s) where a UE, or a group of UEs disperse most (if not all) of their data volume and sessions transactions (i.e. MM and SM messages).

The NWDAF shall be able to provide dispersion statistics or predictions and shall be able to collect UE dispersion related information from NFs.

Dispersion is the percentage of activity that a UE, or group of UEs, generates at a location or a slice during a period of interest. This enables the operator to rank hot locations or slices by the various activities and to identify the top contributors (i.e. UEs) for that activity. When heavy UEs at a particular location or slice are detected/predicted and reported to a policy entity, their QoS attributes, or access to an area of interest or a slice can then be limited.

-	Data dispersion - The percentage of data traffic volume that a UE, or a group of UEs, generated at a location or in a slice during the period of interest.

-	Transaction dispersion - The percentage of MM and SM messages that a UE, or a group of UEs, generated at a location or in a slice during the period of interest.

The operator may classify and assign one of the three mobility classes per dispersion characteristics for a UE or a group of UEs as fixed, camper or traveller. The classification is based on either Data dispersion or Transaction dispersion.

-	Data-Classification - fixed, camper, traveller data-classification per thresholds assigned by the operator.

EXAMPLE:	When a UE disperses, during the period of observation, most (threshold=95%) of its data at a location or a slice, the dispersion data-classification of the UE at that location or slice is "fixed".

-	Transaction-Classification - fixed, camper, traveller transactions-classifications per thresholds assigned by the operator. Top-Heavy UE is one who's dispersion percentile rating, at a location or a slice, is higher than a threshold value defined by the operator (e.g. 0.9 percentile), see percentile ranking in the output tables, clause 6.10.3.

EXAMPLE:	When a UE disperses, during the period of observation, (threshold=40%) of its session transactions at a location or a slice, the dispersion transaction-classification of the UE is camper at that location or slice.

Based on the dispersion analytics, the analytics consumer can determine that a data or transaction hot spot is formed when the amount of data or amount of transactions dispersed by most of the UEs at the area of interest or slice exceeds a certain data or transaction threshold established through trend analysis or operator policy.

The consumer of dispersion analytics may indicate in its request:

-	Analytics ID set to "UE Dispersion Analytics";

-	Dispersion Analytics type set to Data Dispersion Analytics (DDA) or Transaction Dispersion Analytics (TDA);

-	Target of analytics reporting: single UE, any UE, or a group of UEs;

-	Analytics Filter Information: optional list of TA(s), Area(s) of Interest, Cells, or S-NSSAI, Top-Heavy UEs, Fixed UEs, Camper UEs;

-	Analytics target period indicating the time period over which the statistics or predictions are requested;

-	Optionally, preferred granularity of location information: TA level or cell level; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

NOTE:	Care must be taken with regards to load when requesting analytics for "Any UE". This could be achieved by utilizing event filters (e.g. Area of Interest for AMF) and possible analytics filters including Top-Heavy UEs, and/or fixed and/or camping UEs and/or Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

# * * * * Next change

### 6.10.3	Output Analytics

#### 6.10.3.0	General

The NWDAF provides UE dispersion analytics, which can be statistics and/or prediction for data and/or transaction dispersions, to consumer NFs or AFs.

#### 6.10.3.1	Data Dispersion Analysis

The data dispersion analytics results provided by the NWDAF can be UE data dispersion statistics as defined in table 6.10.3.1-1 and/or UE data dispersion predictions as defined in table 6.10.3.1-2.

Table 6.10.3.1-1: Data dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE dispersed its data (see NOTE x). |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data dispersed | Data volume dispersed at this location and application, if application ID is provided. |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >> Data ranking | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group at the location and observation time. (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. |  |

Table 6.10.3.1-2: Data dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytics target period. |
| >> UE location | TA or cells where the UE is predicted to disperse its data (see NOTE x). |
| >> Application ID (1..max) | To identify the application for the UE. |
| >>> Data dispersion | Data volume dispersion prediction at this location and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location). |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Data ranking | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. |  |

The data dispersion analytics results provided by the NWDAF can also be for a UE or group of UEs data dispersion statistics at a given slice as defined in Table 6.10.3.1-3 and/or dispersion predictions as defined in Table 6.10.3.1-4:

Table 6.10.3.1-3: Data dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data dispersed | Data volume dispersed at this slice and application, if application ID is provided. |
| >> Data classification | Data mobility classification for slice: fixed, camper, traveller. |
| >> Data ranking | Ranking of UE data usage at this slice. (See NOTE 2). |
| >> Data percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

Editor note:	Consider the addition of ">>Bandwidth" into tables 6.10.3.1-3 and 6.10.3.1-4, which is the utilized bandwidth at the slice calculated by the NWDAF as the volume over the period. See also SA WG5 Liaison letter S2-2008361 (S5-205436).

Table 6.10.3.1-4: Data dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its data. |
| >> Application ID (1..max) | To identify the application for the UE at the slice. |
| >>> Data dispersion | Data volume dispersion prediction at this slice and application, if application ID is provided. |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this slice). |
| >> Data classification | Data mobility classification for this slice: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Data ranking | Ranking of UE data usage at this slice. See NOTE 2. |
| >> Data percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

NOTE:	The Application ID in the Data Dispersion Analytics is optional. When the Application ID is missing, the Data Dispersion Analytics is applied across all the applications in an AoI or a slice.

#### 6.10.3.2	Transactions Dispersion Analysis

The transaction (MM and MS messages) dispersion analytics results provided by the NWDAF can be UE transaction dispersion statistics as defined in Table 6.10.3.2-1 and/or UE transaction dispersion predications as defined in Table 6.10.3.2-2:

Table 6.10.3.2-1: Transaction dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE dispersed its transactions (see NOTE x). |
| >> Transaction dispersed | Transactions amount dispersed at this location. |
| >> Transaction classification | Transactions mobility classification for this location: fixed, camper, traveller. |
| >> Transaction ranking | Ranking of transactions amount at this location. See NOTE 2. |
| >> Transaction percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. |  |

Table 6.10.3.2-2: Transaction dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of predicted time slots. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytic target period. |
| >> UE location | TA or cells where the UE is predicted to disperse its transactions (see NOTE x). |
| >> Transaction dispersion | Transaction amount predicted to be dispersed at this location. |
| >>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location). |
| >> Transaction classification | Transaction mobility classification for this location: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Transaction ranking | Ranking of transactions amount at this location. See NOTE 2. |
| >> Transaction percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE X: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. |  |

The transaction dispersion analytics results provided by the NWDAF can be UE or group of UEs transaction dispersion statistics at a given slice as defined in table 6.10.3.2-3 and/or dispersion predictions as defined in table 6.10.1.4.2-4:

Table 6.10.3.2-3: Transaction dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed its transactions. |
| >> Transaction dispersed | Transactions amount dispersed at this location. |
| >> Transaction classification | Transaction mobility classification for slice: fixed, camper, traveller. |
| >> Transaction ranking | Ranking of transactions amount at this slice. See NOTE 2. |
| >> Transaction percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

Table 6.10.3.2-4: Transaction dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE1). |
| Time slot entry (1..max) | List of predicted time slots. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its transactions. |
| >> Transaction dispersion | Transaction volume to be dispersed at this slice. |
| >>> Confidence | Confidence of this prediction (i.e. transactions to be dispersed at this slice). |
| >> Transaction classification | Transaction mobility classification for this slice: fixed, camper, traveller. |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Transaction ranking | Ranking of transactions amount at this slice. See NOTE2. |
| >> Transaction percentile ranking | Percentile ranking of the UE in the Cumulative Distribution Function of transactions for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When target of analytics reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. |  |

# * * * * End of change