3GPP TSG-SA WG2 Meeting #143E	 S2-2103301

# Elbonia, April 12 - 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2744 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Thresholds for Priority-based mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the TR conclusion, the threshold values can be applied by the Priority-based steering mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the threshold values to the MAR rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The threshold values cannot be supported by the Priority-based mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.2.11.8	Multi-Access Rule

The following table describes the Multi-Access Rule (MAR) that includes the association to the two FARs for both 3GPP access and non-3GPP access in the case of supporting ATSSS.

Table 5.8.2.11.8-1: Attributes within Multi-Access Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this MAR. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Steering functionality |  | Indicates the applicable traffic steering functionality: Values "MPTCP functionality", "ATSSS-LL functionality". |  |
| Steering mode |  | Values "Active-Standby", "Smallest Delay", "Load Balancing" or "Priority-based". |  |
| Steering Mode Indicator |  | Allows autonomous operation if steering mode is set to "Load Balancing". |  |
| Threshold values |  | A Maximum RTT and/or a Maximum Packet Loss Rate | The Threshold Values are applied by UPF as described in clause 5.32.8. |
| Per-Access Forwarding | Forwarding Action Rule ID | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Action information | Weight | Identifies the weight for the FAR if steering mode is "Load Balancing" | The weights for all FARs need to sum up to 100 |
| (NOTE 1) (NOTE 2) | Priority | Values "Active or Standby" or "High or Low" for the FAR | "Active or Standby" for "Active-Standby" steering mode and "High or Low" for "Priority-based" steering mode |
|  | List of Usage Reporting Rule ID(s) | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. | This enables the SMF to request separate usage reports for different FARs (i.e. different accesses) |
| NOTE 1: The Per-Access Forwarding Action information is provided per access type (i.e. 3GPP access or Non-3GPP access). NOTE 2: The Weight is treated as the default percentages if the Autonomous operation is allowed for the "Load Balancing" steering mode. |  |  |  |

# * * * * End of changes * * * *