3GPP TSG-SA WG2 Meeting #144E (e-meeting)	S2-2103309

# April 12 – April 16, 2021, Elbonia									   (revision of S2-2100799, 2318r04)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2647 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PMF extensions for sending UE-assistance data to UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It has been concluded in TR 23.700-93 that: For all steering modes that will be considered in the normative phase: a) The network may provide a UE-assistance indication, which indicates that (a) the UE can decide how to distribute the UL traffic based on its internal state (e.g. battery level), and (b) the UE can request from UPF to apply the same distribution for the DL traffic, and the UPF can take the UE's request into account when deciding the DL transmission traffic distribution. b) The UE requests from UPF to apply the same distribution for the DL traffic by using the PMF protocol, if available, or another mechanism, if the PMF protocol is not available. This other mechanism will be determined during the normative phase of the work. Therefore, it shall be possible to support the functionality in bullet b). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PMF protocol is extended so that it can enable the UE to send UE-assistance data to UPF, which specify the UL traffic distribution applied by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible for UPF to apply the same distribution for the DL traffic as the distribution for the UL traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.1, 5.32.5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.32.5	Access Network Performance Measurements

#### 5.32.5.1	General principles

When an MA PDU Session is established, the network may provide the UE with Measurement Assistance Information. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether measurement reports need to be sent to the network.

Measurement Assistance Information shall include the addressing information of a Performance Measurement Function (PMF) in the UPF, the UE can send PMF protocol messages to:

-	For a PDU Session of IP type, Measurement Assistance Information contains one IP address for the PMF, one UDP port associated with 3GPP access and another UDP port associated with non-3GPP access;

-	For a PDU Session of Ethernet type, Measurement Assistance Information contains one MAC address associated with 3GPP access and another MAC address associated with non-3GPP access.

NOTE 1:	To protect the PMF in the UPF (e.g. to block DDOS to the PMF), the IP addresses of the PMF are only accessible from the UE IP address via the N3/N9 interface.

NOTE 2:	After the MA PDU Session is released, the same UE IP address/prefix is not allocated to another UE for MA PDU Session in a short time.

The UE and the UPF may need to perform access performance measurements in order to estimate the Round-Trip Time (RTT) and/or the Packet Loss Rate (PLR) that an SDF is expected to experience when transmitted on a certain access type. Based on these measurements and the provisioned ATSSS rules in the UE and MAR rules in the UPF, the UE and the UPF decide how to distribute the traffic of an SDF across the two accesses.

If the UE and the UPF decide to initiate access performance measurements to estimate the RTT and/or the PLR for an SDF, the access performance measurements shall be performed either

(a)	using the default QoS Flow, or

(b)	using the target QoS Flow, which is the QoS Flow that the SDF traffic is transmitted on.

The UE shall perform access performance measurements based on the target QoS Flow only when this is explicitly indicated in the Measurement Assistance Information. Otherwise, the UE shall perform access performance measurements based on the default QoS Flow. The UPF shall perform access performance measurements based on the target QoS Flow only when this is explicitly indicated by SMF. Otherwise, the UPF shall perform access performance measurements based on the default QoS Flow.

Editor's note:	It is FFS how the Measurement Assistance Information indicates whether access performance measurements shall be performed based on (a) or (b). For example, (b) will be used when the Measurement Assistance Information contains different PMF addresses/ports for different QoS Flows, or when the Measurement Assistance Information contains the same PMF address/port for all QoS Flows and a new indicator.

The UE and the UPF may decide not to initiate access performance measurements using PMF over a certain target QoS Flow, when they already have access performance measurements for another target QoS Flow which they determine can be reused

NOTE 3:	How the UE and UPF determine that the performance measurements using a certain target QoS Flow apply to another target QoS Flow is based on implementation e.g. AN resource to QoS Flow mapping in the UE or getting similar access measurements results with other QoS Flow.

Editor's note:	It is FFS how to send PMF message over the target QoS Flow. For transmitting the PMF messages over the target QoS Flow, different alternatives have been identified, e.g. (a) using different PMF addresses/ports for each QoS Flow, or (b) using the same PMF address/port for all QoS Flows and include QFI in the PMF message header.

Editor's note:	It is FFS how the UE and UPF determines target QoS Flow.

The addressing information of the PMF in the UPF is retrieved by the SMF from the UPF during N4 session establishment.

The following PMF protocol messages can be exchanged between the UE and the PMF:

-	Messages to allow for Round Trip Time (RTT) measurements, i.e. when the "Smallest Delay" or "Load-Balancing" steering mode is used;

-	Messages to allow for Packet Loss Rate (PLR) measurements, i.e. when the "Load Balancing" steering mode is used;

-	Messages for reporting Access availability/unavailability by the UE to the UPF;

-	Messages for sending UE-assistance data to UPF. Such messages may be sent from the UE to UPF only when the UE receives the UE-assistance indicator in an ATSSS rule, as specified in clause 5.32.8. Further details are provided in clause 5.32.5.X.

Editor’s note: It is FFS how the UE can send UE-assistance data to UPF when the PMF protocol is not used by the UE.

The PMF protocol is specified in TS 24.193 [109].

The PMF protocol messages used for access availability/unavailability reports shall be sent on the default QoS Flow, i.e. the QoS Flow associated with default QoS rule. The PMF protocol messages used for access performance measurements shall be sent either on the default QoS Flow, or on the target QoS Flow, as specified above.

The QoS Flow associated with default QoS rule for MA PDU Session is Non-GBR QoS Flow.

The UE shall not apply the ATSSS rules and the UPF shall not apply the MAR rules for the PMF protocol messages.

When the UE requests a MA PDU session and indicates it is capable to support the MPTCP functionality with any steering mode and the ATSSS-LL functionality with only the Active-Standby steering mode (as specified in clause 5.32.6.1), the network may send Measurement Assistance Information for the UE to send Access availability/unavailability reports to the UPF. In this case, the UE and UPF shall not perform RTT and PLR measurements using PMF as the UE and UPF can use measurements available at the MPTCP layer.

#### 5.32.5.2	Round Trip Time Measurements

RTT measurements can be conducted by the UE and UPF independently. There is no measurement reporting from one side to the other. RTT measurements are defined to support the "Smallest Delay" or "Load Balancing" steering mode.

The estimation of the RTT by the UE and by the UPF is based on the following mechanism:

1.	The PMF in the UE sends over the user plane PMF-Echo Request messages to the PMF in the UPF, and the PMF in the UPF responds to each one with a PMF-Echo Response message. Similarly, the PMF in the UPF sends over the user plane PMF-Echo Request messages to the PMF in the UE, and the PMF in the UE responds to each one with a PMF-Echo Response message.

2.	In the case of a MA PDU Session of IP type:

-	The PMF in the UE sends PMF messages to the PMF in the UPF over UDP/IP. The destination IP address is the IP address contained in the Measurement Assistance Information and the destination UDP port is one of the two UDP ports contained in the Measurement Assistance Information. One UDP port is used for sending PMF messages to UPF over 3GPP access and the other UDP port is used for sending PMF messages to UPF over non-3GPP access. The source IP address is the IP address assigned to UE for the MA PDU Session and the source UDP port is a UDP port that is dynamically allocated by the UE for PMF communication. This source UDP port in the UE remains the same for the entire lifetime of the MA PDU Session.

-	The PMF in the UPF sends PMF messages to the PMF in the UE over UDP/IP. The source IP address is the same IP address as the one provided in the Measurement Assistance Information and the source UDP port is one of the two UDP ports as provided in the Measurement Assistance Information. One UDP port is used for sending PMF messages to UE over 3GPP access and the other UDP port is used for sending PMF messages to UE over the non-3GPP access. The destination IPv4 address is the IPv4 address assigned to UE for the MA PDU Session (if any) and the destination IPv6 address is an IPv6 address selected by the UE from the IPv6 prefix assigned for the MA PDU Session (if any). The destination UDP port is the dynamically allocated UDP port in the UE, which is contained in all PMF messages received from the UE. If the UE receives Measurement Assistance Information, the UE shall inform the network via the user plane about the UE's dynamically allocated UDP port, and the IPv6 address if IPv6 is used for PMF messages, so that it is possible for the UPF to know the UE's IPv6 address (if applicable) and dynamically allocated UDP port as soon as the MA PDU Session has been established.

3.	In the case of a MA PDU Session of Ethernet type:

-	The PMF in the UE sends PMF messages to the PMF in the UPF over Ethernet. The Ethertype is the Ethertype contained in the Measurement Assistance Information and the destination MAC address is one of the two MAC addresses contained in the Measurement Assistance Information. One MAC address is used for sending PMF messages to UPF over 3GPP access and the other MAC address is used for sending PMF messages to UPF over non-3GPP access. The source MAC address is a MAC address of the UE, which remains the same for the entire lifetime of the MA PDU Session.

-	The PMF in the UPF sends PMF messages to the PMF in the UE over Ethernet. The Ethertype is the same Ethertype as the one provided in the Measurement Assistance Information and the source MAC address is one of the two MAC addresses as provided in the Measurement Assistance Information. One MAC address is used for sending PMF messages to UE over 3GPP access and the other MAC address is used for sending PMF messages to UE over non-3GPP access. The destination MAC address is the MAC address of the UE, which is contained in all PMF messages received from the UE. If the UE receives Measurement Assistance Information, the UE shall inform the network via the user plane about the UE's MAC address so that it is possible for the UPF to know the UE's MAC address as soon as the MA PDU Session has been established.

4.	When the UP connection of the MA PDU session is deactivated on an access, no PMF-Echo Request messages are sent on this access. The PMF in the UPF shall not send PMF-Echo Request on this access if the UP connection is not available or after it receives notification from the (H-)SMF to stop sending the PMF-Echo Request on this access.

5.	The UE and the UPF derive an estimation of the average RTT over an access type and QoS Flow by averaging the RTT measurements obtained over this access type and QoS Flow.

#### 5.32.5.2a	Packet Loss Rate Measurements

The UE and the UPF may decide to estimate the Packet Loss Rate (PLR) for an SDF over both accesses. For example, the UE may take this decision when an ATSSS rule in the UE requires the traffic of an SDF to be steered in accordance with a PLR-based threshold condition (e.g. PLR < 2%).

The UE and the UPF calculate the PLR for an SDF by exchanging PMF-PLR Report messages, as specified below. A PMF-PLR Report message is sent over 3GPP access or over non-3GPP access, using either the default QoS flow or a "target" QoS flow, as specified in clause 5.32.5.1.

The calculation of the PLR by the UE and by the UPF is based on the following mechanism. It is assumed that the PLR should be calculated for a target QoS flow, however, the same mechanism applies when the PLR should be calculated for the default QoS flow.

-	The UE requests from UPF to start counting the number of received UL packets by sending a PMF-PLR Count Request message. The UPF starts counting of the received UL packets over the target QoS Flow and over the access network which the PMF-PLR Count Request message was received from. The UE starts counting the transmitted UL packets over the target QoS Flow and access network when it receives a PMF-PLR Count Response message from UPF.

-	The UE requests from UPF to report the number of counted UL packets by sending a PMF-PLR Report Request message. The UPF sends a PMF-PLR Report Response message including the number of UL packets counted since it received the last PMF-PLR Count Request message.

NOTE:	A PMF-PLR Report Request message can also indicate to UPF to start counting packets if the UE wants to measure the Packet Loss Rate again.

-	The UE calculates the UL packet loss ratio based on the local counting result of the number of transmitted UL packets and reported number of received UL packets in the UPF.

-	The UPF applies the same procedure for calculating the DL PLR

-	When the UP connection of the MA PDU session is deactivated on an access, no PMF-PLR messages are sent on this access. The PMF in the UPF shall not send PMF-PLR message on this access if the UP connection is not available or after it receives notification from the (H-)SMF to stop sending the PMF-PLR message on this access.

-	The UE and the UPF derive an estimation of the average PLR per QoS Flow over an access type by averaging the PLR measurements obtained over this access.

#### 5.32.5.3	Access Availability/Unavailability Report

If required by the network in the Measurement Assistance Information, the UE shall provide access availability/unavailability reports to the network. How the UE detects the unavailability and the availability of an access is based on implementation. When the UE detects the unavailability/availability of an access, it shall:

-	build a PMF-Access Report containing the access type and an indication of availability/unavailability of this access;

-	send the PMF-Access Report to the UPF via the user plane.

The UPF shall acknowledge the PMF-Access Report received from the UE.

#### 5.32.5.X	UE Assistance Data

If the UE receives the UE-assistance indicator in an ATSSS rule (as specified in clause 5.32.8) and the UE decides to apply a different UL traffic distribution for an SDF than the default UL traffic distribution indicated in the Steering Mode component of the ATSSS rule (e.g. because the UE is running out of battery), then the following applies.

-	The UE may send a PMF-UAD (UE Assistance Data) message to UPF that contains information specifying the UL traffic distribution applied by the UE for a particular SDF.

-	A PMF-UAD message contains: (a) correlation information that allows the UPF to identify the associated DL SDF, and (b) UL Distribution Info, indentifying how the UL traffic of the SDF is distributed by the UE.

Editor’s note: The correlation information is information that identifies an existing SDF in both UE and UPF, and could be new information included in the ATSSS rule and in the N4 rules, or it could be existing information such as an ATSSS rule ID that would then need to be sent also from SMF to UPF. The details of the correlation information are FFS.

Editor’s note: It is FFS whether the UE, when it decides to apply a different UL distribution, can change to an arbitrary traffic split percentage or whether the UE would change to using a single access for the SDF. It is further FFS whether the UL Distribution Info needs to contain the traffic split percentage if changed, or whether it can contain a preferred access for DL.

-	The UPF may apply the information in a received PMF-UAD message to align the DL traffic distribution for an SDF with the UL traffic distribution applied by the UE for the same SDF.

END OF CHANGES