3GPP TSG-SA WG2 Meeting #144E e-meeting	S2-2103472

# Elbonia, 12-16 April, 2021								(revision of S2-2102857r02_S2-2100980)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2591 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.502 KI#2 Network Slice Admission Control Function (NSACF) services and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Apple, Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Further to: - the eNS-Ph2 conclusions in the TR23.700-40 1.2.0, clause 8.2 for KI#2; and - a decision from SA2#143E meeting preparation telco on eNS-Ph2 to submit CRs based on the so-called push method. this contribution proposes the new feature addition to TS23.502. The changes are based on TR solution ‘6.10 Solution #10: Max number of PDU Sessions per Network Slice control via NSQ function’ which are fully compatible with the conclusions in the TR. 29/03/2021 – update and avilability check procedures combined in one procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New eNS_Ph2 feature addition for K#2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNS_Ph2 K#2 conclusions are not implemented in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.x (new), 5.2.21.1, 5.2.21.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 2680 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

#### 4.2.11.x	Number of PDU Sessions per network slice availability check and update procedure

The number of PDU Sessions per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of PDU Sessions established on S-NSSAI which is subject to NSAC. The SMF is configured with the information indicating which network slice is subject to NSAC.

NSACF

SMF

2. Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate_Request

4. Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate_Response

3. The number of PDUs per network slice availability check and update.

1. A trigger to perform number of PDUs per network slice availability check and update

NSACF

SMF

2. Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate_Request

4. Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate_Response

3. The number of PDUs per network slice availability check and update.

1. A trigger to perform number of PDUs per network slice availability check and update

Figure 4.2.x.2-1: Number of PDU Sessions per network slice availability check and update procedure

- The SMF triggers the Number of PDU Sessions per network slice availability check and update procedure for the network slices that are subject to NSAC at the beginning of a PDU Session Establishment procedure (clause 4.3.2.2.1 and clause 4.3.2.2.2) and as a last step of successful PDU Session Release procedure (clause 4.3.4.2 and clause 4.3.4.3).

- The SMF sends Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate_Request message to the NSACF. The SMF includes in the message the S-NSSAI for which the number of PDU Sessions per network slice update is required and the update flag which indicates that the number of PDUs established on the S-NSSAI is to be increased if the procedure is triggered at the beginning of PDU Session Establishment procedure or indicates that the number of PDU Sessions on the S-NSSAI is to be decreased if the procedure is triggered at the end of PDU Sessions Release procedure.

- The NSACF updates the current number of PDU Sessions established on the S-NSSAI, i.e. increase or decrease the number of PDU Sessions per network slice based on the information provided by the SMF in the update flag parameter.

If the update flag parameter from the SMF indicates increase the current number of PDU Sessions per network slice and the maximum number of PDU Sessions established on the network slice has not been reached yet, the NSACF increases the number of PDU Sessions for that network slice. If the maximum number of PDU Sessions established on the network slice has already been reached, then the NSACF returns a result parameter indicating that the maximum number of PDU Sessions per network slice has been reached.

If the update flag parameter from the SMF indicates decrease the current number of PDU Sessions per network slice, the NSACF decreases the number of PDU Sessions for that network slice.

- The NSACF acknowledges the update to the SMF with Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate_Response message. If the NSACF returned maximum number of PDU Sessions per network slice reached result, the SMF rejects the PDU Session establishment request with maximum number of PDU Sessions per network slice reached reject cause.

In case of a PDU Session Establishment failure, the SMF triggers another request to the NSACF with the update flag parameter equal to decrease in order to re-adjust back the PDU Session counter in the NSACF.

Editor's note:	It is FFS how to achieve high admission control accuracy.

Editor’s Note: Whether SMF or AMF interacts with the NSACF is FFS.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

### 5.2.21	Network Slice Admission Control Function (NSACF) services

#### 5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NumberOfUEsPerSlice | AvailabilityCheckAndUpdate | Request/Response | AMF |
|  | EACNotify |  | AMF |
| Nnascf_NumberOfPDUsPerSlice | AvailabilityCheckAndUpdate | Request/Response | SMF |

* * * * start of 2nd change * * * *

* * * * start of 3rd change * * * *

#### 5.2.21.x	Nnsacf_NumberOfPDUsPerSlice services

##### 5.2.21.x.1	General

Service Description: The Nnsacf_NumberOfPDUsPerSlice services control the number of PDU Sessions with a network slice for the network slices subject to NSAC. The SMF can request the NSACF to check whether the number of PDU Sessions established on a network slice has reached the maximum number of PDU Sessions per network slice and the SMF can also request the NSACF to update the number of PDU Sessions established on a network slice.

##### 5.2.xx.x.2	Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate service operation

Service Operation name: Nnsacf_NumberOfPDUsPerSliceAvailabilityCheckAndUpdate

Description: Updates the number of PDU Sessions established on a network slice (e.g. increase or decrease). Also, if the number of PDU Sessions on the network slice is to be increased, the NSACF first checks whether the number of the PDU Sessions on that network slice has reached the maximum number of PDU Sessions per network slice threshold. If the maximum number of PDU Sessions on the network slice has already been reached, the PDU Session Establishment procedure is rejected.

Inputs, Required: S-NSSAI, update flag.

The S-NSSAI parameter is the network slice for which the number of PDU Sessions established on a network slice is to be updated.

The update flag input parameter indicates whether the number of the PDU Sessions established on that network slice is to be increased, for example at PDU Session Establishment procedure or decreased, for example at PDU Session Release procedure.

Outputs, Required: maximum number of PDU Sessions per network slice reached, availability status

* * * * end of 3rd change * * * *