3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102471r010

Elbonia, Apr 12 – Apr 16, 2021	(revision of S2-210XXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2759 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of NWDAF discovery of data exposure capability in TS 23.501. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Alignment in TS 23.501 in the extensions of NWDAF NF Profile to support the proposal to use the Data Sources Profile information when NWDAF exposes the DataManagement service in order to solve the FFSs on how an NWDAF discovers another NWDAF with data collection capability in TS 23.288 Clause 6.2.7.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extension to NWDAF Discovery description to consider of which type of data can be exposed by an NWDAF . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution of data collection from NWDAF not complete in R17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2, 6.3.13, 6.3.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR0290, |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.2.6.2	NF profile

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

-	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in TS 23.502 [3] clause 5.2.7.2.2. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g. DNN or DNN list, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data centre.

-	TAI(s).

-	NF load information.

-	Routing Indicator, for UDM and AUSF.

-	One or more GUAMI(s), in the case of AMF.

-	SMF area identity(ies) in the case of UPF.

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	Supported Analytics ID(s), possibly per service, NWDAF Serving Area information (i.e. list of TAIs for which the NWDAF can provide services and/or data), Supported Analytics Delay per Analytics ID (i.e. the Analytics ID(s) associated with a Supported Analytics Delay can be treated with a time less than or equal to the given delay), NF types of the NF data sources, NF Set IDs of the NF data sources, if available, in the case of NWDAF.

NOTE 4:	The NWDAF's Serving Area information is common to all its supported Analytics IDs.

NOTE 5:	The Analytics IDs supported by the NWDAF may be associated with a Supported Analytics Delay.

NOTE 6:	The determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific.

-	Event ID(s) supported by AFs, in the case of NEF.

-	Application Identifier(s) supported by AFs, in the case of NEF.

-	Range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF, in the case of NEF.

NOTE 7:	This is applicable when NEF exposes AF information for analytics purpose as detailed in TS 23.288 [86].

NOTE 8:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 9:	The NRF may store a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) and PCF Group ID and SUPI(s), to enable discovery of UDM, UDR, AUSF and PCF using SUPI, SUPI ranges as specified in clause 6.3 or interact with UDR to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID based on UE identity, e.g. SUPI (see clause 6.3.1 for details).

-	IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [58], Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, in the case of BSF.

-	SCP Domain the NF belongs to.

-	DCCF Serving Area information, NF types of the data sources, NF Set IDs of the data sources, if available, in the case of DCCF.

-	Supported DNAI list, in the case of SMF.

# * * * * Second change * * * *

### 6.3.13	NWDAF discovery and selection

Multiple instances of NWDAF may be deployed in a network.

The NF consumers shall utilize the NRF to discover NWDAF instance(s) unless NWDAF information is available by other means, e.g. locally configured on NF consumers. The NWDAF selection function in NF consumers selects an NWDAF instance based on the available NWDAF instances.

The NRF may return one or more candidate NWDAF instance(s) and each candidate NWDAF instance (based on its registered profile) supports the Analytics ID with a time that is less than or equal to the Supported Analytics Delay.

The following factors may be considered by the NF consumer for NWDAF selection:

-	S-NSSAI.

-	Analytics ID(s).

-	Supported service(s), possibly with their associated Analytics IDs.

-	NWDAF Serving Area information, i.e. list of TAIs for which the NWDAF can provide analytics and/or data.

NOTE 1:	If all services provided by one NWDAF do not support the same Analytics ID, the NWDAF registers the Analytics IDs of the services at the service level.

NOTE 2:	Analytics ID(s) at service level take precedence over Analytics ID(s) at NF level.

NOTE 3:	For discovery of NWDAF supporting Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo services, the Analytics IDs at the NWDAF NF profile are used.

-	NWDAF location information, if the selection is to decide candidates for analytics subscription transfer

-	NF type of the data source.

-	NF Set ID of the data source.

NOTE 4:	NF type of the data source or NF Set ID of the data source can be used when the NWDAF determines that it needs to discover another NWDAF which is responsible for co-ordinating the collection of required data. The NWDAF does a new discovery for a target NWDAF via NRF using NF Set ID of the data source.

NOTE 5:	For discovery of NWDAF supporting Nnwdaf_DataManagement service, at least the NF type of the data source or the NF Set ID of the data source and NWDAF Serving Area information from the NWDAF profile are used.

NOTE 6:	The presence of NF type of the data source or NF Set ID of the data sourcedenotes that the NWDAF can collect data from such NF Sets or NF types.

-	Supported Analytics Delay of the requested Analytics ID(s) (see clause 6.2.6.2).

In the case of multiple instances of NWDAFs deployment, following factors may also be considered:

-	NWDAF Capabilities:

-	Analytics aggregation capability.

-	Analytics metadata provisioning capability.

Editor's note:	Whether analytics metadata provisioning capability to be registered in NRF or to be queried from NRF is FFS.

When selecting an NWDAF for ML model provisioning, the following additional factors may be considered by the NWDAF:

-	Model(s) supported per Analytics ID(s).

# * * * * Third change * * * *

### 6.3.19	DCCF discovery and selection

Multiple instances of DCCF may be deployed in a network.

The NF consumers shall utilize the NRF to discover DCCF instance(s) unless DCCF information is available by other means, e.g. locally configured on NF consumers. The DCCF selection function in NF consumers selects a DCCF instance based on the available DCCF instances.

The following factors may be considered by the NF consumer for DCCF selection:

-	DCCF Serving Area information, i.e. list of TAIs for which the DCCF coordinates Data Sources.

-	S-NSSAI.

-	NF type of the data source.

-	NF Set ID of the data source.

NOTE:	NF Set ID can be used when the NF consumer is a DCCF when the DCCF determines that it needs to discover another DCCF which is responsible for co-ordinating the collection of required data. The DCCF discovers a target DCCF via NRF using NF Set ID of the data source.

# * * * * End of changes * * * *