SA WG2 Meeting #144E e-meeting	S2-2102737r02

Elbonia, April 12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2693 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple ECS Address coexistence and provision to 5GC and UE for one time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in TS 23.548 clause 6.5.2.1, there are Editor’s Note to describe the issue of multiple ECS address (es) and how to provide ECS address towards any UEs For the first editor’s note, as discussed in S2-210xxxx, due to multiple ECS exists, in other to differentiate the service from different providers, the simple way is providing ECS address together with other relation parameters such as provider related (ECSP, ECS ID) and target consumer related (EEC ID, APP ID, AC ID and etc). And for the second editor’s note, as discussed in S2-210xxxx, the ECS address configuration information is provided by reusing service specific information provisioning procedure. If identifiers of target UE(s) or a group of UEs are not provided, then the ECS address(es) are deliver to any UEs by ePCO procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The 3rd party AF provides both ECS address and other relation parameters to 5GC and UE in parameter provision. The relation parameters include address provider related (ECSP, ECS ID) parameters and target consumer related (EEC ID, APP ID, AC ID and etc) The ECS address configuration information is provided by reusing service specific information provisioning procedure. If identifiers of target UE(s) or a group of UEs are not provided, then the ECS address(es) are deliver to any UEs by ePCO procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AF would not be able to provide ECS address to any UEs and UE can’t distinguish between different ECS addresses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3d, 4.15.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

4.15.6.3d	ECS Address Configuration Information Parameters

ECS Address Configuration Information parameters that an AF may provide are described in Table 4.15.6.3d-1. ECS Address Configuration Information is stored as Session Management Subscription data. The ECS Address Configuration Information is associated with a DNN and S-NSSAI included in the message from UDM.

The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s), optionally the corresponding ECS Provider ID and ECS ID

Table 4.15.6.3d-1: Description of ECS Address Configuration Information parameters

| Parameters | Description |
| --- | --- |
| ECS Address | FQDN and/or IP Address of Edge Configuration Server. |
| ECS ID | This IE indicates to the Identifier of ECS. The may be used to differentiate ECSs. |
| ECSP ID | The identifier of the ECS Provider |

* * * End of Change * * *